Source:	Editor

Title:	IVAS Design Constraints (IVAS-4)

Version:	0.0.9 (draft2)

Agenda Item:

- Scope

This document presents the Design Constraints of the EVS Codec Extension for Immersive Voice and Audio Services (IVAS). The development of IVAS was initiated at SA4 #94, approved at SA#77 in September 2017 and the Work Item is described in SP-170611. The target for the standardisation is to complete codec specifications for Release 16.

The remaining sections of this document describe the design constraints for the IVAS codec in detail.

- Introduction

The overall objective of the IVAS Codec work item is to develop a single general-purpose audio codec for immersive 4G and 5G services and applications including the VR use cases envisioned in 3GPP TR 26.918 (See SP-170611).

- IVAS Codec Design Constraints

| Sampling Frequency and Audio Bandwidth | The encoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. The decoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. Note: Functions that are necessary for achieving the arbitrary selection of encoder / decoder input / output sampling rate are envisioned to be a part of the IVAS candidate. The encoder shall support input signals with different input signal bandwidth (NB, WB, SWB, and FB) with frequency masks as defined for EVS. The encoder and decoder shall support 8kHz sampling when EVS bit-exact operation is used (See Backward Interoperability) |
| --- | --- |
| Encoder Input Formats | The encoder shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio [Orange proposal: NOTE: Binaural audio corresponds to spatial encoding of a soundfield as typically captured at the entrance of ear canals and for this reason it is primarily intended for listening over headphones. In terms of spatial encoding binaural audio may be natural (truly recorded with microphones) or artificial (using HRTFs).] [Dolby proposal: Note: Binaural audio is defined as two-channel spatial audio intended for direct headphone playback.] [Editor’s note: Fraunhofer also proposed to delete the previous Editor’s note, by adding a definition in the back of this document.] Binaural audio: two channel stereo audio (2.0) for direct presentation to the left and right ear over a pair of headphones.] - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| [Default renderer] | [Dolby proposal: The IVAS codec candidate shall provide a default renderer, which is the renderer supplied along with IVAS codec candidate, to be used in evaluations of that candidate and for specification.] |
| [Default] [Rendered] Output Formats | The IVAS codec shall support the following rendered output formats: - Mono - Stereo - Binaural audio - Multi-channel Editor’s Note: Corresponding output formats for each input formats is TBD [Ericsson proposal: in accordance with Table 2.] [Fraunhofer proposal: For channel-based audio, the supported rendered output formats shall include the input format or any smaller configuration, i.e. - Input mono (1.0) Output mono (1.0) - Input stereo (2.0) Output stereo (2.0), mono (1.0) - Input surround (5.1) Output surround (5.1), stereo (2.0), binaural stereo, mono (1.0) - Input surround (7.1) Output surround (7.1), surround (5.1), stereo (2.0), binaural stereo, mono (1.0) - [ - Input surround + height (5.1+4) Output surround + height (5.1+4), surround (5.1), stereo (2.0), binaural stereo, mono (1.0) - Input surround + height (7.1+4) Output surround + height (7.1+4), surround + height (5.1+4), surround (7.1), surround (5.1), stereo (2.0), binaural stereo, mono (1.0) ] For binaural audio input, the supported rendered output formats shall include the following configurations: - Input Binaural audio Output binaural audio by means of a stereo (2.0) signal For any other input format, the following output formats shall be included: - Binaural Stereo - Surround audio (5.1 and 7.1) and Surround + height audio (5.1+4 and 7.1+4) - Stereo - Mono Editor’s note: The exact loudspeaker configurations for the surround audio formats (5.1, 7.1, 5.1+4, 7.1.4) is TBD. ] Editor’s Note: Input on the definition of internal renderer and/or external renderer are invited. |
| [Non-Rendered Output Formats] [Pass-Through Operation] | [Ericsson proposal: The IVAS codec shall support pass-through operation, where the output format is identical to the encoder input format, for the following format categories (as specified in Encoder Input Formats): - Channel-based audio - Binaural audio - Scene-based audio - [Spatial audio] - Object-based audio in accordance with Table 2 when required according to IVAS-3, and if not limited by a preferred audio format specified at the encoder. Editor’s Note: The exact codec configurations (bitrates etc.) for which pass-through operation is required will be specified in IVAS-3 (Performance Requirements). In non-pass-through operation, the IVAS codec shall deliver at least one of the optional output formats marked with ‘o’ in Table 2.] [Fraunhofer proposal: The IVAS codec shall support pass-through operation for the following audio formats: - Channel-based audio, including mono (1.0), stereo (2.0; including binauralized input), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. For pass-through operation, the decoder output format is identical to the encoder input format. The exact bitrates for which pass-through operation is to be supported is specified in IVAS Performance Requirements (IVAS-3).] |
| [Interface to external rendering] | [Orange proposal: The IVAS renderer shall provide an interface to an external loudspeaker renderer and/or external binaural renderer.] |
| Bit Rates | When EVS bit-exact operation is used (see Backward interoperability), the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes) Note: As implemented in EVS, the bit rates up to 24.4 kb/s are gross bit rates and they are net bit rates above 24.4 kb/s. In other cases than EVS bit-exact operation: the IVAS codec shall operate at bit rates of [5.9VBR, 7.2, 8, 9.6, 13.2, 16.4], 24.4, 32, 48, 64, 96, 128, 160, 192, 256, [384, 512] kb/s [min and max TBD]. Note: The gross bit rate supported in the DTX/CNG/SID operation is [TBD]. |
| Algorithmic Delay | TBD [Editor’s Note: The EVS Algorithmic delay is 32ms] |
| Complexity | TBD |
| Backward Interoperability | Having interoperability with the EVS is an important feature. The full EVS codec algorithm shall be part of the IVAS candidate codec solution. EVS bit-exact processing shall be used when the input to the IVAS codec is a simple mono signal without spatial metadata and should also be applied whenever possible. Possible exceptions for particular operation modes are [tbd]. When multiple mono audio channels without spatial metadata are negotiated they shall all be bit-exact with EVS. Possible exceptions are [tbd]. The IVAS Codec shall support certain stereo modes of operation which include an embedded bit-exact EVS mono downmix bitstream at the bit-rates from 9.6 kbit/s to 24.4 kbit/s SWB (9.6/13.2/16.4/24.4 kbit/s). Editor’s Note: Definition of bit-exactness for embedded stereo modes is FFS. Note 1: When the IVAS codec uses EVS bit-exact operation then features such as AMR-WB I/O mode, EVS SID update rates and 8kHz sample rate support shall be supported. [Note 2: TS 26.445 supports multiple mono channels in the same way as described in IETF RFC 4867, which supports multi-channel audio content by means of multiple mono channels. Use of the EVS multiple mono interoperability for encoding multi-channel audio is FFS. Being bit-exact with EVS for the multiple mono case would not be appropriate because channels are not synchronized in the EVS encoding of multiple mono channels which may result in different VAD decisions for different channels as noted in the section of A.2.5.1 of TS 26.445. Contributions are invited if some synchronization is necessary for such use case.] |
| Frame length | The candidate codecs shall operate with a frame size of 20 ms. |
| Jitter Buffer Management (JBM) | A JBM solution conforming to the requirements in TS 26.114, except for the functional requirement in sub-clause 8.2.2 of TS 26.114: “Speech JBM used in MTSI shall support all the codecs as defined in clause 5.2.1”, shall be provided with the candidate codecs. [Note: The JBM defined in TS 26.448 may form the basis of the JBM provided with the candidate codecs.] |
| Rate switching | [The candidate codecs shall perform rate switching upon command to the encoder throughout the entire bit rate range at arbitrary frame boundaries. The rate switching may imply switching between different bandwidths and between mono coding modes and coding modes for multiple audio streams.] |
| Packet loss concealment (PLC) | A PLC solution shall be provided by the IVAS candidate codecs. |
| RTP payload format | Candidate codecs shall provide an RTP payload format specification supporting the full set of features and functionality of the IVAS candidate codecs. |
| DTX | The candidate codecs shall provide a complete VAD/DTX/CNG framework. It shall be possible to operate the codec with DTX on or DTX off. SID update frames shall be sent with a frequency not exceeding once per 8 frames. |
| [Encoder Interface] | [Ericsson proposal: The encoder shall provide an interface for indication of a preferred rendered output audio format. All Rendered Output Formats shall be supported. The encoder shall provide an interface for activation/deactivation of pass-through operation for each audio stream individually (e.g. for an object or for an HOA stream).] Editor’s Note: Definition of a common interface is FFS. |
| [Interface for Rendered Output] | [Ericsson proposal: The decoder/renderer shall provide an interface for specification of the output audio format to be rendered. All Rendered Output Formats shall be supported.] |
| [Interface for Non-Rendered Output] | [Ericsson proposal: The decoder shall provide an interface for external rendering supporting all Non-rendered Output Formats. Editor’s Note: Definition of a common interface is FFS.] |
| [External Rendering Interface] | [Fraunhofer proposal: The IVAS codec shall support the following interface formats for optional external rendering solutions: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. ] |
| Interface for binaural rendering | The IVAS decoder/renderer shall provide an interface to provide [HRTF/BRIR] data for binaural rendering. The interface is [tbd]. [Note: There was some support for this interface to follow the SOFA SimpleFreeFieldHRIR convention - See AES69-2015]. The IVAS decoder/renderer shall provide an API to provide [TBD scene displacement data]. [The IVAS decoder/renderer shall support direct headphone presentation.] |
| Control Data For Binaural Audio Rendering | The IVAS decoder/renderer shall support the following control data for binaural audio rendering: [HRTF/BRIR] data for binaural rendering on command line interface. The format for [HRTF/BRIR] data is [tbd]. [Note: There was some support for this interface to follow the SOFA SimpleFreeFieldHRIR convention - See AES69-2015]. [TBD scene displacement data]. |
| Direct headphone presentation | The IVAS codec shall support direct headphone presentation for non-diegetic signals, with metadata indicating if the decoded non-diegetic signal has one channel (with an associated panning gain) or two channels (stereo or binaural). |
| [Decoder/Renderer Motion to Sound Algorithmic Delay] | [The maximum algorithmic delay from a detected change in head roll, azimuth & elevation to a binaural sound rendered within +/- [Y] degree(s) of the detected change shall be [20 ms].] |
| Output gain limitation | TBD |
| [External Renderer API] | [Dolby proposal: An external renderer is a renderer that is connected to the IVAS decoder via the External Renderer API. The IVAS codec candidate shall offer the possibility to connect an external renderer via the external renderer API. The API shall support render of any received and decoded input audio.] |

[Editor’s Note FFS: Tdoc S4-171221 proposes to add high-level design constraints for IVAS codec modes suitable for a spatial conferencing use-case and if agreed, corresponding updates to the design constraints would be made.]

[Definitions]

[Fraunhofer proposal: For the purposes of IVAS work item, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Binaural audio: two channel stereo audio (2.0) for direct presentation to the left and right ear over a pair of headphones.]

[References]

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[Ericsson proposal: Table 2: Output formats, required (marked with ‘x’) or conditionally optional (at least one of the formats marked with ‘o’ are required)]

|  | Non-rendered formats |  |  |  |  |  |  | Rendered formats |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output format Input format |  | Channel-based | Binaural | Scene-based | [Spatial] | Object-based | Mono |  |  | Stereo (non-binaural) |  | Binaural |  | Multi-channel |  |
| Channel-based |  | x (pass-through) o (non-pass-through) |  | o |  |  | x |  |  | x |  | x |  | x |  |
| Binaural |  | o | x (pass-through) o (non-pass-through) |  |  |  | x |  |  | x |  | x |  | x |  |
| Scene-based |  | o |  | x (pass-through) o (non-pass-through) |  |  | x |  |  | x |  | x |  | x |  |
| [Spatial] |  | [o] |  | [o] | [x (pass-through) o (non-pass-through)] |  | [x] |  |  | [x] |  | [x] |  | [x] |  |
| Object-based |  | o |  | o |  | x (pass-through) o (non-pass-through) | x |  |  | x |  | x |  | x |  |
| Combinations of input formats | Output formats of corresponding input formats |  |  |  |  |  |  | x | x |  | x |  | x |  |

[Editor’s note: Some of the texts might go into different sections of this IVAS-4, or even to other documents. The above Table 2 is incorporated to preserve the integrity of Ericsson contribution]

4. Revision history

| Date |  | Meeting | Subject/Comment | Old | New |
| --- | --- | --- | --- | --- | --- |
| 2017-10-13 |  | SA4#95 | Agreement of Initial Skeleton of Design Constraints (IVAS-4) S4-171036 | N/A | 0.0.1 |
| 2017-11-17 |  | SA4#96 | Agreement of updating Initial Skeleton of Design Constraints (IVAS-4) S4-171353 | 0.0.1 | 0.0.2 |
| 2018-02-09 |  | SA4#97 | Agreement of updating Initial Skeleton of Design Constraints (IVAS-4) S4-180265 | 0.0.2 | 0.0.3 |
| 2018-04-13 |  | SA4#98 | Agreement of updating Initial Skeleton of Design Constraints (IVAS-4) S4-180605 | 0.0.3 | 0.0.4 |
| 2018-07-13 |  | SA4#100 | Agreement of updating Design Constraints (IVAS-4) S4-181218 | 0.0.5 | 0.0.6 |
| 2019-02-01 |  | SA4#102 | Agreement of updating Design Constraints (IVAS-4) S4-190248 | 0.0.6 | 0.0.7 |
| 2019-02-18 |  | AHEVS#56 | Changing editorship from Wang Bin to Huan-yu Su AHEVS-443 | 0.0.7 | 0.0.8 |
| 2019-02-20 | Post AHEVS#56 | Incorporating suggested changes from AHEVS#56 | 0.0.8 | 0.0.9 |

Huan-yu SU – Huawei Technologies Co Ltd