3GPP TSG-SA4 Meeting #103	S4-190367

# Newport Beach, CA, USA, 8. -12. April 2019								revision of S4-190173

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.238 | CR | 0003 | rev | 3 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remote Assist / Control Interface |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | E-FLUS |  |  |  |  |  | Date: |  |  | 22/03/2019 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR contains the implementation of the agreements from TD S4-181173 into the architecture section of TS 26.238 regarding the FLUS assist and remote control interface functions on (F-ARC). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of the FLUS assiste and remote control function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1, 4.2.3, 4.4.1, 5.3.2.2a, 7.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

### 4.2.1	General

Figure 4.2-1 depicts the architecture showing the relevant entities for providing an uplink streaming service.

Figure 4.2-1: FLUS architecture

The uplink streaming service architecture is based on a FLUS source located in a UE and a FLUS sink located in either another UE or in the network.

The FLUS source receives media content from one or more capture devices. In the context of this specification, the capture devices are considered as parts of a UE or are connected to it.

When the FLUS sink is located in a UE, the FLUS sink shall forward media content to a decoding and rendering function.

When the FLUS sink is located in the network, the FLUS sink may forward media content to a processing or distribution sub-function. The processing and distribution sub-functions are not in scope of the present specification. The FLUS sink may act as a Media Gateway Function (MGW) and/or an Application Function (AF).

The F reference point connects a FLUS source and a FLUS sink. The F reference point enables the FLUS source to establish and control a single FLUS session.

The F reference point also enables the FLUS sink and the FLUS source to mutually authenticate and authorize each other.

FLUS source and FLUS sink are each split into FLUS media (F-U) end-points, FLUS control (F-C) end-points and FLUS remote assist / control (F-ARC) end-points.

The UE, the FLUS Source and the FLUS sink are considered to be logical devices. Functions are not required to be located in the same physical device. Functions can be spread over multiple physical devices and interconnected via other interfaces.

There may be multiple FLUS assist and remote control end-points for a single FLUS Source . A assist and remote control end-point may be independent from a FLUS Sink, depending on the offered service.

The F reference point shall support security function for confidentiality protection for all, the FLUS control (F-C),  FLUS media (F-U) and FLUS assist and remote control (F-ARC) subfunctions.

Details of the FLUS control and FLUS media functions are shown in Figure 4.2-2.

Figure 4.2-2: Sub-functions of FLUS

Note that in the above diagram, hatched-filled boxes represent FLUS control plane functionality, solid line boxes represent mandatory functionality, and dashed-line boxes corresponds to optional functionality. Also, note that F-C and F-U denote FLUS control and FLUS user plane functionalities, respectively, and do not represent reference points.

FLUS control and F-C: FLUS control plane functionality including the associated processing by FLUS sink of the uploaded media for subsequent downstream distribution, plus FLUS media instantiation selection. F-C may also support configuration of static metadata for the session.

FLUS assist and remote control and F-ARC: The FLUS assist and remote control functionality on the FLUS Source receives messages. The messages affect the behaviour of the FLUS media function in the FLUS Source so that the FLUS Source can be remote controlled or remote assisted. Examples of remotely-provided assist information may pertain to network related conditions, viewership or engagement information from content recipients, or user preference data. Examples of commands issued to the UE-based FLUS remote assist/control are start or stop of the FLUS media upstreaming process in FLUS Source. An example recommendation issued by the UE-based FLUS assist and remote control to the FLUS media is to only upload the first 5 seconds of video to the network FLUS sink, due to current absence of viewership of the live uplink streaming content.

FLUS media and F-U: FLUS user plane functionality which includes setup of one or more media sessions and subsequent media data transmission via media streams. In some cases, a media session establishment protocol (e.g. IMS session set-up for MTSI-based FLUS instantiation) is necessary.

NOTE: F-C is not needed when the FLUS sink is an MTSI client that is only capable of rendering. In such event, logical control plane functions such as media and session descriptions and support for FLUS session establishment are encapsulated in FLUS user plane functionality.

F-C is used to establish and control the FLUS session. F-C allows the FLUS source to

- select a FLUS media instantiation,

- provision static metadata associated with each media session present in the FLUS session,

- select and configure the processing and distribution sub-functions.

The FLUS media instantiation is defined as part of the FLUS session. The user plane (F-U) may also contain the media stream establishment procedures when needed. Multiple media streams may be established for one FLUS session.

A media stream may contain media components of a single content type, e.g., audio, or media components of different content types, e.g., audio and video. A FLUS session may be composed of more than one media stream containing the same content type, e.g., multiple media streams of video.

*** Last Change ***

### 4.2.3	Uplink streaming for PSS-based distribution

The architecture of uplink streaming for subsequent PSS distribution is depicted in Figure 4.2-4.

Figure 4.2-4: Uplink streaming for PSS

The PSS Content Source is located on the UE side and contains the FLUS source.

*** Next Change ***

### 4.4.1	General

A FLUS session may include one or more media streams. Media streams are time bounded to the FLUS session they belong to. When a media stream is active, the FLUS source can send media content to the FLUS sink.

The establishment and / or teardown of a media session can be remotely controlled / influenced using FLUS remote assist / control functionality.

Figure 4.4-1 depicts an example relationship of a FLUS session with a single media session containing two media streams.

Figure 4.4-1: FLUS, media sessions and media streams

When the FLUS sink is located in a UE and the UE renders the received media content directly, the FLUS session may be implicitly present, e.g. it may be realized through IMS/MTSI.

When the FLUS sink is located on the network side and provides Media Gateway functionality, the FLUS session is used to select the media session instantiation and to configure any processing and distribution related sub-functions.

*** Next Change ***

#### 5.3.2.2a	FLUS session establishment for assiste and remote control

When FLUS assist and remote control is needed, the FLUS source is provisioned with the relevant information to establish one or more FLUS assist and remote control sessions (i.e. a URI following the 'ws' or 'wss' scheme [RFC6455]). The FLUS source creates the assist and remote control session and then listens for incoming messages. Note that the FLUS Sink in case of assist and remote control does not necessarily contain a FLUS media subfunction.

Figure 4.4-2: FLUS F-RAC Session creation

1. The FLUS control session for remote assist / control is established triggered by the FLUS Source.

2. On successful creation, the FLUS source receives a positive acknolwedge.

The FLUS Source starts waiting for remote assist/control related data/instructions.

3. In this example, the FLUS Source receives a start command and acts accordingly

*** Next Change ***

## 7.5a	Remote assist / control session using WebSockets

This paragraph defines the establishment and usage of a FLUS remote assist / control session using WebSocket Protocol as specified in [RFC 6455].

To establish the WebSocket based FLUS remote assist / control session, the FLUS source is provisioned with a WebSocket URI, which is either according to 'ws' scheme (unsecure) or according to the 'wss' scheme (secured). The FLUS Source establishes the WebSocket connection according to [RFC6455]. The keyword 'WebSocket' is added as an upgrade token when upgrading the session.

The following WebSocket conditions apply:

- The remote assist / control messages shall be json formated as defined in Clause 7a.

- The FLUS Remote assist / control message is carried as one WebSocket Message.

- For FLUS remote assist / control messages, Data frame messages shall use text type (opcode 0x01). FLUS remote assist / control messages shall be UTF-8 encoded.

*** Next Change ***

# 7a	Uplink Streaming Remote Assist / Control Interface

## 7a.1	Introduction

A FLUS remote assist / control session allows the sending remote assist / control related information or instructions to a FLUS Source. The FLUS source behaviour depends on the FLUS remote assist / control instruction.

This clause defines the FLUTE remote assist / control instructions and the resulting behaviour of the FLUS source.

## 7a.2	General Instruction Format

Remote assist / control related information or instruction are formatted in JavaScript Notation (json). The minimal remote assist / control instuction message shall contain a message_type value pair and a timing json object. The message_type value is a string. The timing object shall contain at least a creation_timestamp, which represents the timestamp at which the instruction message was created. The timing object may contain an action_timestamp, which represents the timestamp at which the instruction shall be executed. All timestamps within the timing object shall be ISO 8601 formatted.

Example:

{

"timing": {

"creation_timestamp": "2019-01-14T13:28:06.419Z",

"action_timestamp": "2019-01-14T13:30:00.000Z"

},

"message_type": "start"

}

## 7a.3	Start Instruction Message

The start remote assist / control instruction represents a trigger instruction to start the live uplink streaming process. The FLUS source shall start streaming uplink data. The value of the message_type property shall be set to “start”.

When the timing object does not contain an action_timestamp property, then the FLUS source shall perform the start of the uplink streaming process at instruction message reception.

When the timing object contains an action_timestamp property, then the FLUS source shall perform the start of the uplink streaming process according to the action_timestamp value. The F-U media plane should be established before.

## 7a.4	Stop Instruction Message

The stop remote assist / control instruction represents a trigger instruction to stop the live uplink streaming process. The FLUS source shall stop streaming uplink data. The value of the message_type property shall be set to “stop”.

When the timing object does not contain an action_timestamp property, then the FLUS source shall perform the stop of the uplink streaming process at instruction message reception.

When the timing object contains an action_timestamp property, then the FLUS source shall perform the stop of the uplink streaming process according to the action_timestamp value.

*** Last Change ***