Source:	Fraunhofer IIS

Title:	Considerations on object-based audio metadata for IVAS

Document for:	Discussion

Agenda Item:	7.5 – IVAS_Codec

# Introduction

Object-based audio is listed as one of the supported formats in IVAS Design Constraints [1]. However, the definition of the format has not yet been formalised and has been discussed in the contribution [2]. Object-based metadata can encompass various characteristics, but the most relevant for communication applications is certainly the position of the object in space and more precisely the direction from which the sound event should reach the listener.

This input document discusses some practical considerations on how to represent the position of the audio object in the metadata. It is the opinion of the source that there is a great advantage to a simple representation allowing manipulation during a pre-processing step such as rotation or translation.

# Representation of the Object position

The position of an element in space can be uniquely determined by a coordinate system. Usually, either the Cartesian coordinate (x,y,z) system or spherical coordinate (azimuth, elevation and radius/distance) system are adopted. The spherical coordinate system has the advantages that it dissociates two perceptually important features of a sound event; its direction-of-arrival and its distance to the listener. Moreover, simple object manipulation like rotation or translation can be easily achieved by computing new spherical coordinates without involving trigonometric operations.

In contribution [2], it was proposed to adopt the spherical coordinate system but also to combine azimuth and elevation in the same indexing while keeping the distance as a separate field. The azimuth and elevation are then coded jointly on 16 bits within a single spherical index. Spherical indexing has the main advantages of obtaining a compact representation of the direction on 2 bytes, which allows a byte-aligned metadata field, while covering quasi uniformly all directions on the sphere. However, spherical indexing requires encoding and decoding procedures to represent and recover direction information, which makes metadata processing more complex. Coding and decoding procedures involve trigonometric operations or table access, which prevent direct manipulations at the metadata level without adding additional complexity

Additionally, it is a common practice in existing object-based audio metadata to separate the fields for each coordinate. It is for example done in MPEG-H [3] and ADM [4] where individual metadata attribute is allocated to each coordinate.

Coordinate resolution is also an important aspect and requirements could vary depending on the application. In [2], it was suggested that 1 degree of resolution for all directions is probably sufficient. The source also shares this opinion, but by relaxing the efficiency aspect of directional representation coding in metadata, it is possible to achieve an even finer resolution at almost no cost. Indeed it is possible to code the azimuth and elevation each on 12 bits with an angular precision of 0.1 degree. For example, MPEG-H metadata offers two angular precisions of 1 and 5 degrees. For 1 degree of resolution, azimuth and elevation are coded on 9 and 8 bits, respectively. In ADM, the angle is given in degree with a decimal precision in XML code.

# Proposal

The source is of the opinion that the advantages of the spherical indexing proposed in [2] are not worth compromising the flexibility and readability of the metadata. Therefore, the source proposes to represent individual spherical coordinates on separate metadata attributes. Since byte alignment can also be a desired feature, the source proposes to code each azimuth and elevation angle on 12 bits for a total of 24 bits per object direction, which is byte aligned with the other fields. The 12-bits field allows an angular precision of 0.1 degree more than sufficient for the intended applications.

In this regard, Table 1 presents a proposal on metadata for audio object (without reviewing Distance, Gain, and Spatial extent).

Table 1. Proposed metadata for object audio

| Field | Bits | Description |
| --- | --- | --- |
| Azimuth | 12 | Azimuth in degree with a resolution of 0.1 degree |
| Elevation | 12 | Elevation in degree with a resolution of 0.1 degree |
| Distance | TBD | TBD |
| Gain | TBD | TBD |
| Spatial extent | TBD | TBD |

# Conclusion

This input document presented some additional considerations over [2] on object-based audio metadata and the way the object positions should be represented. The source suggests to adopt a simple representation of the direction in metadata keeping separated the azimuth and elevation in separated fields.

# References

[1]	Tdoc S4-181218: IVAS Design Constraints (IVAS-4), v0.0.6.

[2]	Tdoc S4-190122: Considerations on object-based audio metadata for IVAS, Nokia Corporation

[3]	ISO/IEC JTC 1/SC 29, ISO/IEC FDIS 23008-3:2018(E), Information technology — High efficiency coding and media delivery in heterogeneous environments — Part 3: 3D audio

[4]	Recommendation ITU-R BS.2076-1, Audio Definition Model