## Source:	Fraunhofer IIS

## Title:	Reference Conditions for IVAS Testing

## Document for:	Agreement

## Agenda Item:	7.5

# 1	Introduction

This contribution proposes reference conditions for IVAS testing. It is asked to include these proposals into the IVAS performance requirements [1]. The proposals are built on the use of EVS, the 3GPP state-of-the-art speech and audio codec, for generating the reference conditions. Similar as for the draft performance requirements on stereo, it is further on proposed to use EVS in a multi-mono fashion to generate the reference conditions.

# 2	Proposal

It is proposed to use EVS for the creation of reference conditions used for evaluation in the IVAS performance requirements. EVS shall be used in the same bandwidth mode (e.g. WB, SWB, FB) as IVAS. EVS shall be used in the following manner to create the reference conditions:

- For stereo/binaural audio/multi-channel input: use 1x EVS instance per channel

- For object input: use 1x EVS instance per object, pass-through of object meta data

- For scene-based audio: use 1x EVS instance per SN3D-normalized Ambisonics coefficient

- For mixed scenes: Combinations of the above, i.e. one EVS instance per each channel, Ambisonics coefficient, or object

Figure 1: Coding of Stereo/Binaural Stereo/Multi-Channel reference conditions using EVS

Figure 2: Coding of Scene-based reference conditions using EVS

Figure 3: Coding of Object-based reference conditions using EVS

The output corresponds to the output of the proposed IVAS pass-through mode, i.e. it is in the same format as the input. Object meta data at the input is passed-through directly.

Using EVS mock-ups as a reference is a flexible approach, which would be the natural alternative to any IVAS solution. Therefore, such references at a given total bit rate have to be at least matched. There should also be improvement over a multi-mono EVS solution possible, and the bit rates of EVS should be chosen appropriately to set challenging, yet realistic, bars for IVAS’ performance requirements.

For simplicity identical bit rates across the multi-mono instances should be used. The following tables list the total bit rate of EVS multi-mono that span a wide range of state-of-the art coding quality (for mono instances) and spatial quality (by means of creating mock-up codecs). Note that not all combinations of EVS bit rate and number of instances of the potential mock-up systems may be useful references.

Table 1: Total Bit Rate (in kbps) of Multi-EVS for 1-8 instances

|  |  | Number of Instances |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EVS Bit Rate |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
|  | 7,2 | 7,2 | 14,4 | 21,6 | 28,8 | 36 | 43,2 | 50,4 | 57,6 |
|  | 8 | 8 | 16 | 24 | 32 | 40 | 48 | 56 | 64 |
|  | 9,6 | 9,6 | 19,2 | 28,8 | 38,4 | 48 | 57,6 | 67,2 | 76,8 |
|  | 13,2 | 13,2 | 26,4 | 39,6 | 52,8 | 66 | 79,2 | 92,4 | 105,6 |
|  | 16,4 | 16,4 | 32,8 | 49,2 | 65,6 | 82 | 98,4 | 114,8 | 131,2 |
|  | 24,4 | 24,4 | 48,8 | 73,2 | 97,6 | 122 | 146,4 | 170,8 | 195,2 |
|  | 32 | 32 | 64 | 96 | 128 | 160 | 192 | 224 | 256 |
|  | 48 | 48 | 96 | 144 | 192 | 240 | 288 | 336 | 384 |
|  | 64 | 64 | 128 | 192 | 256 | 320 | 384 | 448 | 512 |

Table 2: Total Bit Rate (in kbps) of Multi-EVS for 9-16 instances

|  |  | Number of Instances |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EVS Bit Rate |  | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
|  | 7,2 | 64,8 | 72 | 79,2 | 86,4 | 93,6 | 100,8 | 108 | 115,2 |
|  | 8 | 72 | 80 | 88 | 96 | 104 | 112 | 120 | 128 |
|  | 9,6 | 86,4 | 96 | 105,6 | 115,2 | 124,8 | 134,4 | 144 | 153,6 |
|  | 13,2 | 118,8 | 132 | 145,2 | 158,4 | 171,6 | 184,8 | 198 | 211,2 |
|  | 16,4 | 147,6 | 164 | 180,4 | 196,8 | 213,2 | 229,6 | 246 | 262,4 |
|  | 24,4 | 219,6 | 244 | 268,4 | 292,8 | 317,2 | 341,6 | 366 | 390,4 |
|  | 32 | 288 | 320 | 352 | 384 | 416 | 448 | 480 | 512 |
|  | 48 | 432 | 480 | 528 | 576 | 624 | 672 | 720 | 768 |
|  | 64 | 576 | 640 | 704 | 768 | 832 | 896 | 960 | 1024 |

# 3	Conclusion

A method on creating reference conditions for the IVAS performance requirements was presented. It is proposed to include Section 2 of this document into the IVAS Performance Requirements, Section 3.

Parties interested in the discussion on IVAS Performance Requirements are invited to contribute their proposals on challenging, yet realistic performance requirements based on EVS multi-mono.

# 4	References

[1] 3GPP S4-181219, IVAS Performance Requirements (IVAS-3)