3GPP TSG-SA WG4 Meeting #103			Tdoc S4-190473

8th – 12th Apr 2019, Newport Beach, CA, USA

Title:	DRAFT Reply LS (to RAN2) for inclusion of Receive Only Mode MBMS service parameters in USD

Response to:	RAN2

Release:	Release 15

Work Item:	TEI-15

Source:	SA4

To:	RAN2

Cc:	SA2, RAN1

Contact Person:

#### Name:	Charles Lo

E-mail Address:	clo@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR in S4-190467, "USD Information on MBMS ROM Parameters"

1. Overall Description:

SA4 thanks RAN2 for your LS (in R2-1818960/S4-181391) on inclusion of Receive Only Mode MBMS service parameters in the USD.

RAN2 requested SA4 to consider adding in the MBMS Service Announcement, or “USD”, RF information associated with the delivery of Receive Only Mode (ROM) services. Those ROM service related RF parameters include EARFCN, MBMS carrier numerology and bandwidth information. RAN2 indicated that this information is desirable for inclusion in the MBMS Interest Indication sent by the UE to inform the serving eNB that the UE is currently receiving, or is interested to receive, a ROM service from a different eNB, and to enable battery power savings in UE search for MBMS ROM frequencies.

SA4 discussed this matter and agrees that such (optional) inclusion of ROM service in the USD can be useful to the UE, for the reasons stated by RAN2 in their LS. As result, SA4 has decided to add, to the Rel-15 TS 26.346, an optional child element r15:ROMSvcRfParams under the userServiceDescription element of the USBD fragment. The r15:ROMSvcRfParams element contains a Frequency child element which in turn contains the attributes subcarrierSpacing and bandwidth. In offline discussion between SA4 and RAN2 members regarding the RAN2 LS, explicit and known values of those attributes should be defined. Specifically, the allowed values of subcarrierSpacing shall be one of the following numbers in kHz: 1.25, 7.5 or 15, and the allowed values of bandwidth shall be one of the following numbers in MHz: 1.4, 3, 5, 10, 15 and 20.

SA4 asks RAN2 to review the action taken by SA4 towards addressing your request. We have attached the approved CR to Rel-15 TS 26.346 on inclusion of ROM service parameters in the USD.

2. Actions:

To RAN2group

ACTION: 	SA4 asks RAN2 to review the above information, and determine whether the described changes made to the MBMS USD will fulfil your request, and let SA4 know if you have any comments, questions or concerns.

3. Date of Next TSG-SA WG4 Meetings:

SA4#104	5 – 7 July 2019		Cork, Ireland

SA4#105	12 – 16 August 2019		Ljubljana, Slovenia