Source:	HEAD acoustics GmbH, Intel

Title:	Status of round robin test for FS_ANTeM

Document for:	Discussion

Agenda Item:	9.7

# Introduction

This contribution highlight the status of the Round Robin test within the study item FS_ANTeM [1], which investigates different noise field simulations for acoustic terminal testing across different labs. The agreed test plan [2] describes test setups and measurements to conduct each.

# Devices

After the LS-out [3] to CTIA, they agreed to support the effort by providing four mobile phones. In return, CTIA kindly asked for some additional tests to perform in all participating labs (delay, loudness rating, transfer function and speech quality prediction according to ITU-T P.863 – for both, send and receive direction).

In overall, nine phones are available now, as shown in Table 1. All of them are capable of at least AMR and AMR-WB codec, EVS-SWB is supported by six devices. However, due to time constraints, not all phones may be tested in all bandwidth modes.

Table 2 summarizes the number of devices for the various cases.

Table 1: Devices for round robin test

| Device Nbr | Manufacturer | Year | AMR-NB/WB via VoLTE | Test in NB | Test in WB | Test in SWB |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | A | 2018 | no | yes | yes | yes |
| 2 | B | 2018 | yes | yes | yes | yes |
| 3 | C | 2015 | no | yes | yes | no |
| 4 | D | 2012 | no | yes | yes | no |
| 5 | E | 2014 | no | yes | yes | no |
| 6 | F | 2018 | yes | yes | yes | yes |
| 7 | F | 2017 | yes | yes | yes | yes |
| 8 | G | 2018 | yes | yes | yes | yes |
| 9 | H | 2018 | yes | yes | yes | yes |

Table 2: Summary of phone for each BW and access technology

| RAT | VoLTE |  |  | 3G |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth | EVS-SWB | AMR-WB | AMR-NB | AMR-WB | AMR-NB |
| Nbr Devices | 6 | 5 | 5 | 4 | 4 |

# Measurement rooms

The parameters of the measurement rooms evaluated in this study are described in Table 3. All rooms are equipped with sub-woofer setup for the playback according to ES 202 396-1 [4] and with eight satellite loudspeakers for the playback according to TS 103 224 [5].

Table 3: Measurement rooms

| Company | Name | Length [m] | Width [m] | Height [m] | Comment |
| --- | --- | --- | --- | --- | --- |
| HEAD acoustics GmbH | Room 1 | 2.40 | 3.40 | 2.05 | Same chamber type and design as Room 2 |
|  | Room 2 | 2.40 | 3.40 | 2.05 | Same chamber type and design as Room 1 |
|  | Room 3 | 2.90 | 3.10 | 2.05 | Different manufacturer than Room 1,2,4 |
|  | Room 4 | 1.80 | 2.40 | 2.05 | Smallest chamber |
| Intel | Room 5 | 5.57 | 4.67 | 2.93 | Trapezoidal room |
|  | Room 6 | 3.60 | 3.00 | 2.05 |  |

# Test Status

Due to the late availability of the phones provided by CTIA, the measurement are still ongoing. All the measurements are done for room 5 and ongoing for room 6.

It is expected to have all available data by the next 3GPP meeting SA#105 in August.

# Update of test plan

During the preparation and first executions of measurements, some details in the test plan were found in need of change. The sources propose to update the test plan [2] on the following issues:

- Include Table 1 with descriptions of devices.

- More specific description how to determine room parameters RT60 / C80, which should be reported.

- Add measurements requested by CTIA to the list of “basic measurements”.

# References

| [1] | 3GPP S4-181500, „New SID on ambient noise test methodology for evaluation of acoustic UE performance (FS_ANTeM),“ HEAD acoustics GmbH, Qualcomm Incorporated, Deutsche Telekom AG, Intel, HuaWei Technologies Co., Ltd, Busan, South Korea, 25 November 2018. |
| --- | --- |
| [2] | 3GPP S4-190447, „ Test plan for a Round-Robin-Test - comparison of noise field simulations for handset mode,“ HEAD acoustics GmbH, Intel, Newport Beach, USA, 17 April 2019. |
| [3] | 3GPP S4-190554, „LS on Round-robin test plan for FS_AnTEM (To: CTIA),“ TSG SA WG4, Newport Beach, USA, 17 April 2019. |
| [4] | ETSI ES 202 396-1 v1.7.1, „Speech Processing, Transmission and Quality Aspects (STQ); Speech quality performance,“ 10/2017. |
| [5] | ETSI TS 103 224 v1.3.1, „Speech and multimedia Transmission Quality (STQ);. A sound field reproduction method for terminal testing including a background noise database,“ 07/2017. |