Source:	Orange

Title:	Proposals for IVAS Design Constraints

Document for:	Agreement

Agenda Item:	7.5

## Introduction

The present document is based on S4-190450 and brings suggested changes to progress IVAS design constraints.

## Proposals

| Sampling Frequency and Audio Bandwidth | The encoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. The decoder shall support 16, 32, and 48 kHz sampling rates in all operation modes. Note: Functions that are necessary for achieving the arbitrary selection of encoder / decoder input / output sampling rate are envisioned to be a part of the IVAS candidate. The encoder shall support input signals with different input signal bandwidth (NB, WB, SWB, and FB) with frequency masks as defined for EVS. The encoder and decoder shall support 8kHz sampling when EVS bit-exact operation is used (See Backward Interoperability) |
| --- | --- |
| Encoder Input Formats | The encoder shall support the following input formats: - Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD] - Binaural audio - Scene-based audio, first-order (FOA) and up to [N]-order ambisonics. Note: ACN component ordering and SN3D normalization. - [Spatial audio, [N] channels and spatial metadata defined by [TBD].] [Editor’s Note FFS: Spatial metadata definition for the spatial audio format will require further input.] - Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters]. [In addition, the IVAS codec shall support combinations of the above, totalling to no more than [TBD] audio streams. Note: It will be necessary to specify how capture/presentations could be achieved in mobile communications.] |
| IVAS renderer | - Proponents shall provide a renderer solution as part of their IVAS candidate - The renderer will be specified in IVAS series |
| Rendered Output Formats | The IVAS codec shall support the following rendered output formats for the corresponding input format: Encoder Input Format; Renderered Output Format. Multi-channel 7.1; Multi-channel 7.1, Binaural audio, Stereo, Mono.. Multi-channel 5.1; Multi-channel 5.1, Binaural audio, Stereo, Mono.. Binaural audio; Binaural Audio, [Stereo], [Mono]. Stereo; Stereo, Mono. Mono; Mono. |
| External rendering | - Candidates shall provide interface specification to external renderer; requirements on the interface are TBD - Performance on external renderers for IVAS shall be defined in the scope of the ATIAS work item |
| Bit Rates | When EVS bit-exact operation is used (see Backward interoperability), the IVAS codec shall operate at bit rates of EVS (including all EVS Primary and AMR-WB IO modes) Note: As implemented in EVS, the bit rates up to 24.4 kb/s are gross bit rates and they are net bit rates above 24.4 kb/s. In other cases than EVS bit-exact operation: the IVAS codec shall operate at bit rates of [5.9VBR, 7.2, 8, 9.6, 13.2, 16.4], 24.4, 32, 48, 64, 96, 128, 160, 192, 256, [384, 512] kb/s [min and max TBD]. Note: The gross bit rate supported in the DTX/CNG/SID operation is [TBD]. |
| Algorithmic Delay | The algorithmic delay for the complete solution shall be less than or equal to TBD [Editor’s Note: The EVS Algorithmic delay is 32ms] |
| Complexity | The computational complexity and program ROM (PROM) of the candidate codecs shall be measured with [TBD] as the observed worst-case complexity for the complete solution. Requirements: TBD |
| Backward Interoperability | Having interoperability with the EVS is an important feature. The full EVS codec algorithm shall be part of the IVAS candidate codec solution. EVS bit-exact processing shall be used when the input to the IVAS codec is a simple mono signal without spatial metadata and should also be applied whenever possible. Possible exceptions for particular operation modes are [tbd]. When multiple mono audio channels without spatial metadata are negotiated they shall all be bit-exact with EVS. Possible exceptions are [tbd]. The IVAS Codec shall support certain stereo modes of operation which include an embedded bit-exact EVS mono downmix bitstream at the bit-rates from 9.6 kbit/s to 24.4 kbit/s SWB (9.6/13.2/16.4/24.4 kbit/s). Editor’s Note: Definition of bit-exactness for embedded stereo modes is FFS. Note 1: When the IVAS codec uses EVS bit-exact operation then features such as AMR-WB I/O mode, EVS SID update rates and 8kHz sample rate support shall be supported. [Note 2: TS 26.445 supports multiple mono channels in the same way as described in IETF RFC 4867, which supports multi-channel audio content by means of multiple mono channels. Use of the EVS multiple mono interoperability for encoding multi-channel audio is FFS. Being bit-exact with EVS for the multiple mono case would not be appropriate because channels are not synchronized in the EVS encoding of multiple mono channels which may result in different VAD decisions for different channels as noted in the section of A.2.5.1 of TS 26.445. Contributions are invited if some synchronization is necessary for such use case.] |
| Frame length | The candidate codecs shall operate with a frame size of 20 ms. |
| Jitter Buffer Management (JBM) | A JBM solution conforming to the requirements in TS 26.114, except for the functional requirement in sub-clause 8.2.2 of TS 26.114: “Speech JBM used in MTSI shall support all the codecs as defined in clause 5.2.1”, shall be provided with the candidate codecs. [Note: The JBM defined in TS 26.448 may form the basis of the JBM provided with the candidate codecs.] |
| Rate switching | [The candidate codecs shall perform rate switching upon command to the encoder throughout the entire bit rate range at arbitrary frame boundaries. The rate switching may imply switching between different bandwidths and between mono coding modes and coding modes for multiple audio streams.] |
| Packet loss concealment (PLC) | A PLC solution shall be provided by the IVAS candidate codecs. |
| RTP payload format | Candidate codecs shall provide an RTP payload format specification supporting the full set of features and functionality of the IVAS candidate codecs. |
| DTX | The candidate codecs shall provide a complete VAD/DTX/CNG framework. It shall be possible to operate the codec with DTX on or DTX off. SID update frames shall be sent with a frequency not exceeding once per 8 frames. |
| Interface for binaural rendering | The IVAS decoder/renderer shall provide an interface to provide [HRTF/BRIR] data for binaural rendering. The interface is [tbd]. [Note: There was some support for this interface to follow the SOFA SimpleFreeFieldHRIR convention - See AES69-2015]. The IVAS decoder/renderer shall provide an API to provide [TBD scene displacement data]. [The IVAS decoder/renderer shall support direct headphone presentation.] |
| Control Data For Binaural Audio Rendering | The IVAS decoder/renderer shall support the following control data for binaural audio rendering: [HRTF/BRIR] data for binaural rendering on command line interface. The format for [HRTF/BRIR] data is [tbd]. [Note: There was some support for this interface to follow the SOFA SimpleFreeFieldHRIR convention - See AES69-2015]. [TBD scene displacement data]. |
| Direct headphone presentation | The IVAS codec shall support direct headphone presentation for non-diegetic signals, with metadata indicating if the decoded non-diegetic signal has one channel (with an associated panning gain) or two channels (stereo or binaural). [Ericsson propsal: Editor’s Note 1: A definition of diegetic/non-diegetic content is needed. Editor’s Note 2: It should be clarified where the panning gain is specified, encoder or decoder side.] |
| [Decoder/Renderer Motion to Sound Algorithmic Delay] | [The maximum algorithmic delay from a detected change in head roll, azimuth & elevation to a binaural sound rendered within +/- [Y] degree(s) of the detected change shall be [20 ms].] |
| Output gain limitation | TBD |

[Editor’s Note FFS: Tdoc S4-171221 proposes to add high-level design constraints for IVAS codec modes suitable for a spatial conferencing use-case and if agreed, corresponding updates to the design constraints would be made.]

[Definitions]

Binaural audio: Binaural audio is defined as a two-channel spatial representation of a soundfield as typically captured at the entrance of the ear canals and intended for direct presentation to the left and right ears over headphones. In terms of spatial representation, binaural audio may be natural (truly recorded with microphones) or artificial (e.g. using HRTFs). No additional spatialization (e.g. by an additional HRTF/BRIR convolution) should be carried out before direct presentation over headphones.

## List of action items for processing plan

The following issue should be discussed for the processing plan:

- Signaling to indicate binaural audio and other input types

- Encoder interface for optional indication of the expected playback format (if such feature is supported by the candidate payload format)

- Interface to provide [HRTF/BRIR] data for binaural rendering