3GPP TSG-SA WG#4 Meeting #105	S4-190906

# Ljubljana, Slovenia, 12-16 August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0002 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Unicast Streaming Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 25 June 2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When an external Application Service Provider leverages the 5G Media Streaming Architecture, the external AS needs to first interact with the MNO (e.g. using M1d). |  |  |  |  |  |  |  |  |
| Summary of change: |  | An SLA negotiation step is inserted into downlink streaming procedures so that the network external application server provider can negotiate the agreement and get Media AF and / or Media AS access information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First Change *****

## 5.2	Establish a Unicast Media Downlink Streaming Session

This procedure describes the establishment of a Unicast Downlink Streaming Session. A streaming session may use DASH format, 3GP File Format (Progressive Download), 3GP Time Text or other (potentially non-3GPP defined) formats.

Figure 5.2-1: High Level Procedure for progressive download for on-demand media

Steps:

1: The External Application Service provider interacts with the trusted Media Functions (e.g. using reference point M1d) information about accessing the Media Application Servers (Media AS) and the Media Application Functions (Media AF) within the trusted network functions is provided to the external AS.

2: The App triggers the Service and Content Discovery procedure.

The Content Discovery procedure only involves the App and the external Application Server and hence is out of scope of the present document. The External AS provides information for the Media Player Entry points and the Media AFs to the App.

3: A Media Player Entry is selected.

4: The App triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The 5GMS Player establishes the transport session.

7: The 5GMS Player sends the request for the progressive download content.

8: The 5GMS Player receives the initialization information of the progressive download content. The initialization information contains configuration parameters for reception of the media, and optionally also DRM information.

9: The 5GMS Player configures the rendering pipeline for media playback.

10: The 5GMS Player notifies the media session handler, providing the transport session information and some media content related information.

11: Optional: the media session handler establishes a media session with the Media AF. Additional parameters such as the transport session parameters are provided.

12: Optional: 5GMS Player acquires a DRM License from an External AS.

13: The 5GMS Player receives media content and puts it into the rendering pipeline.

14: The 5GMS Player continuously receives and plays back the media content.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.2-2: High Level Procedure for DASH content

Steps:

1: The External Application Service provider interacts with the trusted Media Functions (e.g. using reference point M1d). Information about accessing the Media Application Servers (Media AS) and the Media Application Functions (Media AF) within the trusted network functions is provided to the external AS.

2: The App triggers the Service and Content Discovery procedure. The Content Discovery procedure only involves the App and the external Application Server. The External AS provides information for the Media Player Entry points and the Media AFs to the App.

3: A media content item is selected.

4: The App triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

5: The Media Session Handler triggers the 5GMS Player to start the session.

6: The 5GMS Player establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

7: The 5GMS Player requests the manifest

8: The media access client receives the manifest

9: The 5GMS Player processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

10: The 5GMS Player notifies the media session handler about the manifest. The notification may contain parameters from the manifest

11: Optional.The Media Session handler establishes session with the Media AF.

12: Optional: the 5GMS Player acquires the necessary DRM information, for example a DRM License.

13: The 5GMS Player configures the media rendering pipeline.

14: The 5GMS Player establishes the necessary transport sessions for the content. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

15: The media session handler is notified about the established transport sessions for the content.

16: Optional: The media session handler may notify the media AF about the session and the established transport sessions.

17: The 5GMS Player requests initialization information. The 5GMSP repeats this step for each required initialization segment.

18: The 5GMS Player receives the initialization information.

19: The 5GMS Player requests media segments according to the manifest.

20: the 5GMS Player receives media segments and puts the information into the according media rendering pipeline.

21: Previous steps are repeated according to the manifest information.