Source:	Nokia Corporation

Title:	On shared control metadata for IVAS input audio formats

Document for:	Discussion & Agreement

Agenda Item:	7.5 – IVAS_Codec

# 1. Introduction

In [1], the source presented some real-time conversational use cases and discussion on potential control metadata to be considered for IVAS specifically in case of object-based input. Particularly, it was seen that conversational use cases and their requirements differentiate object-based audio for IVAS from object-based audio traditionally used, e.g., for broadcast applications.

In [2], Dolby Laboratories proposed to work on a consistent input audio and session metadata specification. In view of the source, such specification for IVAS should include at least control metadata that enables consistent experience for new immersive services. In this input document, shared control metadata requirements for IVAS audio inputs and combinations of input formats are discussed. It is furthermore proposed to define IVAS support for a minimum set on control metadata spanning all codec audio inputs.

# 2. Discussion

The IVAS codec is expected to significantly expand the opportunities for immersive audio services and advanced use cases over EVS. This can in part be achieved via support of various input audio formats and some input audio format combinations. It can be expected that at least object-based audio is considered in combination with some other formats. The draft IVAS design constraints (IVAS-4) [3] already specify support for an object-based audio format. However, the exact audio format definition is still TBD.

The role of the EVS codec, and prior conversational codecs, can be seen as a compression engine for the mono voice or audio signal. The wider scope of the immersive services and use cases challenges this thinking for IVAS. It is not only the audio waveform that is being transmitted, but rather an experience that may be real or simulated and allow for various manipulations by the transmitting or particularly the receiving user. Thus, on one hand, we may wish to continue to view the IVAS codec as a compression engine only. On the other hand, we may view it as an enabling technology for the new conversational immersive services and use cases. According to this latter view, there appears a need to include control metadata to specify the relationships of audio streams and the constraints within which, e.g., manipulations are allowed.

We believe control metadata need not be limited to object-based audio. Rather it should additionally be considered at least cases where the IVAS encoder receives combinations of input audio formats. At high bit rates, there may not be much need to consider the encoder functionalities for the various decoder/renderer scene modifications etc. It seems, however, quite obvious that certain input specific information can be useful in the encoder in order to achieve the best possible compression performance towards the lower end of the envisaged bitrate range. In other words, in low-bitrate operation, the encoder can be expected to benefit from information that relates to the input audio streams. For example, should there be information specifying whether a specific audio object is required to be separable at the decoder/renderer or not, very different encoding approaches may be possible.

In view of the source, it will thus become relevant, e.g., for setting of performance requirements (and in the end IVAS candidates meeting or exceeding them) to understand what kind of control information or metadata IVAS will support. This can also have significant service implications in terms of achieving certain audio quality at relevant bit rates.

# 3. Common control metadata for IVAS input formats

At least for various communications scenarios, it is expected that there will be one or more audio inputs to the IVAS encoder that will carry a user voice signal. While rich immersive experiences generally require all signals to pass, there may be for example bitrate constraints that do not always make this possible at high quality. It could still be expected that the capture device/system provides a multitude of signals to the encoder. For communications, at least the voice signal is always needed, and it should also appear clear, natural, understandable and distinguishable. Thus, downmixing all signals into one stereo or one mono waveform may not be desirable.

Based on the above consideration, it seems that there is a need to be prioritize signals (codec inputs) for encoding. For example, instead of applying a downmix, it could then be dropped less important inputs at low-bitrate operation points. Additionally, it seems to be particularly useful to have an indication of an input being intended to carry the voice signal(s).

Thus, we propose for inclusion in IVAS-4 that the IVAS encoder:

- Shall support indication of voice signal input (e.g., flag voice/not voice).

- Shall support audio input prioritization information.

Whether each input is required to carry the voice flag and/or the priority information or a default operation in absence of such metadata can be assumed is FFS.

In addition, support for indication of voice signal channel may be considered. For example, a stereo input signal from some system could carry a voice signal in L channel only. Or, for example, it could be indicated that a 5.1 signal has all voice content in C channel only. This consideration specifically relates to a communications scenario, not pre-mixed commercial content.

# 4. Control metadata for consideration

As already discussed in [1], it is noted some parameters or their effects in rendering for which it would be needed control in IVAS. These include:

- Restrictions of rendering parameters of audio objects (default being no restriction)

- Linking of audio objects (e.g., a pair of audio objects are alternatives in some way)

- Role of the audio object in rendering (e.g., the audio object is not meant to be played back without a spatial bed or some linked audio object)

Based on these observations, the following table of initial “audio object types” was collected for consideration.

| “Name” | General properties | Description (E.g., use case where useful) |
| --- | --- | --- |
| Default | - Separable at the decoder - User-controllable in rendering | Potential default audio object type with full user control. Audio object is mixed in rendering according to any user manipulation or the default metadata instructions. For example, this can be a voice on teleconference that listener is free to move in the space and adjust volume separately etc. |
| Restricted control | - Separable at the decoder - User control in rendering allowed but restricted - Rules on rendering provided | Audio object type with restricted user control. List of restricted rendering parameters provided. For example, the effect of audio focus can be delivered as an audio object that the user can apply to rendering or not (or, e.g., control amount of audio focus using a slider). In this case, user is not allowed to change position of audio object relative to spatial signal and object can be assumed meaningless without the spatial signal. |
| Toggled pair (“alternative and alternating”) | - Separable at the decoder - ‘Alternative sound’ for a linked audio object (with default playback of first one only) - Selecting rendering of one disables the other (“toggling”) | Audio object type allows for toggling between two specific audio objects based on user control. For example, user can for a downstream audio select on-the-fly between two (or more) language tracks. |
| Relatively controlled pair (“alternative but concurrent”) | - Separable at the decoder - ‘Alternative sound’ for a linked audio object (with default playback of both) | Audio object type allows for relative control of two specific audio objects based on user control. For example, user can directly adjust the balance between two audio tracks. Service or content can indicate which objects are together and simplify user control. |
| Additive only (with restricted control) | - Need not be separable at decoder (although may still be preferable) - User control in rendering allowed but restricted - At least some rules on rendering provided - Audio object is mixed with linked audio based on user control - Default operation (mixing on/off) can be an encoder input | Audio object type allows for adding an enhancement or change to audio scene, where the enhancement or change may not be desirable to be presented to user without a spatial audio bed. For example, when separable at decoder, the effect of audio focus can be delivered as an audio object that the user can apply to rendering or not (or, e.g., control amount of audio focus using a slider). Default operation says whether to downmix in encoder or not, if separable transmission is not possible (e.g., due to bit rate). |

We consider it important to specify the IVAS functionality such that advanced audio experiences can be enabled with the codec. Thus, the three types of control metadata should be discussed and considered in the IVAS metadata specification (at least for audio objects).

# 5. Summary

The source considers it important to define some control metadata particularly to address the use cases relating to combinations of input audio formats. In the current document, a discussion on shared control metadata for combinations of input audio formats was presented. This includes indication for voice signal input and input audio prioritization.

In addition, the source believes that object-based audio in IVAS will need a control metadata definition where at least some of the aspects already discussed in [1] are properly addressed.

We propose the following:

- Agree on minimum shared control metadata presented in clause 3.

- Agree on defining a set of shared and object-based audio control metadata addressing advanced use cases / requirements as discussed in clause 4.

# References

[1]	Tdoc S4-190396: “Examples of object-based audio control metadata for IVAS”. Source: Nokia Corporation

[2]	Tdoc S4-190735: “On Input Audio and Session Metadata and Behavioral Requirements of the IVAS Decoder/Renderer”. Source: Dolby Laboratories, Inc.

[3]	Tdoc S4-190839: IVAS Design Constraints (IVAS-4), v0.1.0.