## Source:	Fraunhofer IIS

## Title:	Content Types for IVAS Testing

## Document for:	Discussion and Agreement

## Agenda Item:	7.5

# 1	Introduction

This contribution discusses the content types used for testing IVAS candidates’ performance to be specified in [1]. It consists of a brief discussion on EVS content types, necessities for IVAS, practical issues, and finally a proposal for a set of content types.

# 2.	EVS Content Types

For the EVS performance requirements [2], and in the course of the EVS qualification, selection, and characterization phases, multiple content types that are relevant for the EVS service have been considered, that can generally be subdivided into clean speech, speech with background noise (“noisy speech”), and mixed content/music. The following information is extracted from EVS-3 (with text in italics to indicate text added for completeness):

For clean speech requirements:

- Different input levels (-26dBov, -16dBov, -36dBov)

For noisy speech requirements:

- Three different noise types (Car, Office, Street). Subsets of noise types can be tested for Qualification, Selection, and Characterization

- The SNR values depend on the kind of noise processing applied and the frequency characteristics of the noise files. The following noise types and SNR values are agreed during qualification:

- NB: Car noise at SNR=15 dB

- WB: Street noise at SNR=20 dB

- SWB: Office noise at SNR=20 dB

- The SNR values and allocation of noise types to operation modes are defined in the selection / characterization test and processing plans.

For mixed content and music requirements:

- The definition of mixed content is given in Annexes A and B

- Mixed content and music will be tested at a single level, however the mixed content and music material used for testing should contain the full dynamic range for these input signals

# Annex A: Definition of the Mixed content to be tested in qualification

- Speech over music / music over speech and speech between music / music between speech (e.g., variable level difference, fade-in/out, including speech over low-level music in the background at 20 dB and alternating speech and music)

- Real examples which reflect some application scenarios (see notes)

Notes:

- 1) will be mixed by processing labs and 2) will not be.

- 1) is defined to test them in controlled conditions solving issues on language dependency, SNR, testing material dependency by different test labs, etc. Speech in test items should be understandable

- 2) is intended to contain cases that may not be covered by 1).

- Followings are some examples of application scenarios to be considered (The final list will be determined by contribution, particularly of test signals):

Cellphone transmission of live music session, answering speech, music on hold,  future answering system in company capable with SWB, music with announcement, call to customer service, put on hold, with advertisement, ticket number, professional editing of speech and music, ring-back sound, captured sound from e.g., living room, stadium, disco, concert hall, background music behind speech, etc.

# Annex B: Definition of the Mixed content to be tested in selection

Each music and mixed content selection listening experiment shall contain mixed content items to be composed of the following types;

- Radio Jingle, Movie Trailer

- Advertisement

- On hold content / answering machine message

- Speech over back ground music

The number of items per type shall be equal.

Each mixed content item shall reflect at least one of the following conditions:

- speech over music/audio

- speech between music/audio or music/audio between speech or alternating speech and music/audio

For Stereo content further details on content types were not defined, besides the generic statement on required performance for ‘certain correlated stereo content’:

- For certain correlated stereo content (e.g. speech in conferencing use case, mixed/music, binaural signals), EVS stereo operation shall be not worse than EVS dual-mono operation at (at least) next higher bit rate or better than EVS dual-mono operation at the same rate. Table 13 describes the details of these requirements.

# 3.	IVAS Content Types

IVAS is not only a Stereo extension to EVS, but also aims at providing deeper immersion with support for multi-channel, scene-based and object-based audio. For the Stereo part however, some of the work done for EVS can be re-used to define content types for IVAS Stereo.

The certain correlated stereo content envisioned for EVS Stereo remains valid, which includes e.g. speech in conferencing use case, mixed/music and binaural signals. While binaural signals seem to be a somewhat own format now in IVAS-4 [3], it still becomes evident that speech signals in conferencing scenarios are of relevance, as are mixed-content and music signals. Thus, the categories ‘clean speech’, ‘noisy speech’, ‘mixed/music’ seem still applicable.

While ‘mixed/music’ could often be ‘canned content’, e.g. background music that has been produced in stereo, speech content will be dominantly live content, that is recorded using a number of microphones.

The particular microphone setup plays a big role here, defining the characteristics of the stereo speech. In case of professional recordings there are A-B, M/S, X-Y, ORTF, Binaural (using e.g. HATS) and a multitude of other special microphone setups in use. Previous ITU-T work on stereo coding has mostly focused on use-cases with conference rooms being involved (see e.g. ITU-T STL2019) and seems of higher relevance for a communication codec. While those conferencing scenarios are also applicable to IVAS, the source proposes to focus on scenarios where the speech is UE-recorded, e.g. with smartphones. This means that microphone setups follow the small form factors and are mostly A-B, i.e. spaced omni-directional microphones. Thus, “the list” of relevant microphone setups for recorded speech is derived (in descending priority):

- A-B with small (e.g. 5-15cm) distance

- A-B with medium (e.g. >15cm - <100cm) distance

- A-B with large (e.g. >=1m) distance

- M/S

- Binaurally recorded or rendered

Those setups seem to be sufficient to cover the most important use cases and are all by definition correlated. In general, the recordings shall be of high quality (low recording noise, zero to little acoustic pre-processing) to not mask any artefacts of IVAS candidates. The recorded talkers should be placed at various angles during recording to pan them.

For clean speech the performance requirements shall be based on microphones selected from the list above and selected talker positions.

For noisy speech the performance requirements shall be based on microphones selected from the list above. Clean speech foreground content shall be mixed with ambient noise recordings using the same microphone setup as the clean speech foreground content.

For mixed/music the performance requirements shall be based on available stereo recordings as e.g. available on compact disc.

# 3	Conclusion

In this contribution content types of relevance for IVAS have been discussed by reviewing what has been done and planned for EVS (including EVS Stereo operation). In general, the same content types as used during EVS standardization can be re-used for Stereo operation. The microphone setups add another dimension though, and a list of configurations of relevance has been proposed. While the content types seem also applicable to the other input formats defined for IVAS, the source considers them for further study.

# 4	References

[1] 3GPP S4-181219, IVAS Performance Requirements (IVAS-3), v0.0.3

[2] 3GPP S4-130522, EVS Performance Requirements (EVS-3), v1.4

[3] 3GPP S4-190839, IVAS Design Constraints (IVAS-4), v0.1.0