## Source:	Fraunhofer IIS

## Title:	On the Importance of the Pass-Through Mode

## Document for:	Agreement

## Agenda Item:	7.5

# 1	Introduction

A significant amount of discussion on the so-called “Pass-through mode” for IVAS was observed at the last SA4 meetings. The source would like to emphasize once again the importance of the pass-through operation for channel-, binaural-, scene- and object-based input and asks to agree at least on the general concept for above mentioned audio formats.

# 2	Discussion

The concept of the pass-through mode for IVAS was first introduced in [1] and further elaborated in [2]. Further discussion was taking place e.g. in [3], where also the following arguments were listed:

- Traditionally, speech and audio codecs are neutral with respect to passing through the input format also to the output, i.e. the format on input is also expected to be transmitted as is. It is not clear to the source why IVAS should break with this concept.

- Only the pass-through mode guarantees an unrestrained audio-quality: A maximum audio quality can only be achieved if the scene is transmitted as is and no assumptions on e.g. rendering are made on encoder-side, which are possibly not necessarily matched to the decoder-side (e.g. because this is unknown or because of multiple recipients with different UEs).

- Only the pass-through mode guarantees an unrestrained flexibility: Especially for interactive use-cases, a transmission of channels, objects and SBA components is crucial, because only this allows manipulation on receiver side (e.g. rotation, amplification/attenuation) as needed for these use-cases. This holds true especially for use-cases in the context of 3DoF or conferencing. A rendering performed before transmission would destroy this flexibility.

- An interface for an external renderer is an important key component for manufacturers. It allows the use of individualized rendering technology and is thus important for differentiation. The pass-through of channels, objects and SBA audio could act as such interface and would allow for unrestrained quality for such external renderer technology.

- When comparing rendering technology currently on the market, a high degree of artistic freedom can be observed. This makes the comparison of codecs bound to their individual renderer-technology extremely difficult, because of the differences in the rendering. Only the pass-through mode in combination with a common (evaluation) renderer allows for a fair comparison between candidates, which is seen as an important sampling point when evaluating different candidates submitted for selection or when comparing candidates to reference codecs for the performance requirements.

In addition, the source feels that one important aspect was not addressed in the discussion in a sufficient manner, namely use-cases that do not solely consist of one single one-to-one connection, but connections that feature a multitude of participants. These participants will typically get combined either in an intermediate processing stage (e.g. MCU) or directly on the UE. Since conferencing is an important use case for IVAS, the source feels that this is an important aspect to consider:

- Conversion or pre-rendering of the input audio is problematic, because not all receiving devices might have identical rendering capabilities: A conversion/pre-rendering which might work well for device A might be problematic for device B.
Example: Binaural rendering typically works well for headphone reproduction but not necessarily for loudspeaker setups.

- Conversion or pre-rendering of the input audio might significantly reduce the capability to combine and render multiple sources either at the MCU or the receiving UE in a perceptually sensible way: As soon as multiple individual sources are rendered to the same position, intelligibility and perceived quality drastically drops, which significantly impacts the complete use-case.

- Conversion or pre-rendering of the input audio might significantly limit abilities to manipulate level or position of individual sources: Transmission of individual objects guarantees a high degree of flexibility when it comes to reposition or attenuation/gain individual sources. This advantage of individual objects becomes void once converted to different formatted or pre-rendered to a specific setup.

This can be avoided only if the audio sources survive the codec as is, i.e. individual objects remain individual objects, scene-based audio remains scene-based audio and channel-based audio remains channel-based audio.

# 3	Proposal

The source proposes to agree on the concept of pass-through operation for channel-based, binaural, scene-based and object-based audio and include this into [4]:

*** Start change 1 ***

The IVAS codec shall support pass-through operation for the following audio formats:

- Channel-based audio, including mono (1.0), stereo (2.0), surround (5.1 and 7.1), [surround + height (5.1+4 and 7.1+4), TBD]

- Binaural audio

- Scene-based audio, first-order (FOA) and up to [N]-order Ambisonics.

- Object-based audio, with support for at least [TBD] individual [mono] object streams. Each audio object shall be defined by [TBD metadata parameters].

For pass-through operation, the decoder output format is identical to the encoder input format according to the following definition:

- Channel-based audio: For pass-through operation, the output channel configuration shall be the same as the input channel configuration (e.g. 7.1 input shall result in 7.1 output).

- Binaural audio: For pass-through operation, binaural audio input shall result in binaural audio output.

- Scene-based audio: For pass-through operation, scene-based audio input shall result in scene-based audio output of the same order as the input.

- Object-based audio: For pass-through operation, audio objects input shall result in audio object output of the exact same number of audio objects. Quantization might be applied to accompanying metadata, so not all metadata attributes might be preserved.

- For mixed formats, the same principle applies accordingly for the individual components.

*** End of changes ***

# 4	References

[1] 3GPP S4-180823, On IVAS input-output formats and rendering
[2] 3GPP S4-181127, On IVAS input and output formats
[3] 3GPP S4-190692, On the Importance of the Pass-Through Mode
[4] 3GPP S4-190839, IVAS Design Constraints (IVAS-4), v0.1.0