Source:	Qualcomm Incorporated.

Title:	Measuring Receive Loudness Rating and Sensitivity/Frequency Characteristics on Commercial HaNTE Devices

Document for:	Discussion

Agenda Item:	9.6

# 1	Introduction

3GPP handset terminal acoustic test methodologies [1] and requirements [2] were developed for UE designs supporting traditional earpieces. However, handset manufacturers are now selling UE's featuring non-traditional earpieces (HaNTE devices) such as vibrating display handsets. In the present study, standard acoustic terminal tests defined in [1] are performed on two commercially available HaNTE devices to determine the applicability of such methodologies to vibrating display handsets. The measurements are repeated with multiple mounting positions and at several measurement points along the handset's display. Variability and considerations specific to HaNTE acoustic measurements are discussed.

# 2	Test Setup

Two measurements are made for each device:

- Receiving Loudness Rating (RLR) defined in clause 8.2.2.2 of [1]

- Receiving Sensitivity/Frequency Characteristics (RFR) defined in clause 8.4.2 of [1]

All measurements are conducted in an acoustically isolated anechoic chamber. Two head and torso simulators (HATS) are used for the acoustic measurements, each with its own handset positioning system. Figure 1 shows the two HATS with their respective handset positioners.

Figure 1: HATS 1 and 2 with handset positioners

Measurements are made with an MECRP of ye=17mm, ze=0mm for all devices. HATS 1 measurements are made on a grid of 9 additional MECRP shifts taken from clause 5 of [3]. Figure 2 demonstrates the 10 measurement points.

Figure 2: Arbitrary MECRP (red) and all other RLR/RFR measurement points for HATS 1

HATS 1 measurements are repeated under two handset support conditions shown in Figure 3. Mounting Position 1 (MP1) is a standard device mounting with support pins placed on the back of the handset. In Mounting Position 2 (MP2), the supporting pins are retracted, and the bottom fork is adjusted lower on the handset.

Figure 3: Variable handset support conditions used for HATS 1 measurements

On HATS 2, devices are mounted according to ITU-T Rec. P.64 (A = 21.2°, B = -12.9°, C = 2.3°). Due to contact with the HATS 1 cheek in the standard mounting position, a 5⁰ offset is added to the B angle for measurements on HATS 2.

Device volume is set as close as possible to the nominal RLR of 2 ± 3dB at the defined MECRP to comply with clause 6.2.2 of [2]. Test calls are established using the AMR-WB codec at 12.65 kbps. In what follows, RLR and RFR measurement results are provided for the two commercial HaNTE devices.

# 3	Measurement Results

## 3.1	Device 1

### 3.1.1	HATS 1

#### 3.1.1.1	Receiver Loudness Rating Results

Device 1 was adjusted to a nominal RLR of 1.91dB at the MECRP with MP1 on HATS 1. Table 1 presents Device 1 RLR results for all MECRP shifts at MP1 and MP2 on HATS 1. Figure 4 provides a visualization of the positions which pass the requirements specified in [2].

Table 1: Device 1 RLR results (bold* = within 2±3dB tolerance [2])

| Position |  | RLR with MP 1 (dB) | RLR with MP 2 (dB) | Difference |
| --- | --- | --- | --- | --- |
| ye | ze |  |  |  |
| 17 | 0 | 1.91* | 3.32 | -1.41 |
| 10 | -10 | 7.99 | 10.53 | -2.54 |
| 10 | 0 | 9.79 | 12.00 | -2.21 |
| 10 | 10 | 13.83 | 16.22 | -2.39 |
| 20 | -10 | -0.29* | -0.20 | -0.09 |
| 20 | 0 | 1.01* | 0.98 | 0.03 |
| 20 | 10 | 3.84* | 3.75 | 0.09 |
| 30 | -10 | -0.86* | -3.35 | 2.49 |
| 30 | 0 | 0.74* | -1.48 | 2.22 |
| 30 | 10 | 3.42* | 1.01 | 2.41 |
| Mean |  | 4.14* | 4.28 | -0.14 |

Figure 4: Device 1 RLR results with MP1 (left) and MP2 (right) (green = pass, red = fail)

Seven of ten tested points (including the MECRP) are within the RLR tolerance specified in [2] for MP1 and five of ten for MP2. The average RLRs for both MP1 (4.14dB) and MP2 (4.28dB) lie within the tolerance as well. MP1 RLR results are on average 0.14dB louder than MP2 results with a maximum variation of -2.54 dB at ye=10mm, yz=-10mm. For all ye = 10 positions, MP1 has a higher loudness rating than MP2. For positions with ye = 30, MP1 has a lower loudness rating. And for ye = 20, the RLR is consistent between MP1 and MP2.

#### 3.1.1.2	Receiver Sensitivity/Frequency Response Results

Using the previously defined nominal RLR of 1.68dB at the MECRP, Device 1 RFRs are measured on HATS 1 with two mounting positions at 10 points along the display. Figure 5 shows all RFR measurements (normalized to 0dB[Pa/V] maximum) along with the wideband tolerance from [2]. The arithmetic mean for each mounting position is presented as well. All individual measurements fail the wideband mask. Table 2 reports the maximum deviation from the tolerance mask for each measurement.

Figure 5: Device 1 RFR measured on HATS 1 at two mounting positions

Table 2: Device 1 RFR errors on HATS 1

| Position |  | MP1 Error (dB) | MP2 Error (dB) |
| --- | --- | --- | --- |
| ye | ze |  |  |
| 17 | 0 | -4.12 | -2.07 |
| 10 | -10 | -3.54 | -7.34 |
| 10 | 0 | -3.11 | -6.96 |
| 10 | 10 | -5.68 | -7.55 |
| 20 | -10 | -3.23 | -2.37 |
| 20 | 0 | -4.27 | -4.42 |
| 20 | 10 | -6.20 | -5.87 |
| 30 | -10 | -2.57 | -1.58 |
| 30 | 0 | -0.39 | -1.34 |
| 30 | 10 | -5.69 | -4.23 |
| Mean Error |  | -3.88 | -4.37 |

Variability of RFR results between MP1 and MP2 is presented in Figure 6. A positive value indicates higher sensitivity with MP1. For Device 1, there is a peak in variability (~4dB) at 550Hz and 1.8kHz for all measurement points. Variability never exceeds 6dB.

Figure 6: Device 1 RFR mounting variability

### 3.1.2	HATS 2 Measurements

Device 1 RFR was also measured on HATS 2 with the previously defined MECRP. Device playback was calibrated to a nominal RLR of 0.61dB. Figure 7 shows the HATS 2 results along with HATS 1 MECRP and average RFRs using MP1. Device 1 passes the RFR mask from [2] when measured on HATS 2.

Figure 7: Device 1 RFR measured on HATS 2

## 3.2	Device 2

### 3.2.1	HATS 1 Measurements

#### 3.2.1.1	Receiver Loudness Rating Results

Device 2 was adjusted to a nominal RLR of -0.11dB at the MECRP with MP1. Table 3 presents Device 1 RLR results for all MECRP shifts at MP1 and MP2. Figure 8 provides a visualization of which positions pass the requirements specified in [2].

Table 3: Device 2 RLR results (bold* = within 2±3dB tolerance [2])

| Position |  | RLR with MP1 (dB) | RLR with MP2 (dB) | Difference |
| --- | --- | --- | --- | --- |
| ye | ze |  |  |  |
| 17 | 0 | -0.11* | 0.91* | -1.02 |
| 10 | -10 | 8.17 | 9.59 | -1.42 |
| 10 | 0 | 7.39 | 8.59 | -1.2 |
| 10 | 10 | 7.04 | 7.78 | -0.74 |
| 20 | -10 | 0.34* | -0.12* | 0.46 |
| 20 | 0 | -1.09 | -1.09 | 0.00 |
| 20 | 10 | -1.79 | -2.02 | 0.23 |
| 30 | -10 | 2.18* | 0.04* | 2.14 |
| 30 | 0 | 0.67* | -0.95* | 1.62 |
| 30 | 10 | 0.15* | -1.72 | 1.87 |
| Mean |  | 2.30* | 2.10* | 0.19 |

Figure 8: Device 2 RLR results with MP1 (left) and MP2 (right) (green = pass, red = fail)

Of the ten points tested, five (including the MECRP) are within the RLR tolerance specified in [2] for MP1 and four (including the MECRP) for MP2. The average RLRs for both MP1 (2.30) and MP2 (2.10) lie within the tolerance. MP2 RLR results are on average 0.19dB louder than MP1 with a maximum variation of 2.14dB at ye=30mm, yz=-10mm.

#### 3.2.1.2 Receiver Sensitivity/Frequency Response Results

Using the previously defined nominal RLR of -0.11dB at the MECRP, Device 2 RFRs are measured on HATS 1 with two mounting positions at 10 points along the display. Figure 9 shows all RFR measurements (normalized to 0dB[Pa/V] maximum) along with the wide-band tolerance from [2]. Furthermore, the arithmetic mean for each mounting position is presented. All individual measurements as well as the mean RFR fail the wide-band mask. Table 4 reports the maximum deviation from the tolerance mask for each measurement

Figure 9: Device 2 RFR measured on HATS 1 at two mounting positions

Table 4: Device 2 RFR errors on HATS 1

| Position |  | MP1 Error (dB) | MP2 Error (dB) |
| --- | --- | --- | --- |
| ye | ze |  |  |
| 17 | 0 | -25.43 | -20.58 |
| 10 | -10 | -25.23 | -26.30 |
| 10 | 0 | -23.78 | -24.61 |
| 10 | 10 | -22.12 | -22.49 |
| 20 | -10 | -26.54 | -23.54 |
| 20 | 0 | -25.95 | -21.33 |
| 20 | 10 | -25.63 | -21.25 |
| 30 | -10 | -25.25 | -26.96 |
| 30 | 0 | -26.49 | -24.07 |
| 30 | 10 | -25.74 | -22.01 |
| Mean |  | -25.22 | -23.31 |

Variability of RFR results between MP1 and MP2 is presented in Figure 10. A positive value indicates higher sensitivity with MP1. For Device 2, all individual measurements demonstrate consistent and substantial (at some points > 10dB) variability between 100Hz and 2kHz.

Figure 10: Device 2 RFR mounting variability

### 3.2.2	HATS 2 Measurements

Device 2 RFR was also measured on HATS 2 with the previously defined MECRP. Device playback was calibrated to a nominal RLR of 2.0dB. Figure 11 shows the HATS 2 results along with HATS 1 MECRP and average RFRs using MP1. In-band sensitivity differences between the two HATS exceed 10dB at 250Hz and reach 8dB at 2.5kHz.

Figure 11: Device 2 RFR measured on HATS 2

# 4	Discussion

In the previous clauses, acoustic measurement results for two commercial HaNTE devices were presented. For each device the MECRP was defined as ye=17mm, ze=0mm. Each device was measured on two HATS at the MECRP. On HATS 1, the devices were measured at an additional 9 points in an equally spaced grid proposed in [3]. Furthermore, HATS 1 measurements were made with a standard mounting position and with an alternative mounting position. RLR and RFR results for all measurements were assessed according to the requirements in [2] and variability between HATS, measurement points, and mounting positions was reported.

Device 1 exemplifies a relatively high performing HaNTE device. RLRs stay within an acceptable range over a larger area of the device display when mounted in a standard position (MP1). Although no RFR measurements on HATS 1 pass the mask defined in [2], all positions are within 8dB of passing no matter the mounting position. Furthermore, Device 1 passes the RFR sensitivity mask when measured on HATS 2. Finally, mounting specific variability is limited to < 6dB in the frequency range 100Hz – 2kHz.

Device 2, however, demonstrates poor performance for a HaNTE device. The RLR is more susceptible to measurement point variability and fewer points fulfil the requirements of [2]. Furthermore, Device 2 RFR is far from passing the frequency mask no matter the measurement point (>20dB absolute error) or HATS. Lastly, the Device 2 RFR is prone to high levels of mounting specific variability at frequencies below 2kHz.

With these results in mind, it may be necessary for future acoustic assessments of HaNTE devices to include acoustic qualities which are currently not covered by [1] and [2]. Two qualities are proposed as potential additions:

- RLR consistency over a grid of measurement points around the MECRP

- RFR consistency with mounting variability

# 5	Proposal

The source proposes to add clauses 1-4 to the FS_HaNTE technical report.

# References

[1] 3GPP TS 26.132 "Speech and video telephony terminal acoustic test specification," Release-16.

[2] 3GPP TS 26.131 "Terminal acoustics characteristics for telephony; Requirements," Release-16.

[3] 3GPP SA4-190648 "Initial Measurements for a HaNTE device," HEAD Acoustics GmbH, Cork, Ireland, 01 July 2019.