3GPP TSG-SA WG#4 Meeting #105	S4-191013

# Ljubjana, Slovenia, 12-16 August 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consumption Reporting API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Enensys |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 6th August 2019 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document proposes the API for consumption reporting procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The API for consumption reporting procedure is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

## 4.x Procedures of the M5d interface

### 4.x.y. Procedures of the consumption reporting

These procedures are used by the Media Session Handler and the Consumption Reporting functions of the 5GMS Client to perform consumption report via the M5d interface if Consumption Reporting is applied for a Downlink Streaming session. The metadata indicating that Consumption Reporting is applied for the Downlink Streaming session is described in clause X.

In order to create a resource for the consumption reporting procedure, the Media Session Handler shall send an HTTP POST message to the Media AF to create a new application instance of Consumption Reporting. After receiving the HTTP POST message, if the Media Session Handler is authorized, the Media AF shall create a "Consumption Reporting" application instance.

"Consumption Reporting" resource is addressed by a URI thatis described in clause X. The Media AF shall respond to the Media Session Handler with a 201 Created message including a Location header field containing the URI of the created resource and a message body. The Media Session Handler shall use the URI in the Location header received in the 201 Created message in the subsequent exchanges with the Media AF. The Media AF may include the consumption reporting parameters in the message body.

If the consumption reporting parameters are not present in the message body of the 201 Created message, the Media Session Handler shall send a HTTP GET message including the URI of the "Consumption Reporting" resource to the Media AF to acquire the consumption reporting parameters. The Media AF shall respond with a 200 OK including the consumption reporting parameters in the message body. If the consumption reporting parameters are present in the message body of the 201 Created message, the Media Session Handler should not send a HTTP GET message to the Media AF to acquire the consumption reporting parameters.

When the periodic consumption report event is triggered, the Media Session Handler shall send a HTTP POST message to the Media AF. If several consumption report server URLs are provided in the consumption reporting parameters, the Media Session Handler shall choose one to send the message. The Media AF shall respond a 200 OK message to signal successful processing of the Consumption Reporting request. If the media AF has updated the consumption report parameters, the media AF shall notify the UE by including the the consumption reporting parameters in the body of the 200 OK response.

The Consumption Reporting API defining the resources, data models and related procedures for the creation and management of consumption reporting are described in clause X.

***** End of changes *****