3GPP TSG-SA4 Meeting #106	S4-191115

# Busan, South Korea, October 21-25, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 3 for Media Session Handler; Downlink Network Assistance API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | September 23, 2019 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This document proposes the API for the downlink network assistance procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The downlink network assistance procedure is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change  *****

## 4.x	Procedures of the 5Md interface

### 4.x.y	Procedures of the downlink network assistance

These procedures are used by the Media Session Handler and the Network Assistance functions of the 5GMS Client to request network assistance via the M5d interface. Downlink Network Assistance consists of a downlink network assistance session, in which the Media Session Handler may request delivery boosts, report possible bitrates, and receive bitrate recommendations.

In order to create a resource for the downlink network assistance procedure, the Media Session Handler shall send an HTTP POST message to the Media AF to create a new instance of "Downlink Network Assistance Session".After receiving the HTTP post message, if the Media Session Handler is authorized, the Media AF shall create a "Downlink Network Assistance Session" instance. The "Downlink Network Assistance Session" is addresses by a URI that is described in clause 5.x.3.1. The Media AF shall respond to the Media Session Handler with a 201 Created message including a Location header field containing the URI of the created resource and a message body containing a session identifier. The Media Session Handler shall use the session identifier in subsequent exchanges with the Media AF.

In order to receive a delivery boost, the Media Session Handler shall send an HTTP POST message to the Media AF to create a new instance of "Delivery Boost". After receiving the HTTP POST message, if the delivery boost can be provided, the Media AF creates a "Delivery Boost" instance. The "Delivery Boost" instance is addressed by a URI that is described in clause 5.x.3.1. The Media AF shall repond to the Media Session Handler with a 201 Created message including a Location header field containing the URI of the created resource and a message body containing delivery boost parameters. The Media AF may act as AF towards the PCF and NEF. After receiving the HTTP POST message, if the delivery boost cannot be provided, the Media AF shall respond to the Media Session Handler with a 403 Forbidden message including a message body containing a timeout after which a new delivery boost may be requested.

The Media Session Handler can update the "Delivery Boost" instance by sending an HTTP PATCH message to the Media AF. After receiving the HTTP PATCH message, if the delivery boost change can be provided, the Media AF updates the "Delivery Boost" instance. The Media AF responds to the Media Session Handler with a 200 OK message including a message body containing delivery boost parameters. After receiving the HTTP PATCH message, if the delivery boost cannot be provided, the Media AF shall respond to the Media Session Handler with a 403 Forbidden message including a message body containing a timeout after which a delivery boost may be changed.

In order to create a resource for reporting possible streaming bitrates, the Media Session Handler shall send an HTTP POST message to the Media AF to create a new instance of "Possible Bitrates". After receiving the HTTP POST message, the Media AF shall create a "Possible Bitrates" instance. The "Possible Bitrates" resource is addressed by a URI definded in clause 5.x.3.1. The Media AF shall respond to the Media Session Handler with a 201 Created message including a Location header field containing the URI of the created resource.

The Media Session Handler may modify the set of possible bitates. In order to modify the "Possible Bitrates" instance, the Media Session Handler shall send an HTTP PATCH message to the Media AF. After receiving the HTTP PATCH message, the Media AF shall update the "Possible Bitrates" instance. The Media AF shall respond to the Media Session Handler with a 200 OK message.

In order to receive a bitrate recommendation, the Media Session Handler shall send an HTTP GET message to the Media AF to obtain the "Bitrate Recommendation" resource. The "Bitrate Recommendation" resource is addresses by a URI defined in clause 5.x.3.1. After receiving the HTTP GET message, the Media AF shall responds with a 200 OK message including a message body containing bitrate recommendation parameters.

In order to terminate the Network Assistance Session, the Media Session Handler shall send an HTTP DELETE message to the Media AF to delete the Network Assistance Session instance.

***** Second change *****

## 5.x	DownlinkNetworkAssistance API

### 5.x.1	Overview

The DownlinkNetworkAssistance API is a RESTful API that allows the Media Session Handler to request network assistance for downlink streams. The API defines a set of procedures for the creating network assistance sessions, requesting delivery boosts, and requesting bitrate recommendations.

### 5.x.2	Data model

#### 5.x.2.1	General

This subclause specifies the application data model supported by the API.

#### 5.x.2.2	Structured data types

##### 5.x.2.2.1	Type: DownlinkNetworkAssistanceSession

This type represents a session for downlink network assistance.

Table 5.x.2.2.1-1: Definition of the DownlinkNetworkAssistanceSession data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| sessionId | string | 0..1 | Identifies the downlink network assistance session. (NOTE 1) |  |
| applicationFlowDescription | array(ApplicationFlowDescription) | 0..N | Identifies the traffic flow (NOTE 2) |  |
| NOTE 1: This property shall not be included in the POST request. This attribute shall be provided in the response of a successful downlink network assistance session resource creation. NOTE 2: This property may be excluded in the POST request when the application flow description is unknown when creating the downlink network assistance session resource creation. |  |  |  |  |

##### 5.x.2.2.2	Type: ApplicationFlowDescription

This type represents an application flow description and is used for identifying traffic for QoS treatment.

Table 5.x.2.2.2-1: Definition of the ApplicationFlowDescription data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| sourceIpv4Addr | Ipv4Addr | 0..1 | Identifies the IPv4 address of the media destination (NOTE 1) |  |
| sourceIpv6Addr | Ipv6Addr | 0..1 | Identifies the IPv6 address of the media destination (NOTE 1) |  |
| destIpv4Addr | Ipv4Addr | 0..1 | Identifies the IPv4 address of the media source (NOTE 2) |  |
| destIpv6Addr | Ipv6Addr | 0..1 | Identifies the IPv6 address of the media source (NOTE 2) |  |
| sourcePort | integer | 1 | Identifies the port number of the media destination |  |
| destinationPort | integer | 1 | Identifies the port number of the media source |  |
| protocolType | integer | 1 | Identifies the protocol type. (NOTE 3) |  |
| NOTE 1: One of the properties "sourceIpv4Addr" or "sourceIpv6Addr" shall be included. NOTE 2: One of the properties "destIpv4Addr" or "destIpv6Addr" shall be included. NOTE 3: Either the value of the last protocol type field in IPv4 case, or the value of the last next header field in case of IPv6. |  |  |  |  |

##### 5.x.2.2.3	Type: DeliveryBoost

This type represents a delivery boost instance.

Table 5.x.2.2.3-1: Definition of the DeliveryBoost data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| boostId | string | 0..1 | Identifies the downlink network assistance session. (NOTE 1) |  |
| size | integer | 1 | Identifies the size of the delivery boost, specified in number of bytes. |  |
| NOTE 1: This property shall not be included in the POST request. This attribute shall be provided in the response of a successful devivery boost resource creation. |  |  |  |  |

##### 5.x.2.2.4	Type: DeliveryBoostDenied

This type represents an information message for denied delivery boosts.

Table 5.x.2.2.4-1: Definition of the DeliveryBoostDenied data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| boostId | string | 1 | Identifies the downlink network assistance session. |  |
| retryTime | DateTime | 1 | Identifies the absolute time at which a new boost request will be allowed. |  |

##### 5.x.2.2.5	Type: PossibleBitrates

This type represents a set of possible bitrates. This set contains bitrates required for streaming different representations of a media content. In case of multiple media components (e.g. video and audio), a possible bitrate is the total required bitrate for streaming all media components.

Table 5.x.2.2.5-1: Definition of the PossibleBitrates data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| bitratesId | String | 0..1 | Identifies possible bitrates instance. |  |
| bitrate | integer | 0..N | Identifies a possible streaming bitrate, specified as bits per second. |  |
| NOTE 1: This property shall not be included in the POST request. This attribute will be provided in the response of a successful creation of a set of possible bitrates. |  |  |  |  |

##### 5.x.2.2.6	Type: BitrateRecommendation

This type represents a bitrate recommendation instance. The recommended bitrate is the total bitrate assigned for streaming all media components (e.g. video and audio).

Table 5.x.2.2.6-1: Definition of the BitrateRecommendation data type

| Attribute name | Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- |
| bitratesId | String | 1 | Identifies a possible bitrates instance. |  |
| recommendedBitrate | integer | 1 | Identifies the recommended bitrate. |  |
| guarantee | DeliveryGuarantee | 1 | Identifies if the recommended bitrate is guaranteed by QoS |  |

#### 5.x.2.3	Enumerations

##### 5.x.2.3.1	Enumeration: DeliveryGuarantee

The enumeration DeliveryGuarantee represents the type of guarantee that the network can offer when delivering media content.

Table 5.x.2.3.1-1: Definition of the DeliveryGuarantee enumeration

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| NO_GUARANTEE | The network does not provide any guarantee the recommended bitrate for delivery of media content. | BitrateRecommendation |
| GUARANTEE | The network guarantees the recommended bitrate for delivery of media content, but the network is not suitable for low latency streaming and a normal playout buffer shall be used. | BitrateRecommendation |
| GUARANTEE_LOW_LATENCY | The network guarantees the recommended bitrate and the network is suitable for low latency streaming of media content and a small playout buffer may be used. | BitrateRecommendation |

### 5.x.3	Resources

#### 5.x.3.1	Overview

All resource URIs of this API should have the following root:

{apiRoot}/3gpp-downlinknetworkassistance/v1

All resource URIs in the subclauses below are defined relative to the above root URI.

Figure 5.x.3.1-1: Resources URI structure of the DownlinkNetworkAssistance API

The following resources and HTTP methods are supported for this API:

Table 5.x.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Meaning |
| --- | --- | --- | --- |
| Downlink Network Assistance Session | /session | GET | Forbidden |
|  |  | POST | Create a new Downlink Network Assistance Session resource |
| Individual Network Assistance Session | /session/{sessionId} | GET | Read an exisiting downlink network assistance session resource |
|  |  | PATCH | Replace an existing Downlink Network Assistance Session resource |
|  |  | DELETE | Delete an existing Downlink Network Assistance Session resource |
| Delivery Boost | /session/{sessionId}/deliveryboost | GET | Forbidden |
|  |  | POST | Create a new Delivery Boost resource |
| Individual Delivery Boost | /session/{sessionId}/deliveryboost/{deliveryboostId} | GET | Read the Delivery Boost resource |
|  |  | PATCH | Modify an existing Delivery Boost resource |
|  |  | DELETE | Delete an existing Delivery Boost resource |
| Possible Bitrates | /session/{sessionId}/bitrates | GET | Forbidden |
|  |  | POST | Create a new Possible Bitrates resource |
| Individual Possible Bitrates | /session/{sessionId}/bitrates/{bitratesId} | GET | Read the Possible Bitrates resource |
|  |  | PUT | Replace an existing Possible Bitrates resource |
|  |  | DELETE | Delete an existing Possible Bitrates resource |
| Bitrate Recommendation | /session/{sessionId}/bitrates/{bitratesId}/recommendation | GET | Read the Bitrate Recommendation resource |

#### 5.x.3.2	Resource: Downlink Network Assistance Session

##### 5.x.3.2.1	Introduction

This resource allows a Media Session Handler to create a new downlink network assistance session resource.

##### 5.x.3.2.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session

This resource shall support the resource URI variables defined in table 5.x.3.2.2-1.

Table 5.x.3.2.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |

##### 5.x.3.2.3	Resource methods

###### 5.x.3.2.3.1	GET

This HTTP method is not supported for this resource.

###### 5.x.3.2.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.2.3.3	PATCH

This HTTP method is not supported for this resource.

###### 5.x.3.2.3.4	POST

This POST method shall create a new session resource for downlink network assistance. The Media Session Handler shall initiate the HTTP POST request and the Media AF shall respond to the message. The Media AF shall construct the URI of the created resource.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.2.3.4-1 and table 5.x.3.2.3.4-2.

Table 5.x.3.2.3.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.2.3.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DownlinkNetworkAssistanceSession | 1 | The "sessionId" property shall not be included in this request. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DownlinkNetworkAssistanceSession | 1 | 201 Created | The downlink network assistance session resource was created successfully. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.2.3.5	DELETE

This HTTP method is not supported for this resource.

#### 5.x.3.3	Resource: Individual Network Assistance Session

##### 5.x.3.3.1	Introduction

This resource allows a Media Session Handler to read, modify and delete an existing downlink network assistance instance.

##### 5.x.3.3.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}

This resource shall support the resource URI variables defined in table 5.x.3.3.2-1.

Table 5.x.3.3.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |

##### 5.x.3.3.3	Resource methods

###### 5.x.3.3.3.1	GET

The GET methods allows to read active downlink network assistance sessions. The Media Session Handler shall initiate the HTTP GET message and the Media AF shall respond to this message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.3.3.1-1 and table 5.x.3.3.3.1-2.

Table 5.x.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.3.3.1-2: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DownlinkNetworkAssistanceSession | 1 | 200 OK | The downlink network assistance session information for the session in request URI are returned. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.3.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.3.3.3	PATCH

The PATCH method modifies an existing downlink network assistance session resource. The PATCH method allows a Media Session Handler to change the application flow descriptions. The Media Session Handler shall initiate the HTTP PATCH request and the Media AF shall respond to the message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.3.3.3-1 and table 5.x.3.3.3.3-2.

Table 5.x.3.3.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.3.3.3-2: Data structures supported by the PATCH request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DownlinkNetworkAssistanceSession | 1 | Only the "applicationFlowDescription" property may be changed. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DownlinkNetworkAssistanceSession | 1 | 200 OK | The downlink network assistance session was modified successfully. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.3.3.4	POST

This HTTP method is not supported for this resource.

###### 5.x.3.3.3.5	DELETE

The DELETE method deletes an existing downlink network assistance session instance. The Media Session Handler shall initiate the DELETE request and the Media AF shall respond to the message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.3.3.5-1 and table 5.x.3.3.3.5-2.

Table 5.x.3.3.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.3.3.5-2: Data structures supported by the DELETE request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | None |  | 204 No Content | The downlink network assistance session was terminated successfully. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

#### 5.x.3.4	Resource: Delivery Boost

##### 5.x.3.4.1	Introduction

This resource allows a Media Session Handler to create a new delivery boost instance. The Media Session Handler may request a delivery boost when a playout buffer is (critically) low.

##### 5.x.3.4.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}/deliveryboost

This resource shall support the resource URI variables defined in table 5.x.3.4.2-1.

Table 5.x.3.4.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |

##### 5.x.3.4.3	Resource methods

###### 5.x.3.4.3.1	GET

This HTTP method is not supported for this resource.

###### 5.x.3.4.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.4.3.3	PATCH

This HTTP method is not supported for this resource.

###### 5.x.3.4.3.4	POST

This POST method shall create a new delivery boost instance. The Media Session Handler shall initiate the HTTP POST request and the Media AF shall respond to the message. The Media AF shall construct the URI of the created resource.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.4.3.4-1 and table 5.x.3.4.3.4-2.

Table 5.x.3.4.3.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.4.3.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DeliveryBoost | 1 | The "boostId" property shall not be included in this request. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DeliveryBoost | 1 | 201 Created | The delivery boost was successfully created. The URI of the created resource shall be returned in the "Location" HTTP header. |
|  | DeliveryBoostDenied | 1 | 403 Forbidden | The delivery boost was not created. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.4.3.5	DELETE

This HTTP method is not supported for this resource.

#### 5.x.3.5	Resource: Individual Delivery Boost

##### 5.x.3.5.1	Introduction

This resource allows a Media Session Handler to read, modify and delete an existing delivery boost instance.

##### 5.x.3.5.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}/deliveryboost/{deliveryboostId}

This resource shall support the resource URI variables defined in table 5.x.3.5.2-1.

Table 5.x.3.5.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |
| deliveryboostId | Identifier of the delivery boost instance. |

##### 5.x.3.5.3	Resource methods

###### 5.x.3.5.3.1	GET

The GET methods allows to read active delivery boosts. The Media Session Handler shall initiate the HTTP GET message and the Media AF shall respond to this message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.5.3.1-1 and table 5.x.3.5.3.1-2.

Table 5.x.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.5.3.1-2: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DeliveryBoost | 1 | 200 OK | The delivery boost information for the boost in the request URI are returned. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.5.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.5.3.3	PATCH

The PATCH method modifies an active delivery boost resource. The PATCH method allows a Media Session Handler to change the boost size. The Media Session Handler shall initiate the HTTP PATCH request and the Media AF shall respond to the message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.5.3.3-1 and table 5.x.3.5.3.3-2.

Table 5.x.3.5.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.5.3.3-2: Data structures supported by the PATCH request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | DeliveryBoost | 1 | Only the "size" property can be changed. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | DeliveryBoost | 1 | 200 OK | The delivery boost was modified successfully. |
|  | DeliveryBoostDenied | 1 | 403 Forbidden | The delivery boost was not modified. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.5.3.4	POST

This HTTP method is not supported for this resource.

###### 5.x.3.5.3.5	DELETE

The DELETE method deletes an existing delivery boost. The HTTP DELETE request shall be initiated by the Media Session Handler and the Media AF shall respond.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.5.3.5-1 and table 5.x.3.5.3.5-2.

Table 5.x.3.5.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.5.3.5-2: Data structures supported by the DELETE request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | None |  | 204 No Content | The delivery boost was successfully terminated. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

#### 5.x.3.6	Resource: Possible Bitrates

##### 5.x.3.6.1	Introduction

This resource allows a Media Session Handler to create a new possible bitrates resource.

##### 5.x.3.6.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}/bitrates

This resource shall support the resource URI variables defined in table 5.x.3.6.2-1.

Table 5.x.3.6.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |

##### 5.x.3.6.3	Resource methods

###### 5.x.3.6.3.1	GET

This HTTP method is not supported for this resource.

###### 5.x.3.6.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.6.3.3	PATCH

This HTTP method is not supported for this resource.

###### 5.x.3.6.3.4	POST

This POST method shall create a set of possible bitates. The Media Session Handler reports the bitrates that are required for streaming different representations of the media content. The Media Session Handler shall initiate the HTTP POST request and the Media AF shall respond to the message. The Media AF shall construct the URI of the created resource.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.6.3.4-1 and table 5.x.3.6.3.4-2.

Table 5.x.3.6.3.4-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.6.3.4-2: Data structures supported by the POST request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | PossibleBitrates | 1 | The "bitratesId" property shall not be set. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | PossibleBitrates | 1 | 201 Created | The set of possible bitrates was successfully created. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.6.3.5	DELETE

This HTTP method is not supported for this resource.

#### 5.x.3.7	Resource: Individual Possible Bitrates

##### 5.x.3.7.1	Introduction

This resource allows a Media Session Handler to read, modify and delete an existing set of possible bitrates.

##### 5.x.3.7.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}/bitrates/{bitratesId}

This resource shall support the resource URI variables defined in table 5.x.3.7.2-1.

Table 5.x.3.7.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |
| bitratesId | Identifier of the set of possible bitrates. |

##### 5.x.3.7.3	Resource methods

###### 5.x.3.7.3.1	GET

This resource allows a Media Session Handler to read an existing set of possible bitrates. The Media Session Handler shall initiate the HTTP GET request and the Media AF shall respond to this message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.7.3.1-1 and table 5.x.3.7.3.1-2.

Table 5.x.3.7.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.7.3.1-2: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | PossibleBitrates | 1 | 200 OK | The information about the set of possible bitrates for the instance in the request URI are returned. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.7.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.7.3.3	PATCH

The PATCH method shall modifly an existing set of possible bitrates. The PATCH method allows a Media Session Handler to change the bitrates required for streaming the different representations of the media content. The Media Session Handler shall initiate the HTTP PATCH request and the Media AF shall respond to this message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.7.3.3-1 and table 5.x.3.7.3.3-2.

Table 5.x.3.7.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.7.3.3-2: Data structures supported by the PATCH request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | PossibleBitrates | 1 | Only the "bitrates" property can be changed. |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | PossibleBitrates | 1 | 200 OK | The set of possible bitrates was modified successfully. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.7.3.4	POST

This HTTP method is not supported for this resource.

###### 5.x.3.7.3.5	DELETE

The DELETE method allows a Media Session Handler to delete an existing set of bitrates. The HTTP DELETE request shall be initiated by the Media Session Handler and the Media AF shall respond.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.7.3.5-1 and table 5.x.3.7.3.5-2.

Table 5.x.3.7.3.5-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.7.3.5-2: Data structures supported by the DELETE request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | None |  | 204 No Content | The set of possible bitrates was successfully terminated. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

#### 5.x.3.8	Resource: Bitrate Recommendation

##### 5.x.3.8.1	Introduction

This resource allows a Media Session Handler to obtain a bitrate recommendation from the Media AF. The bitrate recommendation contains the bitrate that is assigned for streaming the media content. The 5GMS Player shall use the recommended bitrate when selecting a representation of the media content in its adaptation logic.

##### 5.x.3.8.2	Resource definition

Resource URI: {apiRoot}/3gpp-downlinknetworkassistance/v1/session/{sessionId}/bitrates/{bitratesId}/
recommendation

This resource shall support the resource URI variables defined in table 5.x.3.8.2-1.

Table 5.x.3.8.2-1: Resources and methods overview

| Name | Definition |
| --- | --- |
| apiRoot | See clause 5.x.3.1. |
| sessionId | Identifier of the downlink network assistance session. |
| bitratesId | Identifier of the set of possible bitrates. |

##### 5.x.3.8.3	Resource methods

###### 5.x.3.8.3.1	GET

This resource allows a Media Session Handler to obtain a bitrate recommendation from the Media AF. The Media Session Handler shall initiate the HTTP GET request and the Media AF shall respond to this message.

This method shall support the URI query parameters, request and response data structures, and response codes, as specified in table 5.x.3.8.3.1-1 and table 5.x.3.8.3.1-2.

Table 5.x.3.8.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Cardinality | Remarks |
| --- | --- | --- | --- |
| none specified |  |  |  |

Table 5.x.3.8.3.1-2: Data structures supported by the GET request/response by the resource

| Request body | Data type | Cardinality | Remarks |  |
| --- | --- | --- | --- | --- |
|  | None |  |  |  |
| Response body | Data type | Cardinality | Response codes | Remarks |
|  | BitrateRecommendation | 1 | 200 OK | The bitrate recommendation information for the set of possible bitrates in the request URI are returned. |
| NOTE: The standard HTTP error response codes also apply. |  |  |  |  |

###### 5.x.3.8.3.2	PUT

This HTTP method is not supported for this resource.

###### 5.x.3.8.3.3	PATCH

This HTTP method is not supported for this resource.

###### 5.x.3.8.3.4	POST

This HTTP method is not supported for this resource.

###### 5.x.3.8.3.5	DELETE

This HTTP method is not supported for this resource.