3GPP TSG-WG Meeting # 106	S4-191235

# Busan, KR,  21 – 25 October 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR |  | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Stage 2 for Dynamic Policy based on Network Slicing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 10/16/2019 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 26.238: "Uplink streaming".

[6]	3GPP TS 26.307: "Presentation layer for 3GPP services".

[7]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]	3GPP TS 26.234: "Transparent end-to-end Packet-switched Streaming Service (PSS); Protocols and codecs".

[9]   		3GPP TS 24.501, Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3

[10]  		3GPP TS 24.526, User Equipment (UE) policies for 5G System (5GS); Stage 3

[11]		3GPP TS 28.530, Management and orchestration; Concepts, use cases and requirements

[12]		3GPP TS 28.531, Management and orchestration; Provisioning

[13] 		3GPP TS 28.541, Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

Second Change

## 5.8	Dynamic Policy based on Network Slicing for Downlink Streaming

### 5.8.1	Flow Chart

The service provider requests the assignment of one or more network slices for the distribution of the service. The service provider indicates the required QoS parameters that can be derived from the service information, such as a DASH MPD. These QoS parameters include, among other things, the GFBR, MFBR and the latency for a GBR flow. Upon successful assignment of the network slices for the service, the Media AF will respond with the list of allowed S-NSSAIs to the service provider. The service provider will signal its offered operation points and their mappings to the S-NSSAI in the service description, e.g. in the DASH MPD as discussed earlier.

The following is the flowchart diagram for this procedure:

The steps are as follows:

- The initial provision step consists of the following sub-steps, with steps b, c, d, and g as exemplary and out of scope of this specification:

  - The external 5GMSA Application provider requests the creation of a new Ingest session. The 5GMSA Application provider indicates the anticipated operation points for the service. An operation point consists of the bandwidth and latency requirements, as well as any other parameters that may influence the policy for the sessions of this application (e.g. the charging profile, coverage area, route selection information,…).

  - The NSMF is checked for suitable network slice instances for this provisioning session. If suitable network slice instance(s) are available, the Media AF selects the one(s) to use. Otherwise, the media AF uses the interfaces defined in [7] to request the creation of new network slice instance(s)  and provisions them for the new distribution configuration..

  - The newly created network slice instances are provisioned.

  - The NS Management function confirms the creation of the new network slice instance(s) to the Media AF and provides the list of S-NSSAI(s) with their corresponding parameters.

  - The Ingest and Distribution Configuration is updated with the information about the network slices.

  - The Media AF confirms successful setup of the Ingest and Distribution Configuration to the Application provider as described in section 5.4.

  - The PCF updates the URSP rules on the target set of UE(s), e.g. based on geographical service area of the offered application as described in 23.502 section 4.2.4.3, 23.503 section 6.6.2.2, and 24.501 Annex D.2.1.

- The UE starts a streaming session by fetching the entry point to the service

- The UE may retrieve information from the Media AF to assist with the route selection for the target operation point of the session. The Media Session Handler may get information about the target operation point from the media player.

- The 5GMSA Player and Session Handler in the UE perform the route selection procedure. The UE, such as the traffic descriptors, the domain descriptors, or the Application descriptors. The UE will use the matching filter to retrieve the matching Route Selection descriptor, which provides the DNN, and the S-NSSAI(s) as defined in 23.503 [4] section 6.6.2 and 23.501 [2] section 5.15.5.3 .

- The UE reuses an existing PDU session with the identified S-NSSAI (as defined in [3] section 4.3.2 and 4.3.3), or requests the establishment or modification of a PDU session with the identified parameters, if one doesn’t exist already. Upon successful PDU session establishment, the AMF notifies the UE of the assigned QoS profile ([2] section 5.7.1.2) for the PDU session

- The streaming of the media content at the target operation point starts.

### 5.8.2	Support for Multiple Operation Points

The service provider may define a set of operation points for the service that it offers as part of the service. Each operation point is mapped into a QoS profile that matches the required resources for receiving the service at that operation point. The selection of the correct PDU session is performed based on information between the Media AF, the Session Handler, and the 5GMSA Player.