| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.928 | CR | pseudo | rev | - | Current version: | 1.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinements to Architectures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GXR |  |  |  |  |  | Date: |  |  | 14/01/2020 |
| Category: | B |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This document assumes the document in S4-200041 and S4-200042 is agreed. |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3]	3GPP TS 26.118: "3GPP Virtual reality profiles for streaming applications".

[4]	ARCore, https://developers.google.com/ar/

[5]	ARKit, https://developer.apple.com/arkit/

[6]	3GPP TR 22.842: "Study on Network Controlled Interactive Service in 5GS".

[7]	3GPP TR 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[8]		3GPP TS 23.501: "System Architecture for the 5G System ".

[9]	Schuemie, Martijn J., Peter Van Der Straaten, Merel Krijn, and Charles A.P.G. Van Der Mast. “Research on Presence in Virtual Reality: A Survey.” CyberPsychology & Behavior, Vol. 4, No. 2. April 2001.

[10]	Ching, Teo Choong. “The Concept of Presence in Virtual Reality.” Medium. 27 August 2016.

[11]	Sparks, Matt. “Don't Break the Spell: Creating Presence in Virtual Reality” Learning Solutions Magazine, 17 July 2017.

[12]		3GPP TS 26.501: "5G Media Streaming Architecture".

[13]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[14]	3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction ".

[15]	3GPP TR 22.891 "Feasibility Study on New Services and Markets Technology"

[X]	"Cloud Gaming: Architecture and Performance", Ryan Shea and Jiangchuan Liu, Simon Fraser University; Edith C.-H. Ngai, Uppsala University; Yong Cui, Tsinghua University; IEEE Network-July/August 2013.

[Y] 	M. Claypool and K. Claypool. Latency and player actions in online games. Communications of the ACM, 49(11):40–45, 2006.

[Z]	 Quax, P., Monsieurs, P., Lamotte, W., De Vleeschauwer, D., and Degrande, N. Objective and subjective evaluation of the influence of small amounts of delay and jitter on a recent first person shooter game. In Proceedings of 3rd ACM SIGCOMM workshop on Network and system support for games (New York, NY, USA, 2004), NetGames ’04, ACM, pp. 152–156.

[W] 	Chen, K.-t., Huang, P., Wang, G.-s., Huang, C.-y., and Lei, C.-l. On the Sensitivity of Online Game Playing Time to Network QoS. Proceedings of IEEE INFOCOM 2006 00, c (2006).

[V]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications"

[T]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in 5G Core network (5GC)"

[S]	S. Schwarz et al., "Emerging MPEG Standards for Point Cloud Compression," in IEEE Journal on Emerging and Selected Topics in Circuits and Systems, vol. 9, no. 1, pp. 133-148, March 2019.

# 6	Mapping Extended Reality to 5G

## 6.1	Introduction

Based on the technologies introduced in clause 4 as well as the core use cases and scenarios introduced in clause 5, this clause maps a set of core technologies to 5G media centric architectures.

## 6.2	XR Processing and Media Centric Architectures

### 6.2.1	Introduction

This clause focuses on rendering and media centric architectures. The architectures are simplified and illustrative, they only consider an XR server and an XR device to identify the functions in XR servers and XR devices that communicate and exchange information, possibly over a 5GS communication. The baseline technologies are introduced in clause 4. These architectures focus on processes where the following main tasks are carried out:

-	Display

-	Tracking and Pose Generation

-	Viewport Rendering

-	Capture of real-world content

-	Media encoding

-	Media decoding

-	Media content delivery

-	5GS communication

-	Media Formats, metadata and other data delivered on communication links

-	Spatial Location Processing

The section also discusses benefits and challenges of the different approaches in terms of required bitrates, latencies, reliability, etc. A main aspect to be addressed in the following are the processes that are involved in the motion-to-photon/sound latency and how the processed may impact the XR viewport rendering.

### 6.2.2	Viewport-Independent delivery

### 6.2.2.1	Architecture

In the viewport independent delivery case, following the architecture in TS 26.118, clause 4.3, tracking and sensor information is only processed in the XR device as shown in Figure 6.2.2-1.. This means that the entire XR scene is delivered and decoded.

Figure 5.2.2-1: Viewport Independent Delivery

#### 6.2.2.2	Use Cases in Context

#### Use cases that may be addressed partially or completely by this delivery architecture are summarized in clause 5.4.

#### 6.2.2.3	Basic Procedures

The basic procedures follow the procedures of 5G Media Streaming in TS 26.501 [X], clause 5. Both, on-demand and live streaming may be considered.

#### 6.2.2.4	Content Formats and Rendering

No content formats for 6DoF streaming are yet fully defined, but the content may for example be a scene for which foreground 3D objects, for example represented by point-clouds, are mixed with background content, for example a omnidirectional scene. The combination of the content may be provided by a scene description that places the object in the 6DoF scene. Typically, the data needs to be streamed into buffers that are jointly rendered.

Due to the significant amount of data that needs to be processed in the device, hardware supported decoding and rendering is necessary.

Such an approach typically requires the delivery and decoding of several video and audio streams in parallel.

#### 6.2.2.5	Relevant QoS and QoE parameters

In the case of viewport-independent streaming, processing of updated pose information is only done locally in the XR device. Delivery latency requirements are independent of the motion-to-photon latency.

In order to provide sufficient content quality, the video material is referably encoded such that the QoE parameters as defined as defined in clause 4.2 can be fulfilled.

The necessary QoS and bitrates on the 5G System depend depend on the type of the XR media as well as on the streaming protocol. Based on information from the workshop "Immersive Media meets 5G" in April 2019 as well as from publicly announced demos, that based on today's equipment and the one available over the next 2-3 years, around 100 Mbps are sufficient bitrates to address high-quality 6DOF VR services. This is expected to allow 2k per eye streaming at 90 fps (see clause 4.2) using existing video codecs (see clause 4.5). The QoE requirements may increase further, for example higher resolution and frame rate, but with the advance of new compression tools, this is expected to be compensated.

The XR media delivery are typically built based on download or adaptive streaming such as DASH (see for example TS 26.118 [3] and TS 26.247 [7]), such that one can adjust quality to the available bitrate to a large extent. Suitable 5QI values are 6, 8, or 9 as defined in  clause 4.3.3.

In the context of the present document, relevant 3D media formats, efficient compression, adaptive delivery as well as the perceived quality of the XR media is of key relevance.

#### 6.2.2.6	Potential Standardization Needs

In the context of the above scenario, the following potential standardization needs are identified:

- Very high-bitrate streaming protocols

- 6DoF Scene Description and XR media integration

- Usage and extensions of existing power-efficient XR device platforms for XR media decoding and rendering.

- Video and audio codec extensions to efficiently code and render graphics centric formats (2D, meshes, point clouds)

- Support of decoding platforms that support the challenges documented in clause 4.5.2.

### 6.2.3	Viewport-dependent Streaming

#### 6.2.3.1	Architecture

In the viewport dependent delivery case, following the architecture in TS 26.118, clause 4.3, the tracking information is predominantly processed in the XR device, but the current pose information is provided to the XR delivery engine in order to include the pose information in the adaptive media requests. In an extension to this in the case of XR and 6DoF, the XR pose and additional information may be shared with the XR content delivery in order to only access the information that is relevant for the current viewports. According to Figure 5.2.2-2, the tracking and sensor data is processed in the XR device for XR rendering, and the media is adaptively delivered/requested based on the XR viewport. A reduced or a viewport optimized scene is delivered and also only a reduced scene is processed. Examples include an object that is not visible is not delivered, or only delivered in low quality, or that only the part of the object that is in the viewport is delivered with the highest quality.

Figure 5.2.2-2 Viewport-dependent Streaming

#### 6.2.3.2	Use Cases in Context

Use cases that may be addressed partially or completely by this delivery architecture are summarized in clause 5.4.

#### 6.2.3.3	Basic Procedures

The basic procedures follow the procedures of 5G Media Streaming in TS 26.501 [X], clause 5. Both, on-demand and live streaming may be considered.

In addition, the request for data is accompanied with information from the XR engine.

#### 6.2.3.4	Content Formats and Rendering

The same formats as discussed in clause 6.2.2.4 apply.

#### 6.2.3.5	Relevant QoS and QoE parameters

Compared to the viewport independent delivery in clause 5.2.2, for viewport dependent streaming, updated tracking and sensor information impacts the network interactivity. Typically, due to updated pose information, HTTP/TCP level information and responses are exchanged every 100-200 ms in viewport-dependent streaming.

From analysis in TR 26.918 [2] and other experience as for example documented the workshop "Immersive Media meets 5G" in April 2019", such approaches can reduce the required bitrate compared to viewport independent streaming by a factor of 2 to 4 at the same rendered quality. The more complex and the more volume the scene includes, the more the streaming rate compared to the full streaming can be reduced.

It is important to note that viewport-dependent streaming technologies are typically also built based on adaptive streaming allowing to adjust quality to the available bitrate. The knowledge of tracking information in the XR Delivery receiver just adds another adaptation parameter. However, generally such systems may be flexible designed taking into account a combination/tradeoff of bitrates, latencies, complexity and quality.

In the context of the present document, relevant formats, efficient compression, organization of XR media for adaptive delivery and decoding as well as the perceived quality of the XR media is of key relevance.

#### 6.2.3.6	Potential Standardization Needs

In the context of the this architecture, the following potential standardization needs are identified:

- The same aspects as defined in clause 6.2.2.6

- In addition, more flexible delivery algorithms that allow decoding and streaming of smaller of data, such as tile-based structures, etc.

### 6.2.4	Viewport Rendering in Network

#### 6.2.4.1	Overview

In a architecture as shown in figure 5.2.3-1 below, the viewport is entirely rendered in the XR server. The XR server generates the XR Media on the fly based on incoming Tracking and Sensor information, for example a game engine. The generated XR media is provided for the viewport in a 2D format (flattened), encoded and delivered over the 5G network. The tracking and sensor information is delivered in the reverse direction. In the XR device, the media decoders decode the media and the viewport is directly rendered without using the viewport information.

Figure 5.2.3-1 Viewport rendering in Network

The following call flow highlights the key steps:

1)	An XR device connects to the network and XR Media Generation application

a)	Sends static XR device information (supported decoders, viewport)

2)	Based on this information, XR server sets up encoder and formats

a)	Loop

b)	XR device collects pose (or a predicted pose)

c)	XR Pose is sent to XR Server

d)	The XR Server uses the pose to generate/compose the viewport

e)	XR Viewport is encoded with regular media encoders

f)	The compressed video is sent to XR Device

g)	The XR device decompresses video and directly renders viewport

Such an architecture enables simple clients, but has significantly challenges on compression and transport to fulfill the latency requirements. Latencies should be kept low for each processing step including delivery, to make sure that the cumulative delay for all the processing steps (including tracking, pose delivery, viewport rendering, media encoding, media delivery, media decoding and display) is within the immersive motion-to-photon latency upper limit of 20ms.

#### 6.2.4.2	Relevant QoS and QoE parameters

The following three cases, with different media bitrates, are considered:

1)		Around 100 Mbps: In this case, the glass needs to perform certain amount of processing and decoding.

2)		Around 1 Gbps: In this case, only lightweight and low-latency compression (e.g. intra only) may be used to provide sufficiently high quality (4k or even 8k at sufficiently high frame rates above 60 fps) and sufficiently low latency (immersive limits of less than 20ms for motion to photon) for such applications. It is still expected that some processing by the glass is needed.

3)		Around 10 Gbps or even more: A full "USB-C like" wireless connection, providing functionalities that currently can only be provided by cable, possibly uncompressed formats such as 8K. The processing requirements for the glass in this case may be minimal.

Note that the lightweight compression or no compression in cases b and c can help to reduce processing delays.

In addition, the formats exported from Game engines needs to be supported by the respective media encoders.

#### 6.2.4.3	Potential Standardization Needs

From the analysis, for case 1, the same aspects as defined in clause 6.2.2.6 apply.

For cases 2 and 3, formats are of less relevance for 3GPP as such formats are typically defined by other consortia, if at all. All of the cases only apply for sidelink communication and should first be addressed in the 5G architecture based on SA1 defined requirements.

### 6.2.5	Split Rendering: Viewport rendering with Time Warp in device

#### 6.2.5.1	Architecture and Procedures

In Figure 5.2.4-1, the viewport is pre-dominantly rendered in the XR server, but the device is able to do time-warping to address local correction.

-	VR graphics workload split into rendering workload on powerful XR server and TW on device

-	Low motion-to-photon latency preserved via on device Asynchronous Time Warping (ATW)

Figure 5.2.4-1: Split Rendering with Asynchronous Time Warping (ATW) Correction

#### 6.2.5.2	Use Cases in Context

The use cases in clause 5.5 may be addressed by this architecture.

#### 6.2.5.3	Basic Procedures

The following call flow highlights the key steps:

1)	An XR Device connects to the network and joins XR application

a)	Sends static device information (supported decoders, viewport)

2)	Based on this information, network server sets up encoder and formats

a)	Loop

b)	XR Device collects pose (or a predicted pose)

c)	Pose is sent to XR Server

d)	The XR Server uses the pose to pre-render the XR viewport

e)	XR Viewport is encoded with 2D media encoders

f)	The compressed media is sent to XR device

g)	The XR device decompresses video

h)	An improved prediction is used for asynchronouos time warping.

#### 6.2.5.4	Content Formats and Rendering

Frame buffers need to be encoded, distributed and decoded. According to clause 4.2.1, relevant formats for frame buffers are 2k by 2k per eye, potentially even higher. Frame rates are expected to be at least 60fps, potentially higher up to 90 fps.The formats of frame buffers are regular texture video signals that are then directly rendered. As the processing is graphics centric, formats beyond commonly used 4:2:0 signals and YUV signals may be considered.

#### 6.2.5.5	Relevant QoS and QoE parameters

With the use of time warp, the latency requirements follow those documented in clause 4.5.2, i.e. the end-to-end latency between the user motion and the rendering is 50ms.

It is known from experiments that with H.264/AVC the bitrates are in the order of 50 Mbps per eye buffer. It is expect that this can be reduced to lower bitrates with improved compression tools (see clause 4.5) but higher quality requirements may absorb the gains. It is also known that this is both content and user movements dependent, but it is expect from experiments that 50 - 100 Mbps is a valid target bitrate.

Regular stereo audio signals are considered, requiring bitrates that are negligible compared to the video signals.

5QI values exist that may address the use case, such 5QI value number 80 with 10ms. However, 10ms with such high bitrates and low required error rates may be too stringent and resource consuming.

The uplink is predominantly the pose information.

#### 6.2.5.6	Potential Standardization needs

In the context of the this architecture, the following potential standardization needs are identified:

- Regular 2D video encoders and decoders that are capable encode and decode 2K per eye as well as 90 fps

- Pose information in the uplink at sufficient frequency

- Content Delivery protocols

- Edge computing discovery and capability discovery

- A simple XR split rendering application framework

### 6.2.6	Generalized XR Split Rendering

In Figure 5.2.5-1, an architecture is shown for which the edge server converts the 3D scene into a simpler format to be processed by the device (e.g. it may provide additional metadata that is delivered with the pre-rendered version). The device recovers the baked media and does the final rendering based on local correction on the actual pose.

-	VR graphics workload split into rendering workload on powerful XR server and simpler rendering on the device.

-	This approach enables to relax the latency requirements to maintain a full immersive experience as time-critical adjustment to the correct pose is done in the device.

Figure 5.2.5-1: VR Split Rendering with XR Viewport Rendering in Device

Such an approach needs careful considerations on the formats of projected media and their compression with media decoders. Also important is distribution of latencies to different components of the system. More details and breakdown of the architectures is necessary. The interfaces in the device however are aligned with the general structure defined above.

In general, the similar requirements and considerations as in clause 6.2.5 apply, but a more flexible framework may be considered by providing not only 2D frame buffers, but different buffers that are split over the network. More analysis is necessary.

### 6.2.7	XR Distributed Computing

This clause provides the architecture for extended reality applications which supports the XR split rendering. The workload for XR processing is split into workloads on XR edge server and the device. The below Figure 5.2.6-1 shows a high-level structure of the XR distributed computing architecture which describes their components and interfaces.

Figure 5.2.6-1: XR Distributed Computing Architecture

An XR client may have following capabilities.

-	XR capture

-	Sensor data processing (e.g., AR pose tracking)

-	XR scene generation

-	XR rendering.

-	2D or 3D Media decoding

-	Metadata (including scene description) processing

-	5G delivery

An XR edge server may have following capabilities.

-	Sensor data processing

-	XR scene generation

-	2D or 3D media encoding

-	Metadata (including scene description) generation

-	5G delivery

An XR client connects to the network and joins XR rendering application. The XR client sends static device information (e.g., sensors, supported decoders, display configuration) to the XR edge server. Based on this information, the XR edge server sets up encoder and formats.

When the XR client has a set of sensors (e.g., trackers and capturing devices, it collects sensor data from sensors. The collected sensor data is processed either locally or at the XR edge server. The collected sensor data or locally processed information (e.g., a current AR pose) is sent to the XR edge server. The XR edge server uses the information to generate the XR scene. The XR edge server converts the XR scene into a simpler format as 2D or 3D media with metadata (including scene description). The media component is compressed, and the compressed media stream and metadata are delivered to the XR client. The XR client generates the XR scene by compositing locally generated or received media and metadata and renders the XR viewport via the XR display (e.g., HMD, AR glass).

For example, the XR client captures the 2D video stream from a camera and sends the captured stream to the XR edge server. The XR edge server performs the AR tracking and generates the AR scene which a 3D object is overlaid over a certain position in the 2D video based on the AR tracking information. The 3D object or 2D video for the AR scene are encoded with 2D/3D media encoders, and the scene description or the metadata is generated. The compressed media and metadata are sent to the XR client. The XR client decodes the media or metadata and generates an AR scene which overlays the 3D object in the 2D video., A user viewport is determined by horizontal/vertical field of view of the screen of a head-mounted display or any other display device. The appropriate part of AR scene for the user viewport is rendered and displayed.

### 6.2.7	XR Conversational

tbd

## 6.3	Summary of Traffic Characteristics

This clause summarizes the typical traffics characteristics for different architectures, based on the findings in this clause.

Legend:

-	DL: Downlink,

-	UL: Uplink,

-	PDB: Packet delay budget,

-	PER: Packet Error Rate,

-	RTT: Round Trip Time

Note: Either RTT applies, or UL and DL PDB applies separately, but RTT and UL/DL PDB cannot apply simultaneously

| Scenario | DL Rate range | UL Rate range | DL PDB | UL PDB | RTT | DL PER range | UL PER range | Traffic periodicity range | Traffic file size distribution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Viewport independent streaming | 100 MBPs | HTTP requests every second. TCP handshake | See adaptive streaming | See adaptive streaming | See adaptive streaming and TCP equation | 10e-6 | 10e-6 | Almost constant | Almost constant |
| Viewport dependent streaming | 25 MBPs | More frequent HTTP requests every 100ms. TCP handshake | See adaptive streaming | See adaptive streaming | See adaptive streaming and TCP equation | 10e-6 | 10e-6 | Almost constant | Almost constant |
| Viewport Rendering in Network case 2 | 1 GBit/s |  |  |  |  |  |  |  |  |
| Viewport Rendering in Network case 3 | 10 Gbit/s |  |  |  |  |  |  |  |  |
| Split Rendering with Time Warp | 100 Mbit/s | 500 kbit/s | 20ms | 10ms | 50ms | 10-6 | 10-3 | Almost constant | Around 50-100 kByte, i.e. 1 packer for each frame |
| Generalized Split Rendering |  |  |  |  |  |  |  |  |  |
| XR Distributed Computing |  |  |  |  |  |  |  |  |  |
| XR Conversational |  |  |  |  |  |  |  |  |  |

## 6.4	Analysis of existing 5QIs

As a summary of the above, existing 5QIs may be used. However, it is considered that for XR applications using split rendering, a more flexible framework is necessary to address different latencies between 10 and 30ms.

Error rates are ffs.