3GPP TSG-SA4 Meeting #107	S4-200123

# Wroclaw, Poland, January 20-24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | <CR#> | rev | 0 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Unicast Downlink Streaming with 5GMSd AS interactions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA |  |  |  |  |  | Date: |  |  | 2020-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Video based services have been instrumental for the massive growth of broadband traffic, with DASH being the dominant technology for video delivery. Video streaming has high demands on the network, which has also been recognized by 3GPP SA1, who state in TS 22.261 that "the 5G system shall enable efficient delivery of content from a content caching application under the control of the operator" and "the 5G system shall be able to provide the required Qos [..] for a service and support the prioritization of resources when necesarry for that service". While the 5GMS architecture addresses the latter in the form of Dynamic Policies and the discussion on Network Assistance is ongoing, we believe that efficient delivery of content requires more attention. In an architecture that enables multiple collaboration models, the 5GMSd AS may be owned by the MNO or the content provider. Furthermore, a Media Player may interact with multiple instances of the 5GMSd AS, enabling a distributed CDN. Some instances may operate as cache, allowing, for instance, the use case of an operator cache with content provider CDN as fall-back. Additionally, it enables use cases where different parts of the content (e.g., advertisement periods, popular representations) are located at different locations. The Media Player is made aware of the different locations via multiple BaseURL directives in the MPD. Considering such scenarios, the Media Player may connect to multiple 5GSMd ASs and content delivery efficiency improves with the coordination of where the Media Player can get the content. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This dCR addresses the interactions with multiple 5GMSd AS instances and procedures for interactions between the Media Player and 5GMSd AS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion about the cardinality of the 5GMSd AS and efficient content delivery using distributed CDN not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | N.A. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** First change ***

## 5.X	Procedures for Unicast Downlink Streaming with 5GMSd AS Interactions

### 5.X.1	General

This clause describes the procedure for a unicast downlink streaming session with 5GMSd AS interactions for exchanging session specific parameters. The session parameter API allows for coordination of content distribution. For example, the Media Player may send the 5GMSd AS a list of upcoming requests or an indication of the required bandwidth.

The Media Player may interact with multiple instances of the 5GMSd AS, allowing a distributed CDN. Instances of the 5GMSd AS reside in the trusted DN, the external DN, or in both. Different instances of the 5GMSd AS are addressed by different FQDNs, which are indicated in the manifest. 5GMSd AS instances may exist of multiple servers, but are addressed using a single FQDN and a load balancer redirects the Media Player to the destination server.

5GMSd AS instances may partially and/or temporarily hold the media content, thus fulfilling the role of a cache. 5GMSd AS interactions may determine the cached media content items, for example the 5GMSd AS may pre-fetch an item or keep certain items in the cache.

### 5.X.2	DASH Streaming

This procedure describes the establishment of a unicast media downlink streaming session with 5GMSd AS interactions. A streaming session may use 3GP File Format (3GP-DASH) or other (potentially non-3GPP defined) formats).

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Figure 5.X-1: Procedure for DASH content

Prerequisite:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming system and has setup content ingest.

-	The 5GMSd Aware Application has received the service announcement from the 5GMSd Application Provider.

Steps:

1: The 5GMSd Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The Service and Content Discovery procedure only involves the App and the external Application Server. The service announcement includes either the whole service access information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the service access information.

2: A media content item is selected.

3: The 5GMSd Aware Application triggers the 5GMSd client to start media playback. The Media Player Entry is provided to the 5GMSd Client.

4: When the 5GMS Aware Application has received a reference to the service access information (See Step 1), the Media Session Handler interacts with the 5GMSd AF to acquire the whole service access information.

5: In parallel, the Media Player is invoked to start media access and playback.

6: The Media Player establishes the transport session for acquiring the MPD referenced by the Media Player Entry.

7: The Media Player requests the MPD

8: The media access client receives the MPD

9: The Media Player processes the MPD. It determines for example the number of needed transport sessions for media acquisition. The 5GMS Player should be able to use the MPD information to initialize the media pipelines for each media stream. The MPD should also contain information to initialize the DRM client, when DRM is used.

10: The Media Player notifies the media session handler about the MPD. The notification may contain parameters from the MPD

11: Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

12: The Media Player configures the media playback pipeline.

13: The Media Player establishes the necessary transport sessions for the content, connecting to an instance of the 5GMSd AS. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

14: The Media Player notifies the media session handler is notified that it is ready to playback and optionally provides transport session parameters.

15: The Media Player and the 5GMSd AS exchange session specific parameters.

16: The Media Player requests initialization information. The Media Player repeats this step for each required initialization segment.

17: The Media Player receives the initialization information.

18: The Media Player requests media segments according to the MPD, optionally providing further session specific parameters.

19: the Media Player receives media segments and puts the segments into the according media rendering pipeline.

20: After time passes, and in parallel, the Media Player establishes necessary transport sessions for the content, connecting to another instance of the 5GMSd AS. For example, the media access client may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations.

21: The media player and the 5GMSd AS exchange session specific parameters.

22: The Media Player requests media segments according to the MPD, optionally providing further session specific parameters.

23: The Media Player receives media segments and puts the segments into the according media rendering pipeline.

24: Previous steps are repeated.

*** End of first change ***