3GPP TSG-SA4 Meeting #107	S4-200220

# Wroclaw, Poland,  Jan 20 - 24, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.118 | CR | 0003 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New VR metrics clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, InterDigital Communications, Inc., Fraunhofer HHI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | VRQoE |  |  |  |  |  | Date: |  |  | 2020-01-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Relevant VR metrics are needed to help service providers and operators to measure and optimize aspects related to the delivered VR service quality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause for VR metrics, containing: - Reference architecture - Comparable quality viewport switching latency metric - Rendered viewport metric - Device metric - Configuration and reporting methods - Informative annex with examples |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to measure characteristics related to the VR service performance. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 9 (new), Annex D (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ================ First change =================

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in  TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in  TR 21.905 [1].

3DOF	3 Degrees of freedom

ACN	Ambisonics Channel Number

API	Application Programming Interface

AVC	Advanced Video Coding

BMFF	Base Media File Format

BRIR	Binaural Room Impulse Response

CMP	Cube-Map Projection

CIBR	Common Informative Binaural Renderer

DASH	Dynamic Adaptive Streaming over HTTP

DRC	Dynamic Range Control

EOTF	Electro-Optical Transfer Function

ERP	EquiRectangular Projection

ESD	Equivalent Spatial Domain

FFT	Fast Fourier Transform

FIR	Finite Impulse Response

FOA	First Order Ambisonics

FOV	Field Of View

GPU	Graphics Processing Unit

HDR	High Dynamic Range

HDTV	High Definition TeleVision

HEVC	High Efficiency Video Coding

HMD	Head Mounted Display

HOA	High Order Ambisonics

HRD	Hypothetical Reference Decoder

HRIR	Head-Related Impulse Responses

HRTF	Head-Related Transfer Function

HTTP	HyperText Transfer Protocol

IFFT	Inverse FFT

IRFFT	Inverse RFFT

MAE	MPEG-H Audio Metadata information

MCC	Metrics Collection and Computation

MHAS	MPEG-H Audio Stream

MIME	Multipurpose Internet Mail Extensions

MPD	Media Presentation Description

MPEG	Moving Pictures Experts Group

NAL	Network Abstraction Layer

OMAF	Omnidirectional MediA Format

PCM	Pulse Code Modulation

RAP	Random Access Point

RFFT	Real FFT

RWP	Region-Wise Packing

SDR	Standard Dynamic Range

SEI	Supplemental Enhancement Information

SN3D	Schmidt semi-normalisation

SOFA	Spatially Oriented Format for Acoustics

SPS	Sequence Parameter Set

SRQR	Spherical Region-wise Quality Ranking

VCL	Video Coding Layer

VST	Virtual Studio Technology

VUI	Video Usability Information

VR	Virtual Reality

## ================ Next change =================

# 9	VR Metrics

## 9.1	General

VR metrics is a functionality where the client collects specific quality-related metrics during a session. These collected metrics can then be reported back to a network side node for further analysis. The metric functionality is based on the QoE metrics concept in 3GP-DASH [8], but further extended to also cover VR-specific metrics. A VR client supporting VR metrics shall support all metrics listed in clause 9.3, and shall handle metric configuration and reporting as specified in clauses 9.4 and 9.5.

## 9.2	VR Client Reference Architecture

### 9.2.1	Architecture

The client reference architecture for VR metrics, shown below in Figure 9.2.1-1, is based on the client architecture in Figure 4.3-1. It also contains a number of observation points where specific metric-related information can be made available to the Metrics Collection and Computation (MCC) function. The MCC can use and combine information from the different observation points to calculate more complex metrics.

Note that these observation points are only defined conceptually, and might not always directly interface to the MCC. For instance, an implementation might relay information from the actual observation points to the MCC via the VR application. It is also possible that the MCC is not separately implemented, but simply included as an integral part of the VR application.

Also note that in this version of this specification not all of the described observation points are necessarily used to produce VR metrics.

Figure 9.2.1-1: Client reference architecture for VR metrics

### 9.2.1	Observation Point 1

The access engine fetches the MPD, constructs and issues segment requests for relevant adaptation sets or preselections as ordered by the VR application, and receives segments or parts of segments. It may also adapt between different representations due to changes in available bitrate. The access engine provides a conforming 3GPP VR track to the file decoder.

The interface from the access engine towards MCC is referred to as observation point 1 (OP1) and is defined to monitor:

-	A sequence of transmitted network requests, each defined by its transmission time, contents, and the TCP connection on which it is sent

-	For each network response, the reception time and contents of the response header and the reception time of each byte of the response body

-	The projection/orientation metadata carried in network manifest file if applicable

-	The reception time and intended playout time for each received segment

### 9.2.2	Observation Point 2

The file decoder processes the 3GPP VR Track and typically includes a file parser and a media decoder. The file parser processes the file or segments, extracts elementary streams, and parses the metadata, if present. The processing may be supported by dynamic information provided by the VR application, for example which tracks to choose based on static and dynamic configurations. The media decoder decodes media streams of the selected tracks into the decoded signals. The file decoder outputs the decoded signals and metadata which is used for rendering.

The interface from the file decoder towards MCC is referred to as observation point 2 (OP2) and is defined to monitor:

-	Media resolution

-	Media codec

-	Media frame rate

-	Media projection, such as region wise packing, region wise quality ranking, content coverage

-	Mono vs. stereo 360 video

-	Media decoding time

### 9.2.3	Observation Point 3

The sensor extracts the current pose according to the user's head and/or eye movement and provides it to the renderer for viewport generation. The current pose may also be used by the VR application to control the access engine on which adaptation sets or preselections to fetch.

The interface from the sensor towards MCC is referred to as observation point 3 (OP3) and is defined to monitor:

-	Head pose

-	Gaze direction

-	Pose timestamp

-	Depth

### 9.2.4	Observation Point 4

The VR Renderer uses the decoded signals and rendering metadata, together with the pose and the knowledge of the horizontal/vertical field of view, to determine a viewport and render the appropriate part of the video and audio signals.

The interface from the media presentation towards MCC is referred to as observation point 4 (OP4) and is defined to monitor:

-	The media type

-	The media sample presentation timestamp

-	Wall clock counter

-	Actual presentation viewport

-	Actual presentation time

-	Actual playout frame rate

-	Audio-to-video synchronization

-	Video-to-motion latency

-	Audio-to-motion latency

### 9.2.5	Observation Point 5

The VR application manages the complete device, and controls the access engine, the file decoder and the rendering based on media control information, the dynamic user pose, and the display and device capabilities.

The interface from the VR application towards MCC is referred to as observation point 5 (OP5) and is defined to monitor:

-	Display resolution

-	Max display refresh rate

-	Field of view, horizontal and vertical

-	Eye to screen distance

-	Lens separation distance

-	OS support, e.g. OS type, OS version

## 9.3	Metrics Definitions

### 9.3.1	General

As the VR metrics functionality is based on the DASH QoE metrics [8], all metrics already defined in [8] are valid also for a VR client. Thus the following sub-clauses only define additional VR-related metrics.

### 9.3.2 Comparable quality viewport switching latency

The comparable quality viewport switching latency metric reports the latency and the quality-related factors when viewport movement causes quality degradations, such as when low-quality background content is briefly shown before the normal higher-quality is restored. Note that this metric is only relevant if the Advanced Video Media profile and region-wise packing is used. Also note that the metric currently does not report factors related to foveated rendering.

The viewport quality is represented by two factors; the quality ranking (QR) value, and the pixel resolution of one or more regions within the viewport. The resolution is defined by the orig_width and orig_height values in ISO/IEC 23090-2 [13] in SRQR (Spherical-Region Quality Ranking) or 2DQR (2-Dimensional Quality Ranking). The resolution corresponds to the monoscopic projected picture from which the packed region covering the viewport is extracted.

In order to determine whether two viewports have a comparable quality, if more than one quality ranking region is visible inside the viewport, the aggregated viewport quality factors are calculated as the area-weighted average for QR and the area-weighted (effective) pixel resolution, respectively.

For instance, if 60% of the viewport is from a region with QR=1, Res=3840 x 2160, and 40% is from a region with QR=2, Res=960 x 540, then the average QR is 0.6 x 1 + 0.4 x 2, and the effective pixel resolution is 0.6 x 3840 x 2160 + 0.4 x 960 x 540 (also see Annex D.1 for more examples).

If the viewport is moved so that the current viewport includes at least one new quality ranking region (i.e. a quality ranking region not included in the previous viewport), a switch event is started. The list of quality factors related to the last evaluated viewport quality before the switch are assigned to the firstViewport log entry. The start time of the switch is also set to the time of the last evaluated viewport before the switch.

The end time for the switch is defined as when both the weighted average QR and the effective resolution for the viewport reach values comparable to the ones before the switch. A value is comparable if it is not more than QRT% (QR threshold) or ERT% (effective resolution threshold) worse than the corresponding values before the switch. If comparable values are not achieved within N milliseconds, a timeout occurs (for instance if an adaptation to a lower bitrate occurs, and the viewport never reaches comparable quality).

Note that smaller QR values and larger resolution values are better. For instance, QRT=5% would require a weighted average QR value equal or smaller than 105% of the weighted average QR before the switch, but ERT=5% would require an effective resolution value equal or larger than 95% of the effective resolution before the switch.

The list of quality factors related to the viewport which fulfills both thresholds are assigned to the secondViewport log entry, and the latency (end time minus start time) is assigned to the latency log entry. In case of a timeout, this is indicated under the cause log entry.

During the duration of the switch the worst evaluated viewport is also stored, and assigned to the worstViewport log entry. The worst viewport is defined as the viewport with the worst relative weighted average QR or relative effective resolution, as compared to the values before the switch.

If a new viewport switching event occurs (e.g. yet another new region becomes visible) before an ongoing switch event has ended, only the N milliseconds timeout is reset. The ongoing measurement process continues to evaluate the viewport quality until a comparable viewport quality value is achieved (or a timeout occurs).

The observation points needed to calculate the metrics are:

-	OP2 File Decoder: SRQR/2DQR information

-	OP3 Sensor: Gaze information

-	OP4 VR Renderer: Start of switch event detection (alternatively, region coverage information from SRQR/2DQR can be used as strict rendering pixel-exactness is not required)

-	OP5 VR Application: Field-of-view information of the device

The accuracy of the measured latency depends on how the client implements the viewport switching monitoring. As this might differ between clients, the client shall report the estimated accuracy.

The thresholds QRT, ERT, and the timeout N, can be specified during metrics configuration (see clause 9.4) as attributes within parenthesis, e.g. "CompQualLatency(QRT=3.5,ERT=6.8,N=900)". If a threshold or the timout is not specified the client shall use appropriate default values.

The data type ViewportDataType is defined in Table 9.3.2-1 below, and identifies the direction and coverage of the viewport.

Table 9.3.2-1: ViewportDataType

| Key |  | Type | Description |
| --- | --- | --- | --- |
| ViewportDataType |  | Object |  |
|  | centre_azimuth | Integer | Specifies the azimuth of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | centre_elevation | Integer | Specifies the elevation of the centre of the viewport in units of 2−16 degrees. The value shall be in the range of −90 * 216 to 90 * 216, inclusive. |
|  | centre_tilt | Integer | Specifies the tilt angle of the viewport in units of 2−16 degrees. The value shall be in the range of −180 * 216 to 180 * 216 − 1, inclusive. |
|  | azimuth_range | Integer | Specifies the azimuth range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |
|  | elevation_range | Integer | Specifies the elevation range of the viewport through the centre point of the viewport, in units of 2−16 degrees. |

The data type Viewport-Item is defined as shown in Table 2. Viewport-Item is an Object which identifies a viewport and quality-related factors for the region(s) covered by the viewport.

Table 9.3.2-2: ViewportItem

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| ViewportItem |  |  |  | Object |  |
|  | Position |  |  | ViewportDataType | Identifies the viewport |
|  | QualityLevels |  |  | List | List of different quality levels regions within the viewport |
|  |  | Coverage |  | Float | Percentage of the viewport area covered by this region |
|  |  | QR |  | Integer | Quality ranking (QR) value of this region |
|  |  | Resolution |  | Object | Resolution for this region |
|  |  |  | Width | Integer | Horizontal resolution for this region |
|  |  |  | Height | Integer | Vertical resolution for this region |

The comparable quality viewport switching latency metric is specified in Table 9.3.2-1 below.

Table 9.3.2-1: Comparable quality viewport switching latency metric

| Key |  |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- | --- |
| CompQualLatency |  |  |  |  | List | List of comparable quality viewport switching latencies |
|  | Entry |  |  |  | Object |  |
|  |  | firstViewport |  |  | ViewportItem | Specifies information about the first viewport |
|  |  | secondViewport |  |  | ViewportItem | Specifies information about the second viewport |
|  |  | worstViewport |  |  | ViewportItem | Specifies information about the worst viewport seen during the switch duration |
|  |  | time |  |  | Real-Time | Wall-clock time when the switch started |
|  |  | Mtime |  |  | Media-Time | Media presentation time when the switch started. |
|  |  | Latency |  |  | Integer | Specifies the switching delay in milliseconds. |
|  |  | Accuracy |  |  | Integer | Specifies the estimated accuracy of the latency metric in milliseconds |
|  |  | Cause |  |  | List | Specifies a list of possible causes for the latency. |
|  |  |  | Entry |  | Object |  |
|  |  |  |  | code | Enum | A possible cause for the latency. The value is equal to one of the following: - 0: Segment duration - 1: Buffer fullness - 2: Availability of comparable quality segment - 3: Timeout |

### 9.3.3 Rendered viewports

The rendered viewports metric reports a list of viewports that have been rendered during the media presentation.

The client shall evaluate the current viewport gaze every X ms and potentially add the viewport to the rendered viewport list. To enable frequent viewport evaluations without necessarily increasing the report size too much, consecutive viewports which are close to each other may be grouped into clusters, where only the average cluster viewport data is reported. Also, clusters which have too short durations may be excluded from the report.

The viewport clustering is controlled by an angular distance threshold D. If the center (i.e. the azimuth and the elevation) of the current viewport is closer than the distance D to the current cluster center (i.e. the average cluster azimuth and elevation), the viewport is added to the cluster. Note that the distance is only compared towards the current (i.e. last) cluster, not to any earlier clusters which might have been created.

If the distance to the cluster center is instead equal to or larger than D, a new cluster is started based on the current viewport, and the average old cluster data and the start time and duration for the old cluster is added to the viewport list.

Before reporting a viewport list, a filtering based on viewport duration shall be done. Each entry in the viewport list is first assigned an "aggregated duration" equal to the duration of that entry. Then, for each entry E, the other entries in the viewport list are checked. The duration for a checked entry is added to the aggregated duration for entry E, if the checked entry is both less than T ms away from E, and closer than the angular distance D from E.

After all viewport entries have been evaluated and have received a final aggregated duration, all viewport entries with an aggregated duration of less than T are deleted from the viewport list (and thus not reported). Note that the aggregated duration is only used for filtering purposes, and not itself included in the viewport list reports.

Some examples of metric calculation are shown in Annex D.2.

The observation points needed to calculate the metrics are:

- OP3 Sensor: Gaze information

- OP5 VR Application: Field-of-view information of the device

The viewport sample interval X (in ms), the distance threshold D (in degrees), and the duration threshold T (in ms) can be specified during metrics configuration as attributes within parenthesis, e.g. "RenderedViewports(X=50,D=15,T=1500)". Note that if no clustering or duration filtering is wanted, the D and T thresholds can be set to 0 (e.g. specifying "RenderedViewports(X=1000,D=0,T=0)" will just log the viewport every 1000 ms). If no sample interval or thresholds values are specified the client shall use appropriate default values.

The rendered viewports metric is specified in Table 9.3.3-1.

Table 9.3.3-1 Rendered viewports metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| RenderedViewports |  |  | List | List of rendered viewports |
|  | Entry |  | Object |  |
|  |  | startTime | Media-Time | Specifies the media presentation time of the first played out media sample when the viewport cluster indicated in the current entry is rendered starting from this media sample. |
|  |  | duration | Integer | The time duration, in units of milliseconds, of the continuously presented media samples when the viewport cluster indicated in the current entry is rendered starting from the media sample indicated by startTime. "Continuously presented" means that the media clock continued to advance at the playout speed throughout the interval. |
|  |  | viewport | ViewportDataType | Indicates the average region of the omnidirectional media corresponding to the viewport cluster being rendered starting from the media sample time indicated by startTime. |

### 9.3.4	VR Device information

This metric contains information about the device, and is logged at the start of each session and whenever changed (for instance if the rendered field-of-view for the device is adjusted). If an individual metric cannot be logged, its value shall be set to 0 (zero) or to the empty string.

The observation point needed to report the metrics is:

- OP5 VR Application: Device information

Table 9.3.4-1: Device information

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| VrDeviceInformation |  |  | List | A list of device information objects. |
|  | Entry |  | Object | A single object containing new device information. |
|  |  | start | Real-Time | Wall-clock time when the device information was logged. |
|  |  | mstart | Media-Time | The presentation time at which the device information was logged. |
|  |  | deviceIdentifier | String | The brand, model and version of the device. |
|  |  | horizontalResolution | Integer | The horizontal display resolution, per eye, in pixels. |
|  |  | verticalResolution | Integer | The vertical display resolution, per eye, in pixels. |
|  |  | horizontalFoV | Integer | Maximum horizontal field-of-view, per eye, in degrees. |
|  |  | verticalFoV | Integer | Maximum vertical field-of-view, per eye, in degrees. |
|  |  | renderedHorizontalFoV | Integer | Current rendered horizontal field-of-view, per eye, in degrees. |
|  |  | renderedVerticalFoV | Integer | Current rendered vertical field-of-view, per eye, in degrees. |
|  |  | refreshRate | Integer | Display refresh rate, in Hz |

## 9.4	Metrics Configuration and Reporting

### 9.4.1	Configuration

Metrics configuration is done according to clauses 10.4 and 10.5 in DASH [8], but can also include any metrics defined in clause 9.3.

### 9.4.2	Reporting

Metrics reporting is done according to clause 10.6 in DASH [8], with the type QoeReportType extended to handle the additional VR-specific metrics according the the XML schema in clause 9.4.3. In this version of the specification the element vrMetricSchemaVersion shall be set to 1.

### 9.4.3	Reporting Format

<?xml version="1.0"?> <xs:schema xmlns:xs="http://www.w3.org/2001/XMLSchema" targetNamespace="urn:3gpp:metadata:2020:VR:metrics" xmlns:hsd="urn:3gpp:metadata:2011:HSD:receptionreport" xmlns="urn:3gpp:metadata:2020:VR:metrics" elementFormDefault="qualified"> <xs:complexType name="VrQoeReportType"> <xs:complexContent> <xs:extension base="QoeReportType"> <xs:sequence> <xs:element name="vrMetric" type="VrMetricType" minOccurs="0" maxOccurs="unbounded"/> <xs:element name="vrMetricSchemaVersion" type="unsignedInt"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> </xs:extension> </xs:complexContent> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="VrMetricType"> <xs:choice maxOccurs="unbounded"> <xs:element name="compQualLatency" type="CompQualLatencyType" maxOccurs="unbounded"/> <xs:element name="renderedViewports" type="RenderedViewportsType" maxOccurs="unbounded"/> <xs:element name="vrDeviceInformation" type="VrDeviceInformationType" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:choice> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="CompQualLatencyType"> <xs:sequence> <xs:element name="firstViewport" type="ViewportItem"/> <xs:element name="secondViewport" type="ViewportItem"/> <xs:element name="worstViewport" type="ViewportItem"/> <xs:element name="time" type="xs:dateTime"/> <xs:element name="mtime" type="xs:duration"/> <xs:element name="latency" type="xs:unsignedInt"/> <xs:element name="accuracy" type="xs:unsignedInt"/> <xs:element name="cause" type="unsignedInt" minoccurs="0" maxoccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:cmplexType> <xs:complexType name="RenderedViewportsType"> <xs:sequence> <xs:element name="startTime" type="xs:duration"/> <xs:element name="duration" type="xs:unsignedInt"/> <xs:element name="viewport" type="ViewportDataType"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:cmplexType> <xs:complexType name="VrDeviceInformationType"> <xs:sequence> <xs:element name="start" type="xs:dateTime"/> <xs:element name="mstart" type="xs:duration"/> <xs:element name="deviceIdentifier" type=cs:string/> <xs:element name="horizontalResolution" type=cs:unsignedInt/> <xs:element name="verticalResolution" type=cs:unsignedInt/> <xs:element name="horizontalFoV" type=cs:unsignedInt/> <xs:element name="verticalFoV" type=cs:unsignedInt/> <xs:element name="renderedHorizontalFoV" type=cs:unsignedInt/> <xs:element name="renderedVerticalFoV" type=cs:unsignedInt/> <xs:element name="refreshRate" type=cs:unsignedInt/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:cmplexType> <xs:complexType name="ViewportItem"> <xs:sequence> <xs:element name="position" type="ViewportDataType"/> <xs:element name="qualityLevel" type="QualityLevelEntry" maxOccurs="unbounded"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> <xs:complexType name="ViewportDataType"> <xs:sequence> <xs:element name="centreAzimuth" type="xs:unsignedInt"/> <xs:element name="centreElevation" type="xs:unsignedInt"/> <xs:element name="centreTilt" type="xs:unsignedInt"/> <xs:element name="azimuthRange" type="xs:unsignedInt"/> <xs:element name="elevationRange" type="xs:unsignedInt"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:cmplexType> <xs:complexType name="QualityLevelEntry"> <xs:sequence> <xs:element name="coverage" type="xs:double"/> <xs:element name="qr" type="xs:unsignedInt"/> <xs:element name="width" type="xs:unsignedInt"/> <xs:element name="height" type="xs:unsignedInt"/> <xs:any namespace="##other" processContents="lax" minOccurs="0" maxOccurs="unbounded"/> </xs:sequence> <xs:anyAttribute processContents="skip"/> </xs:complexType> </xs:schema>

Annex D (informative):
VR metrics calculation examples

# D.1 Comparable quality viewport switching latency

This sub-clause illustrates how the weighted average QR value and the effective resolution can be calculated.

The quality level of each region is determined with its respective quality ranking (QR) value. A viewport can be covered with multiple regions. A quality level value for the viewport can be derived as weighted average of the QR values of the regions covering the viewport. The weight of each region is defined as the percentage of the viewport area covered by the corresponding region. The viewport quality level can be calculated by the following equation.

QualityLevelviewport=i=1NQRi×Coveragei/100

N: Number of regions covering the viewport

QR[i]: QR value of i-th quality ranking region

Coverage[i]: The viewport coverage value (in percent) of i-th quality ranking region

Figure D.1-1: An example of a viewport covered by four quality ranking 2D regions

Figure D.1-1 is an example of a viewport covered by four quality ranking regions. The quality of the viewport is equal to the weighted sum of the quality ranking value and the coverage percentage value of each quality ranking region.

The resolution of each region is determined by its respective width and height values in pixel which are available in the quality ranking box under the name orig_width and orig_height. Note that these values are already normalized to represent the full-sphere resolution you would get if the resolution of this region would be used for the full sphere.

The effective resolution (i.e. the total number of original pixels) for the content visible in the viewport can be derived as the weighted average of the resolution of each region covering the viewport. The weight of each region is defined by the percentage of the viewport area covered by the corresponding region. The effective viewport resolution can be calculated by the following equation.

EffectiveResolutionviewport=i=1Nwidthi×heighti×Coveragei/100

N: Number of regions covering the viewport

width[i]: The width component of the original source pixel resolution for the i-th quality ranking region

height[i]: The height component of the original source pixel resolution resolution for the i-th quality ranking region

Coverage[i]: The viewport coverage value (in percent) of i-th quality ranking region

Coverage [1]

Coverage [3]

Coverage [4]

Region [1]

Region [2]

Region [4]

Region [3]

Coverage [2]

Figure D.1-2: An example of a source packed image with four quality ranking 2D regions with different resolutions

Resolution [3]

Resolution [2]

Resolution [4]

Viewport

35%

15 %

40 %

10%

Coverage [1]

Coverage [2]

Coverage [3]

Coverage [4]

Resolution [1]

Figure D.1-3: An example of a viewport covered by four different quality ranking 2D regions

Figure D.1-2 is an example of a source with four regions with different resolution. Figure D.1-3 represents an example of a viewport which is covered by the four quality ranking 2D regions. The effective viewport resolution is equal to the weighted sum of the resolution for each quality-ranking 2D region and its corresponding viewport coverage percentage value.

Figure D.1-4 – Comparable quality viewport switching latency measurement example

Figure D.1-4 presents an example of the metric measurement operation. The viewport quality is evaluated at time t0, and then again at time t1. The media playback module renders the high-resolution sub-picture #1 at time t1. The user viewing orientation is gradually changing from sub-pic#1 to sub-pic#2 as the time progresses.

At time t2, the media playback module starts to render the buffered low-quality representation of sub-pic#2 as the viewport moves into sub-picture #2. At time t2, the viewport quality drops in values as compared to the viewport quality at time t1, and a new sub-picutre (sub-pic #2) is rendered. A viewport switching event is identified at time t2.

The viewport quality values evaluated at t1 identifies the first viewport. The viewport position and viewport quality level list are assigned to the attributed Position and QualityLevel of the firstViewportItem.

An effective viewport resolution and viewport QR quality value for the new viewport that is comparable to that of the firstViewportItem after viewport switching time is logged at time t4.  The new viewport position identifies the Position of the secondViewportItem. The corresponding QualityLevel list for the secondViewportItem is assigned.

The associated viewport values stored for the worst viewport quality during the switch is assigned to the field Position of the worstViewportItem. The corresponding QualityLevel list for the worstViewportItem is also assigned.

The comparable-quality viewport switching latency is measured as the time interval between the logged times for firstViewportItem (t1 in this example) and secondViewportItem (t4 in this example).

# D.2 Rendered viewports

Figure D.2-1 illustrates an example of clustering and the associated viewports. The first three evaluated viewports are all with the distance D (indicated by the blue circle), and are thus assigned to the same cluster. Note that the cluster center moves a bit for each new viewport which is added to the cluster.

Viewport #4 is too far away from the center of cluster #1, and thus starts a new cluster, which eventually gathers three viewport members. Then viewport #7 is too far away from the center of cluster #2, and again starts a new cluster.

Figure D.2-1 Clustering example

For each cluster j, the final averaged viewport parameters can be derived as follows, assuming there are N viewports in the j:th cluster. Note that the center azimuth and tilt averaging also needs to handle the special case around -180/180 degrees, as some values might be positive (e.g. 176 degrees), while others might be negative (e.g. -178 degrees). This special case is not shown in the equations below.

Note also that the azimuth and elevation range (i.e. the visible coverage of the viewport) might often be the same for every viewport, unless the user explicitly changes the field-of-view for the device. For consistency, and to catch any during-session field-of-view changes, these two parameters should still be averaged.

Figure D.2-2 below illustrates an example of the duration filtering. The user starts by looking at the upper left part of the media (viewports #1 to #3), then make a very brief glance to the right (viewport #4), and then moves back to the upper-left again (viewports #5 and #6). Then the user moves his gaze to the lower-right part (viewports #7 to #10).

Assume here that the duration T is set to 4 times the value of the viewport sample rate X, i.e. a cluster needs to have a duration corresponding to at least four viewports to be reported. Here four clusters are formed, but before filtering only cluster #4 would be reported. After filtering, clusters #1 and #3 are close enough both in time and distance to add to each other's aggregated duration, so each of them will be be assigned an aggregated duration of 5, and thus be reported. Cluster #2, the quick gance up to the right, has too short duration and will not be reported.

Figure D.2-2 Duration filtering example