3GPP TSG-SA4 Meeting #108e	S4-200553

# e-meeting, 2 - 9 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | - | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client APIs for 5GMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | DASH Streaming not existing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds DASH Streaming |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No DASH |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR assumes that document S4-200509 is agreed. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 4.5	Procedures of the M4d (Media Streaming) interface

### 4.5.1	Procedures for DASH Session

This procedure is used by a 5GMSd Client to establish a DASH session via the M5d interface. In order to establish such a session, the 5GMSd AS shall host an MPD and the MPD URL shall be known to the Media Player, in this case a DASH player.

The Media Player receives an MPD URL from the Media Session Handler or from the 5GMSd-Aware Application. The Media Player shall send an HTTP GET message including the URI of the MPD resource to the 5GMSd AS to fetch the MPD. The 5GMSd AS shall respond with a 200 (OK) message that includes the MPD.

Additional procedures for reactions to different HTTP status codes are provided in TS 26.247 [X] and ISO/IEC 23009-1 [Y], clause A.7.

Additional procedures for handling partial file responses are provided in TS 26.247 [X], clause A.9.

### 4.5.2	Procedures for Progressive Download Session

This procedure is used by a 5GMSd client establish a Progressive Download session via the M5d interface.

Details are tbd

=====  CHANGE  =====

## 5.X	DASH Distribution

A 5GMSd AS should support the service requirements of the DASH-IF Core Interoperability Point as defined in DASH-IF IOP [Z], clause 6.1.2.

A Media Player should support the General Client Processing Requirements and Recommendations of the DASH-IF Core Interoperability Point as defined in DASH-IF IOP [Z], clause 6.1.3.

Editor's Note: Attached to this document the draft of DASH-IF IOP v5. It will be ready by mid of May. Also attached a slide set on the main tools addressed.