3GPP TSG-SA4 Meeting #110e	S4-201197

19-28 August 2020	revision of S4-201020

Agenda item	11.5

Source: 	Intel

Title: 	On Metrics for ITT4RT

Document for	Agreement

# Introduction

This contribution proposed VR QoE metrics collection and reporting framework for ITT4RT, by reusing (i) existing metrics developed for DASH-based VR Streaming in TS 26.118 and (ii) existing QoE metric configuration and reporting techniques for MTSI in TS 26.114.

# Proposed Updates to ITT4RT PD

## 9.X	QoE Metrics for ITT4RT

## 9.X.1	Introduction

This section documents a VR QoE metrics collection and reporting framework for ITT4RT, by reusing (i) existing metrics developed for DASH-based VR Streaming in TS 26.118 [X] and (ii) existing QoE metric configuration and reporting techniques for MTSI in TS 26.114 [Y]. The VR QoE metrics may be collected by service providers and used to enhance the immersive media quality and experiences as part of ITT4RT. A client reference model is also provided with observation and measurement points for collection of the metrics.

## 9.X.2	Client Reference Model

The client reference architecture for VR metrics, shown below in Figure 9.X.2.1, is based on the receiver architecture in Figure 6.1. It also contains a number of observation points where specific metric-related information can be made available to the Metrics Collection and Computation (MCC) function. The MCC can use and combine information from the different observation points to calculate more complex metrics.

Note that these observation points are only defined conceptually, and might not always directly interface to the MCC. For instance, an implementation might relay information from the actual observation points to the MCC via the VR application. It is also possible that the MCC is not separately implemented, but simply included as an integral part of the VR application.

Figure 9.X.2.1: Client reference architecture for VR metrics

### Observation Point 1

Based on one or more received RTP media streams, the UE parses, possibly decrypts and feeds the elementary stream to the HEVC decoder.

The interface from the RTP receiver towards MCC is referred to as observation point 1 (OP1) and is defined to monitor:

-	The reception time for each received RTP packet as well as packet losses and interarrival times of the RTP packets.

- 	Relevant RTCP packet information including RTCP sender and receiver reports containing packet statistics, as well as related RTCP feedback messages for rate adaptation as well as for viewport information signalling.

### Observation Point 2

The HEVC decoder receives the elementary bitstream and obtains the decoder output signal, referred to as the "texture", as well as the decoder metadata. The Decoder Metadata contains the Supplemental Information Enhancement (SEI) messages, i.e., information carried in the omnidirectional video specific SEI messages, to be used in the rendering phase.

The interface from the HEVC decoder towards MCC is referred to as observation point 2 (OP2) and is defined to monitor:

-	Media resolution

-	Media codec

-	Media frame rate

-	Media projection, such as region wise packing, region wise quality ranking, content coverage

-	Mono vs. stereo 360 video

-	Media decoding time

### Observation Point 3

The sensor extracts the current pose according to the user's head and/or eye movement and provides it to the renderer for viewport generation. The current pose may also be used by the VR application to control the RTP receiver to signal for instance viewport information to the RTP sender using RTCP feedback messages.

The interface from the sensor towards MCC is referred to as observation point 3 (OP3) and is defined to monitor:

-	Head pose

-	Gaze direction

-	Pose timestamp

-	Depth

### Observation Point 4

The output signal, i.e., the decoded picture or "texture", is then rendered using the Decoder Metadata information contained in relevant SEI messages contained in the video elementary streams. The Decoder Metadata is used when performing rendering operations such as region-wise unpacking, projection de-mapping and rotation toward creating spherical content for each eye. The VR Renderer uses the decoded signals and rendering metadata, together with the pose and the knowledge of the horizontal/vertical field of view, to determine a viewport and render the appropriate part of the video and audio signals.

The interface from the media presentation towards MCC is referred to as observation point 4 (OP4) and is defined to monitor:

-	The media type

-	The media sample presentation timestamp

-	Wall clock counter

-	Actual presentation viewport

-	Actual presentation time

-	Actual playout frame rate

-	Audio-to-video synchronization

-	Video-to-motion latency

-	Audio-to-motion latency

### Observation Point 5

The VR application manages the complete device, and controls the RTP receiver, the HEVC decoder and the rendering based on media control information, the dynamic user pose, and the display and device capabilities.

The interface from the VR application towards MCC is referred to as observation point 5 (OP5) and is defined to monitor:

-	Display resolution

-	Max display refresh rate

-	Field of view, horizontal and vertical

-	Eye to screen distance

-	Lens separation distance

-	OS support, e.g. OS type, OS version

## 9.X.3	Metrics Definitions

Based on the client reference model described in clause 9.X.2, the VR QoE metrics described in TS 26.118 may be reused as potential metrics candidates for ITT4RT metrics collection and reporting, namely:

- Comparable quality viewport switching latency, as described in clause 9.2 and Annex D.1 of TS 26.118

- Rendered viewports, as described in clause 9.3 and Annex D.2 of TS 26.118

- VR device information, as described in clause 9.4 of TS 26.118

### Comparable quality viewport switching latency

The comparable quality viewport switching latency metric reports the latency and the quality-related factors when viewport movement causes quality degradations, such as when low-quality background content is briefly shown before the normal higher-quality is restored. Note that this metric is only relevant in case of viewport dependent processing, i.e., when VPD capability is negotiated.

Editor’s Note: Detailed syntax and measurement process for this metric are TBD.

The observation points needed to calculate the metrics are:

-	OP2 HEVC Decoder: QR information

-	OP3 Sensor: Gaze information

-	OP4 VR Renderer: Start of switch event detection

-	OP5 VR Application: Field-of-view information of the device

### Rendered viewports

The rendered viewports metric reports a list of viewports that have been rendered during the media presentation.

Editor’s Note: Detailed syntax and measurement process for this metric are TBD.

The observation points needed to calculate the metrics are:

- OP3 Sensor: Gaze information

- OP5 VR Application: Field-of-view information of the device

### VR Device information

This metric contains information about the device, and is logged at the start of each session and whenever changed (for instance if the rendered field-of-view for the device is adjusted).

Editor’s Note: Detailed syntax and measurement process for this metric are TBD.

The observation point needed to report the metrics is:

- OP5 VR Application: Device information

## 9.X.4 Metrics Configuration and Reporting

### Configuration

Metrics configuration is done according to MTSI in TS 26.114, but can also include any metrics above defined for ITT4RT. Both OMA-DM and QMC approaches described in clauses 16.3 and 16.5 of TS 26.114 may be used to configure the QoE metrics for ITT4RT.

### Reporting

Metrics reporting can be done according to MTSI, with the type QoeReportType extended to handle the additional VR-specific metrics according the XML schema in clause 16.4.1 of TS 26.114.

# Proposal

It is proposed to adopt the proposed updates to the text in Section 2 in the ITT4RT permanent document.