Source:	MTSI SWG Chairman

Title:	MTSI SWG Report during SA4#110-e

## Document for:	Approval Agenda Item:	13.3

Executive summary

The 3GPP SA4 MTSI SWG met for four telco sessions during SA4#110-e, and handled the other documents via the MTSI_SWG email reflector.

A total of 56 delegates participated while 40 Tdocs were discussed with SWG-status concluded for37 Tdocs.  Below is a summary of what was agreed during this meeting.

Maintenance

- TS 26.238: agreed CR to FLUS F-C Stage 3 based on a draft CR agreed at SA4#109-e

- TS 26.114:

  - agreed CR to clarify use of RAN Delay Budget reporting for coverage enhancements only in LTE

  - agreed editorial CR and CR to update to references

- TS 26.223: agreed CR to update references

ITT4RT

- Agreed to a 2 phase approach to completing the feature in Rel-17, where Phase 1 prioritizes specifying basic real-time 360 degree video functionality.  This phasing also includes agreements on providing EVS-based solution(s) in Rel-17, and using IVAS when the specification becomes available.

- Agreed to include in the permanent document the following:

  - Updated Semantics for Signaling of Camera Calibration Parameters

  - Support of 360 fisheye video and associated SDP signalling

  - Further details on viewport-dependent delivery

  - Consistent vs. high-quality viewport with margins

  - On Metrics for ITT4RT

  - Potential Solutions for multiple overlays

- Scheduled four telcos before SA4#111-e

  - To advance the work on audio aspects, including specification of audio metadata, the MTSI SWG is proposing a joint telco with the EVS SWG on September 9th

FS_FLUS_NBMP

- Agreed to include in the Permanent Document call flows for the different deployment scenarios.

  - Agreed discuss extensions to SA6 Edge architecture in a joint telco with FS_EMSA experts

- Scheduled two telcos before SA4#111-e

  - Also proposing a 3rd telco as joint with the MBS SWG on FS_EMSA for September 17th

New Study Item Proposal

- After discussing a proposed SID on Media Negotiation Extensions for Super Resolution it was agreed to schedule a telco to continue discussions as follows

| Telco (Topic: Proposed SID on Media Negotiation Extensions for Super Resolution Date: 30 Sept 2020, Time 06:30-08:00 CEST, Host: Qualcomm) | - Continue discussion on proposed SID for Media Negotiation Extensions for Super Resolution - Contribution submission deadline: 23:59 CEST, Sept 25, 2020 |
| --- | --- |

The output documents from the MTSI SWG sessions are:

| 13 | Reports and general issues from sub-working-groups |  |
| --- | --- | --- |
| 13.3 | MTSI SWG | 1188 |
| 14 | CRs to completed features in Release 16 and earlier | TS 26.114 CR: 1006, 1189, 1190 TS 26.223 CR: 1191 TS 26.238 CR: 1192 |
| 16 | Release 17 Features |  |
| 16.2 | ITT4RT (Support of Immersive Teleconferencing and Telepresence for Remote Terminals) | PD: 1194 TP: 1193 |
| 17 | Study Items |  |
| 17.6 | FS_FLUS_NBMP (Feasibility Study on the use of NBMP in E_FLUS) | TP: 1203 |

Agreed in MTSI SWG

No status in MTSI SWG

# SWG Minutes during SA4#110-e

## 11.1 Opening of the session

Mr. Nikolai Leung (Qualcomm, Chairman of MTSI SWG) opened the e-meeting sessions on August 19th, and the Telco sessions at 06:06 CEST on August 21st.

The minutes are shared online here:

https://drive.google.com/file/d/1GRIKeS5UkfcNsASY8zoPC0S5saSymtg4/view?usp=sharing

Bo Burman, Charles Lo, and Iraj Sodagar agreed to serve as the acting secretaries for the meeting.

Draft Schedule for the Telcos:

Friday August 21st:

11.1	Opening of the session

11.2	Registration of documents

11.3	Reports and liaisons from other groups

11.4	CRs to Features in Release 16 and earlier

11.5	ITT4RT

Monday August 24th:

11.5	ITT4RT (invited EVS SWG members to discuss 1018 and 1022)

11.6	FS_FLUS_NBMP

Tuesday August 25th:

11.8	New Work / New Work Items and Study Items (1068)

11.6	FS_FLUS_NBMP (1146)

11.5	ITT4RT (1045, 1047, 1142, 1153, 1154, 1023)

Wednesday August 26th:

Wash-up of any open items and other documents

- email agreements for: 1045, 1047, 1142, 1153, 1154, 1197

- 1193, 1194, 1195, 1198, 1199

11.9	Any Other Business

11.10	Close of the session

## 11.2 Registration of documents

The following documents were registered before the meeting:

| 11.1 | Opening of the session |  |
| --- | --- | --- |
| 11.2 | Registration of documents |  |
| 11.3 | Reports and liaisons from other groups |  |
| 11.4 | CRs to Features in Release 16 and earlier | 26.114: 1006, 1041, 1057 26.223: 1043 26.238: 1053 |
| 11.5 | ITT4RT (Support of Immersive Teleconferencing and Telepresence for Remote Terminals) | PD: 1024 1018, 1019, 1020, 1022, 1045, 1047, 1142, 1153, 1154, 1151 1021 26.114: 1023 TP: 1025 |
| 11.6 | FS_FLUS_NBMP (Feasibility Study on the use of NBMP in E_FLUS) | PD: 1061 TP: 1062 1146 |
| 11.7 | Others including TEI |  |
| 11.8 | New Work / New Work Items and Study Items | FS_MNE_SR: 1068 1069 |
| 11.9 | Any Other Business |  |
| 11.10 | Close of the session |  |

The agenda and allocation of documents were approved.

## 11.3 Reports and liaisons from other groups

None.

## 11.4 CRs to Features in Release 16 and earlier

| S4-201006 | Editorial Improvements | Samsung Electronics Romania |
| --- | --- | --- |

Sent for email approval by 21Aug 0600 CEST

No comments received.

Document was agreed, to plenary agenda item 14.

| S4-201041 | Draft CR 26.114 Update of References | Samsung R&D Institute India |
| --- | --- | --- |

Sent for email approval by 21Aug 0600 CEST

No comments received.

Document was revised to S4-201190.

| S4-201190 | CR 26.114 Update of References | Samsung R&D Institute India |
| --- | --- | --- |

S4-201190 was agreed without presentation, to plenary agenda item 14.

| S4-201043 | Draft CR 26.223 Update of References | Samsung R&D Institute India |
| --- | --- | --- |

Sent for email approval by 21Aug 0600 CEST

No comments received.

Document was revised to S4-201191.

| S4-201191 | CR 26.223 Update of References | Samsung R&D Institute India |
| --- | --- | --- |

S4-201191 was agreed without presentation, to plenary agenda item 14.

| S4-201053 | Correction of FLUS F-C Stage 3 | Ericsson LM |
| --- | --- | --- |

Sent for email approval by 21Aug 0600 CEST

Hyun-Koo email comments:

1. In Table 7.1.1-1, the sink resource should be sinks.

2. In 7.3, 7.4.1 and 7.4.2.1 , there is unnecessary space between "Example" and ":".

3. In the second sentence of the second paragraph above the Table 7.4.2.1-1, I suggest to remove "shall" since the session properties could not be updated. I think the following sentences in the current specification are enough to describe the normative procedure. The same comment for 7.4.2.2.

Revised to S4-201192.

| S4-201192 | Correction of FLUS F-C Stage 3 | Ericsson LM |
| --- | --- | --- |

Sent for email agreement by 25 Aug 1600 CEST

Document was agreed via email at 25 Aug 19:30 CEST and will be sent to agenda item 14

| S4-201057 | Correction to RAN Delay Budget Reporting for Coverage Enhancements in NR | Qualcomm Incorporated |
| --- | --- | --- |

Sent for email approval by 21Aug 0600 CEST

Kyunghun Jung email comments:

Hi, meaning of the new sentence is not clear.

RAN delay budget reporting is specified in TS 36.331 [160] for E-UTRA and TS 38.331 [163] for NR, while the use of RAN delay budget reporting for coverage enhancements is only specified for E-UTRA.

as RAN delay budget reporting can have many benefits. Did you intend to mean the following:

while the use of RAN delay budget reporting is specified for coverage enhancements only in E-UTRA.

Min Wang email comments:

Indeed delay budget reporting has multiple benefits. Using it for coverage enhancements is one of them. The intention of this CR is to clarify that up to Rel-16, the use of delay budget reporting for coverage enhancements is specified for E-UTRA only. I am OK with your proposed modification to the change if you think it is more clearer.

Kyunghun Jung email comments:

I think the new version will be clearer for the developers. (^^)

Revised to Tdoc S4-201189.

| S4-201189 | Correction to RAN Delay Budget Reporting for Coverage Enhancements in NR | Qualcomm Incorporated |
| --- | --- | --- |

S4-201189 was agreed without presentation, to plenary agenda item 14.

## 11.5 ITT4RT (Support of Immersive Teleconferencing and Telepresence for Remote Terminals)

| S4-201025 | ITT4RT Timeplan v0.8.0 | Intel, Nokia Corporation (ITT4RT Rapporteurs) |
| --- | --- | --- |

Presented by Ozgur Oyman of Intel.

Discussion:

- Igor: Propose launching Doodle poll right now.

- Nik: Suggest starting Sep 9 and have telcos every other week; SEP9, SEP23, OCT7, OCT21.

- Iraj: Can we have a master calendar with all telcos? Don’t want overlap with MPEG.

- Ozgur: Is having 4 telcos reasonable? (no objection). MPEG is the week of OCT10-14, so no overlap with the above.

The document was revised to S4-201193 (v0.8.1).

| S4-201193 | ITT4RT Timeplan v0.8.1 | Intel, Nokia Corporation (ITT4RT Rapporteurs) |
| --- | --- | --- |

There was agreement that relevant aspects of the phasing as agreed in S4-201202 will be copied into the updated time plan.

This document was not treated in the MTSI SWG and will be sent directly to agenda item 16.2 in closing plenary.

| S4-201024 | ITT4RT Permanent Document v0.8.0 | Intel, Nokia Corporation (ITT4RT Rapporteurs) |
| --- | --- | --- |

Presented by Ozgur Oyman of Intel.

Discussion:

- Saba: Editorial; missing heading formatting of 9.12.

The document was agreed.

S4-201194 was assigned to make a further update to the PD based on agreed input at this meeting.

| S4-201194 | ITT4RT Permanent Document v0.8.1 | Intel, Nokia Corporation (ITT4RT Rapporteurs) |
| --- | --- | --- |

1194 will go directly to closing plenary without status

| S4-201019 | ITT4RT: Updated Semantics for Signaling of Camera Calibration Parameters | Intel |
| --- | --- | --- |

Presented by Ozgur Oyman of Intel

Discussion:

- Igor: This is a much better presentation than binary representation. So we are supporting it.

- Nikolai: Imed wanted to make clear that the entrance parameter to be optional.

- Ozgur: yes that is the case, and we support Qualcomm’s concern about this parameter to be optional.

- Igor: the description is fine and we can improve it when we do the normative language.

The document was agreed.

| S4-201018 | Spatial Audio related Metadata in ITT4RT | KPN N.V. |
| --- | --- | --- |

Discussed in the August 24th Telco.

Presented by Simon Gunkel of KPN N.V.

Discussion:

- Stefan (Dolby): terminology on speaker orientation vs. direction - these are different (perspectives of speaker vs. listener).

- Simon: direction in this document refers to direction at which speaker is speaking

- Stefan: trying to clarify concept relative to speaker located 45 degree to listener’s right, and speaker may be directing speech towards listener or some other direction

- Iraj: position of person in scene and direction he is talking - 2 parameters involved; this info may need to be dynamically updated

- Simon: for ITT4RT and trying to simplify characteristics for virtual experience; up now have not considered position of people; direction can change and speaker could turn his direction of speech - this need to be dynamically signaled

- Peter: suggest using term “talker” instead of “speaker”

- Ozgur: head orientation and whether that is currently part of viewport signaling - currently it is not; have not addressed such aspect for speech; how orientation and direction signaled in OMAF? Worth further consideration

- Tomas: 360 video with listerener centric scene - most important is direction of talker is that what you intend? Depending on format…..

- Simon: scene has some kind of audio, with multiple users with mono audio; properties of room considered in 2nd part of document; where people are placed not yet addressed.

- Tomas: need to further discuss audio formats

- Stephane: orientation should be defined here; generally rotation in spatial system. Need to describe metadata

- Simon: what’s real and minimal requirement for ITT4RT; current positioning tied to OMAF and 360 degree and how rendered to users;

- Stefan: come back to starting with simple case, is so, would not mention talker orientation; if talker rotates his head, becomes more complex. Is metadata you propose really simple info and could that be understood by renderers?

- Simon: even 360 case still have spatial properties - positions of users itself necessary but not sufficient; verifies that position of users not yet covered?

- Ozgur: correct, limited to 3DoF and such experience for remote user for video and speech components; only viewport info is currently considered relevant

- Imed: MPEG and scene description solves problem by assigning audio source as child node of visual object of speaker; relative transformations to visual objects; can also rotate direction of the sound. Seems like we need something similar here.

- Simon: for ITT4RT no guarantee of visual rendering of talker; just capture 360 room and joiners get bidirectional audio exchange.

- Iraj: question audio experts: parameters for rendition that marks sense to audio codec. Do we need object based audio? Don’t have position nor direction of talker; if transmit such info would it lead to something useful for listener; do we need more advanced codecs for this?

- Tomas: describe scene where talker is placed relative to capture points. but direction of talker’s speech is additional aspect.

- Simon: concern more on how to redern audio user joined via VR; purely considering for now one user joining remote and how to render that user

- Tomas: render remote participant close to that point; why is orientation of remote users important?

- Simon: center is position of 360 degree camera - how to capture room with different people ; for remote joiners, he has typically single microphone with simple audio link. To render that audio to other remote users or conference room, how to do that? Not much complication since remote user can only rotate and not move

- Simon: check with audio experts whether this makes sense from audio perspective

- Naotaka: should describe scenario step-by-step; in conference room and someone is talking; when move head in different direction, the scene should move. Expect VR listener moves head position, talker’s audio to change; thinks change in talker’s audio position should be signaled

- Simon: propose to add capability into requirements of permanent doc and elaborate description and work on SDP

- Naotaka: The two concepts of SDP giving initial configuration and RTCP feedback providing updates should be described more clearly.

- Stefan: in changing loudness to remote user - how to determine if talker is actually speaking to that remote user?

- Nik: any comments on 2nd part of recommendation?

- Milan: room description is for rendering to remote user of another remote user such that it sounds like a talker in the room, is that correct understanding?

- Simon: yes

- Nik: suggest Simon to provide revised document providing clarification to comments raised today

- Simon: can do this; can we get agreement to adding the two suggested requirements to the PD?

- Markus: concern that before adding requirements that we clearly understand the feature, which doesn’t seem yet achieved

- Tomas: agree need more clarification and discussion on both requirements

- Ozgur: similar reservations to adding requirements. Not yet clear direction on solution for audio - urge audio experts to contribute solution proposals with timeline in mind; need to be doable in Rel-17 timeframe

- Simon: will revise the contribution and distribute

The document was revised into S4-201195

| S4-201195 | Spatial Audio related Metadata in ITT4RT | KPN N.V. |
| --- | --- | --- |

Presented by: Simon Gunkel of KPN

Discussion:

- Simon: thinks room properties is pretty straightforward and suggest adding to Requirements section; whereas on direction of talker defer to future telco

- Naotaka-san: term “remote” causes some confusion; to him it means external user relative to conference room; on two directions of talker, please clarify basic requirements: what experience for remote listener who may move his head - does audio environment adjust accordingly? Also, on phased approach to ITT4RT, spatial audio is not covered as say for Phase 1 or 2

- Simon: remote participant indeed refers to external use joining conference. Thinks direction of talker may be too complex for this WI, but still important to specify as requirements. Might consider in future telcos to work on audio aspects

- Stephane: on room configurations, we had some discussion on Mon meeting. Would be interesting to further characterize, suggest don’t add requirements until at least some joint session with EVS group

- Tomas: similar comment as Stephane; many ways besides the cited metadata to render audio

- Simon: only proposes to add sentence as shown - something agreed to trigger future work; no solution framework is implied, just describe room properties. Sentence of concern is the bullet point in document under first first dashed item under audio properties of the room

- Stephane: other document from Ozgur on phased approach - IVAS to be eventual target solution; as this requirement would be imposed also on IVAS, should first have discussion with EVS group (e.g. room configuration to be signaled via SDP)

- Simon: please clarify scope of IVAS - room config. is system metadata level not on encoding

- Stephane: IVAS is not on codecs itself, but also aspects on how to apply IVAS to multimedia telephony, includes SDP and RTP payload aspects

- Simon: still think we’re discussing different levels of the system, but agrees coordination with IVAS people is desirable

- Stephane: prefers such interaction via joint session MTSI with EVS

- Tomas: audio formats support to then consider rendering; seems too early to describe room configuration

- Nik: suggests that we agree to have a future MTSI call and invite EVS people to attend, for now let’s note this document

- Stephane: prefers done as joint meeting

Ozgur mentions proposed ITT4RT telcos in Sept and Oct; Nik suggests advising Imre/EVS group to enable joint mtg

1195 is Noted with plan for future MTSI/ EVS joint telco

| S4-201022 | ITT4RT: Proposed Conclusions for Permanent Document | Intel |
| --- | --- | --- |

Discussed in the August 24th Telco

Presented by: Ozgur Oyman of Intel

- So far the work on 360 video delivery has led to many new functionality documented in PD (latest version in -1024). Now in Rel-17 timeframe, we need to have a more structured approach to work to complete WI. Propose 2-phased approach: Phase 1 completion upon Jan 2021, SA4#112 (immersive video functionality, SDP nego, encap formats, RTCP viewport info signaling, signaling camera calibration SDP signaling overlays and still background. Thinks spatial audio needs to be included in Phase 1).

Discussion:

- Tomas: audio feature done under Phase 1: do you foresee additional audio work in Phase 2?

- Ozgur: before introducing basic 360 video in spec, need to have basic audio solution to go with that, may not need spatial audio in Phase 1 but can be moved to Phase 2 work; Possible in Rel-18 to have IVAS audio solution. Desire audio expert input that such is possible.

- Tomas: we have at least available codecs for Phase 1 audio when IVAS is not p/o solution

- Stefan: multi-mono coding needed; not necessary to get spatial experience which requires audio rendering to be defined. Limited support for audio could specify dual-mono audio. Given no characterization of dual-mono for EVS; not clear whether this could be completed in Phase 1 on quality characterization.

- Ozgur: do you mean more time may be needed for even multi-mono EVS coding - that could require additional work of such in Phase 2?

- Stefan: yes

- Ozgur: significant work on 360 video but not in spatial audio. Envisions limited spatial audio done on Phase 2

- Nik (wearing QC hat) - needs to check with our internal codec team

- Tomas: Phases are pretty closely spaced

- Ozgur: for Phase 2, 6 additional features for 360 video; cannot accomplish all these in Phase 1; 360 video features in Phase 1 should not be dependent on Phase 2 work. For audio, no clear split now between Phase 1 and Phase 2 work due to lack of clear understanding nor inputs in PD nor discussions with audio experts. Need better understanding to establish basis for audio part. Even Jan 2021 may be too early to complete basic audio solution. Seems only crucial commitment for audio is completion of spatial audio solution by end of work item.

- Naotaka-san: originally 2-phase approach for audio proposed; based on today’s discussion are you thinking now that there needs to be no audio commitment by the end of Phase 1?

- Ozgur: doesn’t see IVAS audio done by the end of Rel-17. Ultimately we want to introduce IVAS to ITT4RT, but might not be a full IVAS solution by the end of Rel-17.

- Nik (wearing QC hat): concern of partial solution in TS. Would prefer the basic video solution not to be formally agreed CRs until audio solution is also available to be documented in spec.

- Ozgur: OK for basic video support in Rel-17 as dCRs; don’t wish for end of Rel-17 not to have audio solution; question is how long to impose the video solution dependency on audio; would at least wish to see basic video to be specified as Rel-17 solution independent of audio readiness.

- Stefane: need acoustic requirements also for spatial audio

- Ozgur:  MTSI work should not involve acoustic testing

- Tomas: not clear about specification value with solution for only video and not audio

- Ozgur: companies who think the audio component is essential for ITT4RT need them to commit to contributions. His company is more interested in 360 video.

- Tomas: What is the value of a video solution only w/o audio?

- Stefan Dohla: not sure what IVAS will offer; at least mono audio should be available - not be limited to “silent” teleconferences

The document was revised into S4-201196

| S4-201196 | ITT4RT: Proposed Conclusions for Permanent Document | Intel |
| --- | --- | --- |

Was sent for email discussion.

Comments from JaeShin:

Hi Ozgur,

I have two questions on the revised way forward document.

Are you going to divide into two docs of PD according to the phase 1 and 2? Or just to make time schedule plans for more progress efficiency?

Defining two phases looks like making two different considerations (such as specifications or documents or from lower version to higher version, etc.)

Also, I think signalling of camera calibration parameters in phase 1 may affect network-based stitching through the use of NBMP (or edges) in phase 2 because session-level SDP negotiation of camera-parameters will be used to network based stitching.

Best Regards,

Jaeshin

Reply from Ozgur:

I have two questions on the revised way forward document.

Are you going to divide into two docs of PD according to the phase 1 and 2? Or just to make time schedule plans for more progress efficiency?

[Oz] PD will stay as one document. The intention is to structure the work item timeplan based on the proposed phases for better efficiency.

Defining two phases looks like making two different considerations (such as specifications or documents or from lower version to higher version, etc.)

[Oz] This is not what is proposed. Versioning of the specifications is already established via 3GPP rules.

Also, I think signalling of camera calibration parameters in phase 1 may affect network-based stitching through the use of NBMP (or edges) in phase 2 because session-level SDP negotiation of camera-parameters will be used to network based stitching.

[Oz] As we agreed in the PD, the baseline solution for network-based stitching and associated signaling of camera calibration parameters will be according to SDP-based procedures. As one alternative, PD also mentions that camera calibration parameters may be carried in JSON format (instead of being carried in an SDP attribute), but still be referenced from the SDP. Use of NBMP in ITT4RT is not yet well established in the PD and requires more study in my opinion. As such, I do not see any dependencies to the work in Phase 2 – i.e., work of Phase 1 would only specify the SDP-based procedures for signaling of camera calibration parameters.

After receiving offline comments the document was revised into S4-201198.

| S4-201198 | ITT4RT: Proposed Conclusions for Permanent Document | Intel |
| --- | --- | --- |

Presented by Ozgur Oyman of Intel

Discussion:

- Igor: In phase 1, bullet e, please add view-independent. We have to enable view-independent, even if it doesn’t need the specific RTP.

- Ozgur:  that is in item b.

- Igore: it is not clear. I thought e is about view dependent and independent. Then add the view-independent to item b.

- Ozgur: agreed to add.

- Igor: I had a comment for video for waiting audio and vice versa, I see you added a comment.

- Ozgur: only at the end of phase 2, if audio is not ready, video will proceed.If audio is not ready in phase 1, video will wait for phase 2.  if the limited audio support can not be completed in phase 2, then video will proceed to CR.

- Igor: I don’t want audio to hold video. We can put pressure on audio.

- Ozgur: the current plan achieves putting video in R17. We have one more meeting at the end of Phase 2 to complete video without audio. We can add a specific meeting SA4#114.

- Tomas: My point was the typo here.

- Stephane: We should not be pessimistic that we won’t have minimal audio support.

- Igor: if the minimal work is easy, why not remove the entire text?

- Ozgur: if it is easy to achieve, why not put phase 1 as deadline? I suggest we get back to the original text which has phase 1 as the deadline.

- Igor: agree with Ozgur since it provides a larger margin.

- Nik: if we go with this approach, it’s possible to agree with video CR and audio may not be completed?

- Ozgur: yes, but the basic mono-EVS audio support doesn’t require much work. So we have enough time to have audio support by the end of phase 1. I agree with Igor not to squeeze everything to the end.

- Igor: the text should be SA4#112.

- Nik: if we have basic mono-EVS at phase 1, will we stop working on dual mono-EVS in phase 2?

- Igor: I don’t think it means you stop additional work. How many solutions do we need for audio?

- Ozgur: we will have multiple audio solutions in any case. If mon-evs is available and then upgrade mono-evs to multi mon-evs, when IVAS is ready, we will still have multiple solutions.

- Nik: as long as we agree on multiple audio solutions, it’s ok. If we have an additional solution in phase 2, we add that one.

- Ozgur: Is your concern about backward compatibility? We need to make the call at the end of Phase 1, so see if we want to have both mono-EVS, multi mono-EVS, etc.

- Tomas: you have to apply the same change in the corresponding text in Phase 1 section.

- Igor: the sentence in Phase 1 section contradicts the other paragraph. the last sentence should be removed.

- Ozgur: the 360-video will be DCR in Phase 1 and CR in Phase 2. So it is consistent.

- Tomas: it is confusing.

- Igor: the text in phase 2 relates to phase 1, so it should be in phase 1 section.

- Ozgur: the agreement on CR happens in phase 2, so paragraph is about agreement in phase 2.

- Nik: we change it to “start of phase 2”.

The document was revised into S4-201202

| S4-201202 | ITT4RT: Proposed Conclusions for Permanent Document | Intel |
| --- | --- | --- |

S4-201202 was then agreed without presentation.

It was also agreed that relevant aspects of the phasing will be copied into the updated time plan S4-201193.

| S4-201020 | On Metrics for ITT4RT | Intel |
| --- | --- | --- |

Presented by Ozgur Oyman of Intel

Discussion:

- Bo: there is a concept of comparable quality. Since we have losses in RTP, we might want to consider loss in the metrics.

- Ozgur: ultimately what is to be measured is for example measuring the time to change a new port. The quality characterization would include the RTP losses.

- Bo: So the comparable quality should include the effect of loss?

- Ozgur: We might consider defining the comparable quality as a range rather than absolute number, due the fact that the losses may be different and result in a different quality. So the principle is still applicable, but how we implement it in the context of RTP delivery, we need to refine it, with a clear measurable process.

- Bo: I agree using the same principle is OK but we need to define the comparable quality for RTP.

- Igor: This document has two parts: client ref model, which is ok to be adopted in PD. For the two metrics, we recently have 3 different types in render processing, and each of them will have an impact on how the metrics are used. So I don’t think we should move the metrics to PD at this stage. The metrics definition should wait.

- Nik: we park the document for now. We need more discussion on Monday.

- Ozgur: any planning for opening email discussion on this?

- Nik: contact the interested parties and do the discussion.

The document was revised in to S4-201197

| S4-201197 | On Metrics for ITT4RT | Intel |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

Comment from Imed:

Hi Ozgur,

This seems to be a verbatim copy from VR QoE for streaming. What are the real-time conferencing specific-aspects? Is everything here applicable to real-time VR?

Br, Imed

Comment from Ozgur:

Hi Imed,

I think you were not present during last week’s MTSI session when I explained the differences. Observation Point 1 in the client reference architecture is the main difference, considering RTP-based transport. As such we need to take losses into account, which may for instance impact comparable quality viewport switching latency. Also metrics configuration and reporting are based on MTSI-based mechanisms rather than DASH.

At the same time, the contribution does argue that a significant portion of the work that is already done in TS 26.118 on VR QoE for streaming can be reused to derive metrics for ITT4RT, and proposes to leverage this when developing metrics for ITT4RT.

Best regards,

Ozgur

BTW, maybe it was not clear from the initial email, so please look at the revised version of the contribution in document S4-201197, this is what is proposed for email agreement.

Discussion on Wed meeting:

- Imed: response from Ozgur is fine

1197 is Agreed

| S4-201045 | ITT4RT: Update of Permanent Document to support 360 fisheye video | Samsung Electronics Iberia SA |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

1045 is Agreed

| S4-201047 | ITT4RT: SDP signalling for 360 fisheye video | Samsung Electronics Iberia SA |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

Comment from Bo:

Just a minor question; would it be possible to have fisheye as a parameter to the existing 3gpp_360video attribute instead, considering that fisheye is a type of 360 video, or is fisheye too dissimilar to other 360 video?

Comment from Imed:

I have a question related to the SDP signaling. Why is the number of circular images proposed to be a session-level parameter.

Shouldn’t it be a media level parameter? All circular images are regions of one video stream.

Furthermore, I suggest that you consider exposing only essential static metadata, leave the rest to in-band SEI message signaling.

Further discussion during Wed telco session:

Eric describes use of SDP to further describe session level parameters for fisheye

- Imed: if offer multiple fisheye streams should that be obvious from a number of media lines?

- Eric: some streams may contain more than one fisheye image

- Imed: then can declare this at media level

- Eric: packing might be different depending on configuration - i.e. not limited to single circular image per stream

1047 is Agreed

| S4-201142 | Further details on viewport-dependent delivery | Nokia Corporation |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

Comment from Imed:

Hi Saba,

I have a few comments to the different approaches:

Approach 1: no region signaling is needed. However, when sending the full 360, the encoding/decoding capabilities of both parties may be exceeded. For example need to encode/decode 6k per eye in real-time. Also need for a smart video encoder that allows dynamic bitrate allocation per region.

Approach 2:  mixed quality does not need tiling, would be similar to the previous approach (less flexible because of the rectangular tiling). Mixed resolution requires extensive processing at the receiver side to rewrite the bitstream. Also requires running 2 encoders on the sender side in real-time one of them at High Resolution.

Approach 3: sending just the current viewport still requires region signaling on a per frame basis. Rotation affects the signaling as it might contain warping. Not sure if the region signaling can accommodate this type of adjustment for rotation.

Approach 4: This is the easiest and most practical approach for real-time communication. As the HR region follows the viewport, rwp signaling per frame is required.

In summary, I am ok with documenting these in the PD but we should not recommend or mandate any particular way of doing VPD VR in ITT4RT.

Br, Imed

Reply from Saba:

Thanks a lot for the comments. I agree, the intention is not to mandate one of these at this point. While not proposed in this contribution, having some guidelines or recommendations is beneficial in our opinion in order to ensure interoperability. Also, it will help the group streamline the functionality, signalling requirements and metrics.

1142 is Agreed to be added to PD

| S4-201153 | Potential Solutions for multiple overlays | Tencent |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

Comments from Charles:

Just a few comments on doc 1153:

- Regarding Use Case/Scenario 2, it still would be useful to explain more clearly in the document the rationale for sender to assert control on whether or how much the receiver can view overlay(s) from a source other than the sender. The first two sentences of the bullet item on page 2 seems only to repeat each other rather than provide additional clarification information.

- Sec. 5.2 – regarding the subset of the sender’s overlays that the receiver wishes to acquire: similar to the case that the sender might be able to exert a certain prerogative as in my question above, i.e., by overriding the receiver’s preference, is it intended in your proposed solution that the sender might reject the SDP offer from the receiver – e.g. the sender requires the receiver to receive all of the sender’s overlays? And then might it be left to the receiver decision to either accept all or none the sender’s SDP answer (note that I’m not an SDP offer/answer expert)?

- Sec. 5.3 – how does the user’s interaction control feature as described, such as for the 2nd bullet item, interact with the feature of pure sender control of overlay display by receiver as described in Use Case 2 above? I.e, which has precedence?

Thanks,

Charles

Comments from Rohit:

Just a few comments on doc 1153:

- Regarding Use Case/Scenario 2, it still would be useful to explain more clearly in the document the rationale for sender to assert control on whether or how much the receiver can view overlay(s) from a source other than the sender. The first two sentences of the bullet item on page 2 seems only to repeat each other rather than provide additional clarification information.

Noted

- Sec. 5.2 – regarding the subset of the sender’s overlays that the receiver wishes to acquire: similar to the case that the sender might be able to exert a certain prerogative as in my question above, i.e., by overriding the receiver’s preference, is it intended in your proposed solution that the sender might reject the SDP offer from the receiver – e.g. the sender requires the receiver to receive all of the sender’s overlays? And then might it be left to the receiver decision to either accept all or none the sender’s SDP answer (note that I’m not an SDP offer/answer expert)?

The assumption here is, the sender sends the list of all overlays to the receiver via SDP. The receiver decides (based on its resource availability) whether it wants to receive all or just a subset of overlays; and this (subset) list is conveyed back to the sender via SDP answer. The sender does not require the receiver to receive all of the overlays. In case the receiver wishes to receive only a subset of overlays, overlay_priority can be used to decide which overlays to receive.

- Sec. 5.3 – how does the user’s interaction control feature as described, such as for the 2nd bullet item, interact with the feature of pure sender control of overlay display by receiver as described in Use Case 2 above? I.e, which has precedence?

All parameters in sec. 5.3 are in fact coming from use case 2 i.e parameters in sec 5.3 are all sender controlled.

Comment from Imed:

Hi Abhishek,

I am worried that there is mixing of the media aspects of multi-party conferencing and conference management aspects (which are part of the application layer). The latter is used to control participants, assign permissions and roles of each participant and so on (e.g. what a solution like Zoom offers).

All these aspects should be out of scope.

So in brief, aspects like which participant is allowed to show/share what content should not be part of ITT4RT.

Another question on the subset list attribute. Since each overlay is a separate stream, wouldn’t this be part of the negotiation? Why would we need an additional attribute for this?

Br, Imed

Discussion during Wed telco session:

- Rohit: Iraj has responded to Imed’s comments

- Imed: has not yet reviewed them

- Imed upon review of Iraj’s response - understand there is no intent for authorization?

- Iraj: more about when sender side is presenting don’t wish to see overlay atop the presentation

- Imed: this is then more application layer function

- Iraj: could do so strictly at application management or part of media integrity consumption

- Imed: would like to others’ opinion; none expressed

- Nik: might make sense to add editor’s note on whether it’s conference management domain or part of media presentation

- Bo: thinks IMS conf mgmt should be the default - extend to support overlay handling, given e.g. floor control already part of conf mgmt for multi-party media handling (Annexes S & T of TS 26.114)

- Rohit indicated he can make changes to Sec. 2 per comments and adding a note on that we should consider whether application management should take care of the use-cases or this is part of ITT4RT scope.

1153 is revised to S4-201200

| S4-201200 | Potential Solutions for multiple overlays | Tencent |
| --- | --- | --- |

The document was sent out for email agreement by 27 Aug 15:00 CEST

| S4-201154 | Consistent vs. high-quality viewport with margins | Nokia Corporation |
| --- | --- | --- |

Sent for email approval by 26Aug 16:00 CEST

Comments from Charles:

Hi Igor and Saba,

I have a few comments on doc 1154:

- The two margin operational methods under Examples: perhaps it was described in the past, but “Consistent Quality” is not defined – how does it compare visual quality wise to HQ and the so-called “relatively lower-quality” viewport option? I assume it is lower than HQ but still the comparison between margin quality vs. “relatively lower-quality” viewport is not evident. Is the intention that in place of HQ viewport along with HQ smaller-size margin, to provide an overall lower-than-HQ viewport and margin at the so-called CQ? This needs to be more clearly explained.

- On the first sentence below the two listed methods, what does “latter” specifically refer to? Is such cited benefit achieved specifically by Method 2, or generally attainable in either method because of the presence of some amount of margin at the same quality as the viewport? If your answer is that the benefit at least applies to Method 2, then it seems objective of MTHQ delay is fundamentally unattainable since CQ in my understanding implies less than HQ by definition – as such how could then MTHQ be possible? (Notice I highlighted the HQ portion of this term).

Thanks,

Charles

Response from Saba:

- The two margin operational methods under Examples: perhaps it was described in the past, but “Consistent Quality” is not defined – how does it compare visual quality wise to HQ and the so-called “relatively lower-quality” viewport option? I assume it is lower than HQ but still the comparison between margin quality vs. “relatively lower-quality” viewport is not evident. Is the intention that in place of HQ viewport along with HQ smaller-size margin, to provide an overall lower-than-HQ viewport and margin at the so-called CQ? This needs to be more clearly explained.

SA: CQ in this case itself is not a quality level. The idea is that the receiver would prefer more of the bandwidth to be dedicated to providing a “consistent” visual quality so that the chance of having low-quality background content after head motion is minimized. The actual viewport quality in the case of CQ can be as high as the allowed bandwidth can support. The intention is in fact as you state for restricted bandwidth, hence it states “if needed, a relatively lower-quality viewport”. If there is sufficient bandwidth, a sender may end up delivering a wide margin with the highest quality it supports.

- On the first sentence below the two listed methods, what does “latter” specifically refer to? Is such cited benefit achieved specifically by Method 2, or generally attainable in either method because of the presence of some amount of margin at the same quality as the viewport? If your answer is that the benefit at least applies to Method 2, then it seems objective of MTHQ delay is fundamentally unattainable since CQ in my understanding implies less than HQ by definition – as such how could then MTHQ be possible? (Notice I highlighted the HQ portion of this term).

SA: Latter means CQ, but CQ as I explained above is not in itself a quality level. The MTHQ delay is the delay observed from the point where the quality in the viewport deteriorates to when it comes back to the viewport quality. This does not mean a fixed specific high-quality so MTHQ will still be attainable for CQ.

Comment from Imed:

Hi Saba,

I have a question on this approach of signaling a choice between HQ or larger margin.

Why does the receiver need to know about this choice? How can it effectively weigh the benefits and disadvantages of each? Is it the app or the user that will make this decision? What if things change during the session and that choice is no longer optimal?

Why can’t the sender adaptively decide, e.g. if the user is moving rapidly because of the nature of the current scene, it might decide to increase the margin and reduce quality; if the scene is focused on a certain region and the user is not looking around that much, then the sender may decide to reduce the margin and increase the quality.

In general, I am in favor of exposing less to the receiver and maintaining the intelligence in the sender side.

Reply from Saba:

I’ve revised the contribution for clarity, the change is highlighted and you can find my responses to the questions inline below.

https://www.3gpp.org/ftp/tsg_sa/WG4_CODEC/TSGS4_110-e/Inbox/Drafts/MTSI/S4-201154%20-%20rev1.doc

We hope our answers address your requests for clarifications.

I have a question on this approach of signaling a choice between HQ or larger margin.

Why does the receiver need to know about this choice? How can it effectively weigh the benefits and disadvantages of each? Is it the app or the user that will make this decision? What if things change during the session and that choice is no longer optimal?

SA: Whether the choice is exposed to the user and if it will be set once at the start of the session or can be renegotiated will depend on the application. However, there may be cases where a concentrated HQ in the viewport is desired, e.g., in case of a demo, the user may want the best possible quality in the viewport all the time.

Why can’t the sender adaptively decide, e.g. if the user is moving rapidly because of the nature of the current scene, it might decide to increase the margin and reduce quality; if the scene is focused on a certain region and the user is not looking around that much, then the sender may decide to reduce the margin and increase the quality.

SA: Yes, a smart sender can do as you suggest. But it doesn’t serve the same purpose. What you are suggesting still ends up lowering the quality in case of head motion, which may not be desirable. If a user wants the quality at highest possible, increasing the margins and lowering the quality in case of head motion is exactly what the user does not want.

Discussion on Wed telco session:

- Saba: comments from Charles should be addressed in revision just posted

- Saba: have I addressed your comments properly?

- Imed:... checking… his question is on sequences of content sender may notice lack of motion and increases viewport quality but decreases margin, whereas for opposite, sender reduces quality for same bitrate. Not understanding why deferring such control to the receiver side. What’s the value?

- Saba: I addressed your question via email: there might be certain user cases that wants to maintain CQ in viewport and don’t want sender to be over-reaching/over-smart; depends on use case

- Imed: not clear what boolean flag implies - might result in over-smart receiver

- Saba: still wish for receiver to have some level of control

- Ozgur: thinks proposal is reasonable: doesn’t prohibit control of viewport but leaves room for some preference expressed by receiver -w/o limiting sender’s option

- Iraj: agrees with Saba and Ozgur on permitting some control to receiver via preference indication; this provides some balance between sender and receiver control; thinks Nokia proposal is good

- Imed: still unsure such binary choice is given - without expressing amount of margin or specific quality

- Saba: there can be some connection to min/max margins signaled by sender

- Imed: OK

1154 is revised to S4-201201

| S4-201201 | Consistent vs. high-quality viewport with margins | Nokia Corporation |
| --- | --- | --- |

S4-201201 was agreed without presentation.

| S4-201021 | ITT4RT: Editorial Improvements for Permanent Document | Intel |
| --- | --- | --- |

The document was withdrawn.

| S4-201023 | Draft CR to TS 26.114 on Video Support for ITT4RT | Intel |
| --- | --- | --- |

The document was noted.

## 11.6 FS_FLUS_NBMP (Feasibility Study on the use of NBMP in E_FLUS)

| S4-201062 | FS_FLUS_NBMP: Updated Workplan | Tencent |
| --- | --- | --- |

Document was sent out for email discussion to agree on telco dates.

This was revised into S4-201199.

| S4-201199 | FS_FLUS_NBMP: Updated Workplan | Tencent |
| --- | --- | --- |

Document was presented.

Deadlines for document submission is missing.

This was revised into S4-201203.

| S4-201203 | FS_FLUS_NBMP: Updated Workplan | Tencent |
| --- | --- | --- |

The TP was agreed to be presented in the plenary agenda item 17.6

| S4-201061 | FS_FLUS_NBMP: Draft of Permanent Document | Tencent |
| --- | --- | --- |

Document was agreed via email.

| S4-201146 | FS_FLUS_NBMP: Call Flows and Edge Data Network Architecture for FLUS | Tencent |
| --- | --- | --- |

Presented by Iraj Sodagar of Tencent.

Discussion:

- Imed: I’m getting worried that we’re stepping on the toes of EMSA. Maybe it would be good to discuss the boundaries of EMSA and NBMP. I’m also wondering if it is a security risk to control the application server directly. It should go through an AS owned by the MNO?

- Iraj: I have a similar contribution to EMSA, combining SA6 edge server and 5GMSA. I agree on the similarities and FLUS_NBMP and EMSA need to agree on the type of interactions between an application provider and FLUS sink and the EMSA, and if there are any direct interactions between edge servers and those. We need to find the gaps first and part of the goal is to find those. We need to investigate if we need such interfaces (E9, E10, E11). I think Richard’s document discussed interactions with trusted application servers. We have not really discussed the role of the application provider yet.

- Imed: I still believe we need joint sessions between EMSA and FLUS.

- Iraj: Probably a good idea. A couple of joint calls? Do you agree with the approach? Do the study here and see what the gaps are?

- Imed: Yes, that’s the approach in EMSA too.

- Prakash: I had a similar question as Imed’s and got the answer, so I’m good.

- Nik: Do we add up to clause 2 to the PD and discuss clause 3 onwards with EMSA?

- Iraj: Do you mind if we also include clause 3?

- Imed: OK, my concern is clause 4.

The document was agreed including causes 2 and 3, but not 4 onwards.

## 11.7 Others including TEI

No contributions were received.

## 11.8 New Work / New Work Items and Study Items

| S4-201068 | revised SID on Media Negotiation Extensions for Super Resolusion | Huawei Technologies France |
| --- | --- | --- |

Presented by Yidan Teng of Huawei.

Discussion:

- Ozgur: Was this super-resolution metadata proposed in any other standards organization?

- Yidan: As far as I know, the main algorithms need no transfer of metadata.

- Ozgur: You could e.g. have user-generated content, not only conferencing, and you may need some alignment. Therefore, I would be interested if 3GPP could leverage work done in other groups.

- Yidan: I’m not aware of any other groups studying this. Do you think we should extend the use cases described in the proposal? Maybe we can make some limitations here using new use cases?

- Ozgur: For the case of conferencing and 3GPP environment, we need to tailor it to our needs, but it would be good to know of other solutions elsewhere.

- Imed: Thanks for the updates to this proposal. You mentioned that you’re not allowed to signal different resolutions. Where does this come from? I think imageattr can specify receive and send resolutions separately.

- Yidan: I think SR can be realized in the UE, but deep-learning algorithms can have high computation cost and may be better realized in the network. For the different resolutions, if we need to change image size, we need new SDP negotiations, IETF RFC 6236 (imageattr).

- Imed: imageattr can indicate send and receive resolutions. The answer can reduce the number of resolutions from the offer, but how would it work? I think the use case was not very clear; was the sender aware of the SR or not (is it transparent to the sender)? Is the MCU re-authoring the SDP?

- Yidan: There may be a slight change of the SDP for SR but the rest will be unchanged. We have done tests and SR update takes about 20 ms and SDP re-negotiation to update image resolution around a second.

- Imed: Would the SDP sender announce the different resolutions for SR? Does it control it?

- Yidan: Maybe we can add an attribute. That’s just one solution, but there could be other options.

- Saba: In 2.3 of the discussion paper, I would assume that both ends in the communication have uplink restrictions and therefore would end up in symmetry? Also should we include other methods beside SR to achieve the same functionality in this SI?

- Yidan: I think SR is of great value. We can negotiate capability for SR, even if it can alternatively be local to the UE. Do you know of other solutions?

- Saba: I’m not sure that this problem exists, because you could offer the resolutions and pick the one that is suitable for them.

- Yidan: Once you picked the resolution, it can be bad in worse network conditions. How can you change resolution when you need another round of SDP offer-answer?

- Iraj: If you want to change resolution dynamically during the session, why don’t you use SPS/VPS during the session, changing to a lower one to save the bandwidth?

- Yidan: You mean dynamic lower resolution?

- Iraj: Yes.

- Yidan: Can they change back up?

- Iraj: Yes.

- Yidan: If you negotiated the preferred resolution initially, can you change later?

- Iraj: Yes, you can change quantization and resolution during the session (in the bitstream). Why are you not considering this as part of the video encoding-decoding, rather than at the system layer?

- Yidan: We believe that the negotiation sets the resolution of the video to use.

- Iraj: My point is that you can change the resolution during the session without re-negotiation.

- Nik: Suggest to bring this for email discussion at the MTSI mailing list.

- Yidan: Can we bring it to a telco?

- Nik: Let’s make a request for a telco.

The document was noted.

## 11.9 Any Other Business

The chair called the delegates attention to a telco being scheduled for Sept 30th to continue discussing the SID proposal on Media Negotiation Extensions for Super Resolution.

## 11.10 Close of the session

The MTSI SWG chairman, Nikolai Leung thanked the delegates and closed the session at 18:20 CEST on August 26, 2020.

# Annex A: Meeting Agenda

MTSI SWG Tdoc Reservation (S4-201188 - S4-201207): 1188 (SWG report), 1189, 1190, 1191, 1192, 1193, 1194, 1195, 1196,1197, 1198, 1199, 1200, 1201, 1202, 1203

| 11.1 | Opening of the session |  |
| --- | --- | --- |
| 11.2 | Registration of documents |  |
| 11.3 | Reports and liaisons from other groups |  |
| 11.4 | CRs to Features in Release 16 and earlier | 26.114: 1006a, 1041->1190awp, 1057->1189awp 26.223: 1043->1191awp 26.238: 1053->1192a |
| 11.5 | ITT4RT (Support of Immersive Teleconferencing and Telepresence for Remote Terminals) | PD: 1024a, 1194 1018->1195n, 1019a, 1020->1197a, 1022->1196->1198->1202awp, 1045a, 1047a, 1142a, 1153->1200, 1154->1201awp, 1151w 1021w 26.114: 1023n TP: 1025->1193 |
| 11.6 | FS_FLUS_NBMP (Feasibility Study on the use of NBMP in E_FLUS) | PD: 1061a TP: 1062->1199->1203awp 1146a |
| 11.7 | Others including TEI |  |
| 11.8 | New Work / New Work Items and Study Items | FS_MNE_SR: 1068n 1069w |
| 11.9 | Any Other Business |  |
| 11.10 | Close of the session |  |

Sent for email discussion

# Annex B: List of participants

56 delegates attended

## Self-registration attendance list

If you are attending, please remove the underline if your name is listed.  Otherwise please add your name and organization to the list.

| Name | Organization Represented |
| --- | --- |
| Abhishek, Rohit | Tencent |
| Ahsan, Saba | Nokia |
| Aoki, Shuichi | NHK |
| Bouazizi, Imed | Qualcomm |
| Bruhn, Stefan | Dolby Laboratories, Inc. |
| Burman, Bo | Ericsson LM |
| Curcio, Igor | Nokia |
| Deshpande, Sachin | Sharp |
| Dillen, Paul | Philips International B.V. |
| Döhla, Stefan | Fraunhofer IIS |
| Gudumasu, Srinivas | InterDigital |
| Gunkel, Simon | KPN N.V. |
| Hamza, Ahmed | InterDigital Communications, Inc. |
| Han, Jae-Shin | LG Electronics Inc. |
| Hu, James | AT&T |
| Isberg, Peter | Sony |
| Jelinek, Milan | VoiceAge Corporation |
| Jung, Kyunghun | Samsung Electronics |
| Kolan, Prakash | Samsung Research America |
| Laaksonen, Lasse | Nokia |
| Lee, Brian | Dolby Laboratories |
| Leung, Nikolai | Qualcomm Incorporated |
| Lo, Charles | Qualcomm Incorporated |
| Ma, Jingwang | Huawei |
| Morita, Naotaka | NTT |
| Moriya, Takehiro | NTT |
| Multrus, Markus | Fraunhofer IIS |
| Munoz, Isaac | Qualcomm |
| Oyman, Ozgur | Intel |
| Pan, Qi | Huawei |
| Plante, Fabrice | Apple |
| Pousi, Timo | Ericsson LM |
| Ragot, Stéphane | Orange |
| Saha, Jayeeta | ETSI MCC |
| Sanchez, Yago | Fraunhofer HHI |
| Schevciw, André | Qualcomm |
| Sen, Deep | Apple |
| Singer, David | Apple |
| Sodagar, Iraj | Tencent |
| Szucs, Paul | Sony Corporation |
| Teng, Yidan | Huawei |
| Toftgård, Tomas | Ericsson |
| Varga, Imre | Qualcomm |
| Wang, Min | Qualcomm Incorporated |
| Yang, Hyun-Koo | Samsung Electronics |
| Yip, Eric | Samsung Electronics |

## GoTo Meeting telco session attendance list (as produced by the SA4 Secretary)

Name

AT&T - James Hu

Apple - Fabrice Plante

CMCC-Yujian Yin

Charles Lo

China Mobile - Jiajun Zhou

Ericsson - Bo Burman

Fraunhofer HHI - Gurdeep Singh

Fraunhofer HHI - Yago Sanchez

HUAWEI-Qi Pan

Huawei - Yidan Teng

Intel - Ozgur Oyman

InterDigital - Ahmed Hamza

InterDigital - Srinivas Gudumasu

Jayeeta Saha - GTM0

KPN N.V. - Simon Gunkel

Min Wang

NTT - Naotaka Morita

Nokia - Saba Ahsan

QUALCOMM - Nikolai Leung

Qualcomm - Imed Bouazizi

SONY - Paul Szucs

SONY - Peter Isberg

Samsung - Eric Yip

Samsung - Hyunkoo Yang

Samsung - Kyunghun Jung

Samsung - Prakash Kolan

Samsung - Ryan Lee

Tencent - Iraj Sodagar

Tencent-Rohit Abhishek

Tencent-Zhuoyun Zhang

VoiceAge - Milan Jelinek

Fraunhofer IIS - Markus Multrus

Fraunhofer IIS - Stefan Döhla

HUAWEI- Qi Pan

Huawei - Jingwang Ma

Huawei-Yidan Teng

MCC - Jayeeta Saha

NHK - Shuichi AOKI

NOKIA - Lasse Laaksonen

Nokia - Igor Curcio

Orange - Stéphane Ragot

Philips - Paul Dillen

Sharp - Sachin Deshpande

Apple - David Singer

Apple - Deep Sen

Dolby - Brian Lee

Dolby - Stefan Bruhn (SA4 VC)

Ericsson - Timo Pousi

Ericsson - Tomas Toftgård

Jayeeta Saha - GTM (C=2)

LGE - Jaeshin Han

NTT - Takehiro Moriya

NTT Takehiro Moriya

Qualcomm - Imre Varga

Qualcomm - Andre Schevciw

Qualcomm - Isaac Munoz

Nikolai Leung, Qualcomm Inc., with detailed minutes provided by Bo Burman, Charles Lo, and Iraj Sodagar