3GPP TSG-SA4 Meeting #110e	S4-201056

# Virtual meeting, August 19 - 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 1.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on Metrics Reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Metrics reporting clauses 7.8 and 11.4 are currently empty. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add text in clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to provision or report metrics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8, 11.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

================================== First change ==================================

## 7.8	Metrics Reporting Provisioning API

### 7.8.1	Overview

The Metrics Reporting Provisioning API allows an MNO or a 5GMSd Application Provider to configure the Metrics Collection and  Reporting procedure for a particular Provisioning Session at interface M1d.

### 7.8.2	Resource structure

The Metrics Reporting Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.8.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.8.2-1: Metrics Reporting Configuration resource

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a metrics configuration | metrics-reporting-configuration | POST | Create a configuration; returns the {configurationId}. |
| Set metrics configuration | metrics-reporting-configuration/{configurationId} | PUT | Set or update a configuration, activates reporting. |
| Read metrics configuration |  | GET | Read the values of an existing configuration. |
| Delete metrics configuration |  | DELETE | Delete a configuration, disables reporting. |

### 7.8.3	Data model

The data model for the MetricsReportingConfiguration resource is specified in Table 7.8.3-1 below:

Table 7.8.3-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| scheme | Array(URI String) | 0..1 | The scheme associated with this metrics configuration. A scheme may be associated with 3GPP or with a non-3GPP entity. If not specified, the 3GPP metrics scheme "urn:3GPP:ns:PSS:DASH:QM10" from TS 26.247 shall apply. |
| serverAddresses | Array(URL String) | 1..N | A list of 5GMSd AF addresses to which metric reports shall be sent for this metrics configuration. |
| dataNetworkName | String | 0..1 | The Data Network Name (DNN) which shall be used when sending metrics report for this metric configuration. If not specified, the default DNN shall be used. |
| reportingInterval | DurationSec | 0..1 | The sending interval between metrics reports for this metric configuration. If not specified, a single final report shall be sent after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | The proportion of streaming sessions that shall report metrics for this metric configuration. If not specified, reports shall be sent for all sessions. |
| urlFilters | Array(String) | 1..N | A list of content URL patterns for which metrics reporting shall be done for this metric configuration. If not specified, reporting shall be done for all URLs. |
| metrics | Array(String) | 1..N | A list of metrics which shall be collected and reported for this metric configuration. For the 3GPP scheme "urn:3GPP:ns:PSS:DASH:QM10" the listed metrics shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [7], and the quality reporting scheme and quality reporting protocol as defined in clauses 10.5 and 10.6, respectively, of [7] shall be used. If not specified, the complete (or default if applicable) set of metrics associated with the specified scheme will be collected and reported. |

================================== Next change ==================================

## 11.4		Metrics Reporting API

### 11.4.1	Overview

The Metrics Reporting API allows the Media Session Handler to send metrics reports to the 5GMSd AF.

### 11.4.2	Resource structure

The Metrics Reporting API is accessible through the following URL base path:

{apiRoot}/3gpp-m5d/v1/metrics-reporting/

Table 11.4.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API.

Table 11.4.2-1: Operations supported by the Metrics Reporting API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Send a metrics report | None | POST | Upload a metrics report to the 5GMSd AF |

### 7.4.3	Data model

The format of the uploaded metrics report shall be as specified in TS 26.247 clause 10.6.