Source:	Samsung Electronics Co., Ltd.

Title:	FS_5GSTAR: Use case categorization (Revised)

Agenda Item:	10.9

Document for:	Discussion and Agreement

- Introduction

Currently, there are 15 existing use cases agreed from TR26.928 and 4 more new cases collected during 5GSTAR discussions, tentatively. With those, this contribution proposes a baseline categorization for further detailed discussion of requirements and device functional structures.

Note that new use cases still require confirmation as to whether they can provide new features, and the proposed categorization here may need to be revised accordingly.

- Categorization of use cases

Based on the media to be transmitted through uplink/downlink, and the delay requirements for each scenario, the collected use cases can be categorized as follows:

| Categorization | Media flow1) | Media generation characteristics | Media delivery characteristics2) |
| --- | --- | --- | --- |
| AR/MR conversational | Bi-directional between 2 UEs | User real-time capture | Low latency |
| AR/MR messaging | Uni-directional between 2 UEs | User or 3rd party non real-time capture | Best effort |
| AR/MR download | Uni-directional by single UE | 3rd party non real-time pre-capture | Best effort |
| AR/MR streaming | Uni-directional by single UE | 3rd party non real-time pre-capture | Low latency |

Note 1) This represents the flow only for media data. Other signalling information such as pose or sensor data are not considered here.

Note 2) Specific numbers for latency requirements expected to be dependent on the specific service and use case, which are for further investigation.

- AR/MR conversational: A user captures objects or surroundings using a media camera and transmits the captured media via uplink in a real-time manner. The captured media may be avatars, an actual 3D face/body, 2D/360 video of surroundings (or even 3D), or 3D audio. All media are possibly processed at the edge, delivered to another user having AR/MR glasses, and finally represented. The total end-to-end delay should be kept sufficiently low in order not to cause any annoying delay, depending on the service. A receiver device may exchange pose information to the edge for media processing (e.g., split rendering).

- AR/MR messaging: A user captures objects or surroundings using a media camera and shares them with other user(s). The delay for capturing and uplink/downlink transmission do not need to be as strict as that of the AR/MR conversational use cases. The captured media is primarily non-timed (e.g., image, picture), but timed media (e.g., video) is also possible based on the services.

- AR/MR download: The pre-captured/generated and stored media in the application server is downloaded and represented in the AR/MR glasses. The uplink for media transmission is not required and the downlink delay does not need to be as strict as that of AR/MR conversational use cases. The captured media is primarily non-timed (e.g., image, picture), but timed media (e.g., video) is also possible based on the services.

- AR/MR streaming: The pre-captured/generated and stored media in the application server is streamed in a real-time manner. The timed media is primarily transmitted in the downlink where the delay should be sufficiently low such that is comparable to current streaming services. The AR/MR glasses may send its pose information to the edge for media processing (e.g., split rendering).

Table X-1 details its mapping to the proposed category.

[Table X-1] List of categorized use cases

| Category | No3) | Use Case Title |
| --- | --- | --- |
| Conversational | 7 | Real-time 3D Communication |
|  | 8 | AR guided assistant at remote location (industrial services) |
|  | 9 | Police Critical Mission with AR |
|  | 12 | 360-degree conference meeting |
|  | 15 | XR Meeting |
|  | 16 | Convention / Poster Session |
|  | 17 | AR animated avatar calls |
|  | 18 | AR avatar multi-party calls |
|  | 19 | Front-facing camera video multi-party calls |
|  | 20 | AR Streaming with Localization Registry |
|  | 2.2 | AR remote cooperation |
|  | 2.3 4) | AR remote advertising |
|  | 2.4 | AR Conferencing |
| Messaging | 1 | 3D Image Messaging |
|  | 8 | AR guided assistant at remote location (industrial services) |
|  | 11 | Real-time communication with the shop assistant |
|  | 2.3 4) | AR remote advertising |
| Download | 2 | AR Sharing |
|  | 10 | Online shopping from a catalogue – downloading |
|  | 11 | Real-time communication with the shop assistant |
|  | 23 | Spatial Shared Data |
| Streaming | 20 | AR Streaming with Localization Registry |
|  | 2.1 | Streaming volumetric video for glass-type MR devices |

Note 3) For existing use cases in TR26.928, the same use case number is used. For those in 5GSTAR, the section number in the permanent document is used (tentatively). Re-numbering may be needed after confirmation.

Note 4) Needs further clarification.

Note 5) Some of use cases may be duplicated into multiple categories based on the descriptions.

- Proposal

We propose to include the text and table in section 2 of this document into the Permanent Document as a baseline for detailed discussions on device functional structures and the way forward.