Source:	Samsung Electronics Co., Ltd.

Title:	ITT4RT: On text for Fisheye Video

Agenda Item:	11.5

Document for:	Discussion and Agreement

# Introduction

Section X.4.5 Fisheye Video of the dCR (S4aM200591) was taken directly from the text in the latest version of the Permanent Document (v0.8.1), which was before the agreement decision of the updated text in S4-AHM584 on fisheye SDP signalling during the 16th September Telco on ITT4RT.  Since AHM584 was agreed (minus a bracketed section), this documents provides an update of the text for section X4.5 Fisheye Video, integrating the agreed text and improving readability in the dCR.

# Changes

## *** Start change ***

## X.4.5	360-Degree Fisheye Video

## X.4.5.1	Identifying the 360-degree fisheye video stream

The SDP attribute 3gpp_fisheye is used to indicate a 360-degree fisheye video stream.

ITT4RT clients supporting 360-degree fisheye video shall support the 3gpp_fisheye attribute and shall support the following procedures:

- when sending an SDP offer, the ITT4RT-Tx client includes the 3gpp_fisheye attribute in the media description for video in the SDP offer

- when sending an SDP answer, the ITT4RT-Rx client includes the 3gpp_fisheye attribute in the media description for video in the SDP answer if the 3gpp_fisheye attribute was received in an SDP offer

- after successful negotiation of the 3gpp_fisheye attribute in the SDP, for the video streams based on the HEVC codec, the MTSI clients exchange an RTP-based video stream containing an HEVC or AVC bitstream with fisheye omnidirectional video specific SEI messages as defined in clause X.3

ITT4RT-Tx clients that support both 360-degree projected video and 360-degree fisheye video may include both 3gpp_360video and 3gpp_fisheye attributes as alternatives in the SDP offer, but an ITT4RT-Rx client shall include only one attribute (either 3gpp_360video or 3gpp_fisheye, based on support or selection) in the SDP answer.

## X.4.5.2	360-degree fisheye video SDP attribute parameters

Media-line level parameters are defined in order to aid session establishment between the ITT4RT-Tx and ITT4RT-Rx clients for 360-degree fisheye video, as well as to describe the fisheye video stream as identified by the 3gpp_fisheye attribute.

- Total number of fisheye circular videos at the capturing terminal

Depending on the camera configuration of the sending terminal, the 360-degree fisheye video may be comprised of multiple different fisheye circular videos, each captured through a different fisheye lens.

- <fisheye>: this parameter indicates the total number of fisheye circular videos output by the camera configuration at the terminal

- Fisheye circular video static parameters

In order to enable the quick selection of desired fisheye circular videos by the ITT4RT-Rx client during SDP negotiation, the following static parameters are defined for each fisheye circular video.  These parameters are defined from the video bitstream fisheye video information SEI  message as defined in ISO/IEC 23008-2 [R4] and ISO/IEC 23090-2 [R1].

- <fisheye-img-X> = [<id-X> <azi> <ele> <til> <fov>]

  - <id>: an identifier for the fisheye video

  - <azi>, <ele>: azimuth and elevation indicating the spherical coordinates that correspond to the centre of the circular region that contains the fisheye video, in units of 2-16 degrees.  The values for azimuth shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive, and the values for elevation shall be in the range of −90 * 216 (i.e., −5 898 240) to 90 * 216 (i.e., 5 898 240), inclusive

  - <til>: tilt indicating the tilt angle of the sphere regions that corresponds to the fisheye video, in units of 2−16 degrees. The values for tilt shall be in the range of −180 * 216 (i.e., −11 796 480) to 180 * 216 − 1 (i.e., 11 796 479), inclusive

  - <fov>: specifies the field of view of the lens that corresponds to the fisheye video in the coded picture, in units of 2−16 degrees.  The field of view shall be in the range of 0 to 360 * 216 (i.e., 23 592 960), inclusive

- Stream packing of fisheye circular videos

Depending on the terminal device capabilities and bandwidth availability, the packing of fisheye circular videos within the stream can be negotiated between the sending and receiving terminals.

- <maxpack>: this parameter indicates the maximum supported number of fisheye videos which can be packed into the stream by the terminal device

An example SDP offer is shown in table X.A.

Table X.A: Example SDP offer with 360-degree fisheye video attribute parameters

SDP offer

m=video 49154 RTP/AVP 99 a=tcap:1 RTP/AVPF a=pcfg:1 t=1 b=AS:10000 b=RS:0 b=RR:2500 a=rtpmap:99 H265/90000 a=fmtp:99 profile-id=1; level-id=93; a=imageattr:99 send [x=1920,y=1080] [x=1280,y=720] a=3gpp_fisheye:99 fisheye=4 [id=1 azi ele til fov] [id=2 azi ele til fov] [id=3 azi ele til fov] [id=4 azi ele til fov]; maxpack=2; a=sendonly

As an example, a receiving terminal which only receives 360-degree fisheye video (and possibly sends a 2D video to the sender) replies with an SDP answer containing only the selected fisheye videos equal to the number as selected by the value of maxpack in the corresponding m-line, which is set to recvonly.

## X.4.5.3	Viewport dependent delivery of fisheye video

By exposing the coverage information of each fisheye circular video using the parameters in section X.4.5.2, the collective multitude of which makes up the whole 360-degree video, a ITT4RT-Rx client can opt to select only the required fisheye circular videos needed to render the current viewport of the user.

Through the parameters defined in section X.4.5.2, a ITT4RT-Rx client can select the desired fisheye packing configuration of the video stream during SDP negotiation, as well as the initial desired fisheye videos using the id parameter.

## *** End change ***

# Proposal

We propose to include the updated changes to the dCR text as specified in section 2 into the next version of the dCR.