3GPP TSG-SA4 Meeting #111e 	S4-201433

# Still at Home,  11. – 20.November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | <CR#> | rev | <Rev#> | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the missing SdfMethod type definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 6.11.2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The SdfMethod type is used within the ServiceAccessInformation resource type, but not defined. The intention is that the Media Session Handler should use some specific Service Data Flow Description method. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A definition of the SdfMethod type is added to the specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.3.1, New section 11.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformtion resource is specified in Table 11.2.3.1-1 below:

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | RO | Unique identification of the M1d Provisioning Session. |
| StreamingAccess | Object | 0..1 | RO |  |
| mediaPlayerEntry | URL String | 1..1 | RO | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |
| ClientConsumptionReporting‌Configuration | Object | 0..1 | RO |  |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |
| DynamicPolicyInvocationConfiguration | Object | 0..1 | RO |  |
| serverAddresses | Array(URL String) | 1..N | RO | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |
| validPolicyTemplateIds | Array(String) | 1..N | RO | A list of Policy Template identifiers which the 5GMSd Client is authorized to use. |
| sdfMethods | Array(SdfMethod) | 1..N | RO | A list of recommended service data flow description methods (descriptors). E.g. 5-Tuple, ToS, 2-Tuple, etc, which should be used by the Media Session Handler to describe the service data flows for the to be policed traffic. |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. Example: "HD_Premium" |
| ClientMetricsReportingConfiguration | Object | 0..1 | RO |  |
| serverAddresses | Array(URL String) | 1..N | RO | A list of 5GMSd AF addresses to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) |
| dataNetworkName | String | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |
| urlFilters | Array(String) | 1..N | RO | A list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |
| metrics | Array(String) | 1..N | RO | A list of metrics which shall be reported. |
| NetworkAssistanceConfiguration | Object | 0..1 | RO |  |
| serverAddress | URL String | 1..1 | RO | Address of the 5GMSd AF that offers the APIs for 5GMSd AF-based Network Assistance, for access by the 5GMSd Media Session Handler. This address shall be an opaque URL, following the 5GMS URL format. |

**** Next Change ****

#### 11.2.3.2	SdfMethods enum

The data model for the SdfMethod enumeration is specified in Table 11.2.3.1-1 below:

Table 11.2.3.2-1: Definition of SdfMethod enumeration

| Enumeration value | Description |
| --- | --- |
| 5Tuple | The Media Session Handler shall use 5-Tuples for Service Data Flow descriptions. The 5 Tuple should not contain a wildcard. |
| 2Tuple | The Media Session Handler shall use a 2-Tuple of UE IP and Server IP as Service Data Flow Description. |
| typeOfServiceMarking | The Media Session Handler shall apply Type of Service (ToS) marking to the Service Data Flow. |
| flowLabel | The Media Session Handler shall apply IPv6 flow label marking and provide the IPv6 flow label of the Service Data Flow. |
| domainName | The Media Session Handler shall provide the domain name of the 5GMSd AS. |

**** Last Change ****