3GPP TSG-SA4 Meeting # 112-e	S4-210185

1st – 10th February 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR | <CR#> | rev |  | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR Introduction of Pause/Resume in ITT4RT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc., Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## X.6.4	Overlays

ITT4RT clients supporting the ‘Overlay’ feature may define an overlay source and overlay configuration in the SDP. An ITT4RT client that supports overlays

- shall support a video or image stream indicated by a media line in the SDP as the source of an overlay.

- [may support a conditional overlay of a region from a 360-degree video as the source of an overlay.]

- [may support an externally specified overlay source, referenced by a URI.]

Editor’s Note: The solution for image overlays is FFS.

### X.6.4.1 Video Media

Any visual media that is defined with the ‘m=video’ line, includes the attribute mid and does not have the attribute a=3gpp_360video in the SDP may be rendered as an overlay by an ITT4RT-Rx client. An ITT4RT client shall include the attribute mid in the overlay media description. If an overlay is to be associated with a particular overlay configuration, the mid shall be used to associate the overlay media description to the the overlay configuration, which is later described in X.6.4.3.

### X.6.4.2 Overlay Configuration

[…]

Second Change

### X.6.7 Support for Stream Pausing/Resuming

An ITT4RT-Tx client shall use the a=rtcp-fb ccm pause attribute and parameter values as specified in [RFC 5104] and [RFC 7728] to indicate the capability to support receiving and acting on PAUSE and RESUME requests targeted for RTP streams it sends.  The optional parameter setting of a=rtcp-fb ccm pause config=3 could be used by the ITT4RT-Tx client to indicate that it will only receive and react to PAUSE and RESUME requests but will not send them.

An ITRT4RT-Rx client shall use the a=rtcp-fb ccm pause attribute and parameter values as specified in [RFC 5104] and [RFC 7728] to indicate the capability to support sending PAUSE and RESUME requests targeted for RTP streams it receives.  The optional parameter setting of a=rtcp-fb ccm pause config=2 could be used by the ITT4RT-Rx client to indicate that it will only send PAUSE and RESUME requests but does not support receiving these requests.