3GPP TSG SA WG4 #113e	S4-210609

# E-meeting, 6th – 14th April 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.116 | CR | <CR#> | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Chroma Sample Location Type for BT.2020 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For BT.2020, the chroma_sample_loc_type_top_field in the VUI needs to be set to 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Sets the value for profiles that may use BT.2020 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambigous specification leading to interop problems |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.3.5, 4.5.4.5, 4.5.5.5, 4.5.6.5, 4.5.7.5, 4.5.8.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

#### 4.5.3.5	Colour information

A Bitstream conforming to the H.265/HEVC Full HD Operation Point shall use either Recommendation ITU-R BT.709 [3] colorimetry or Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance.

-	BT.709 [3] shall be signalled by setting colour_primaries to the value 1, transfer_characteristics to the value 1 and matrix_coeffs to the value 1.

-	BT.2020 [4] shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9. The chroma_sample_loc_type_top_field shall be set to 2.

A Receiver conforming to the H.265/HEVC Full HD Operation Point shall be capable of decoding Bitstreams that use Recommendation ITU-R BT.709 [3] and ITU-R BT.2020 [4] colorimetry. Such a Receiver should support ITU-R BT.2020 [4] signalling and provide an appropriate mapping of the signal to the supported colour space of the device.

NOTE:	Colour spaces are not associated to any particular spatial resolution.

=====  CHANGE  =====

#### 4.5.4.5	Colour information

A Bitstream conforming to the H.265/HEVC UHD Operation Point shall use Recommendation ITU-R BT.2020 [4] colorimetry in non-constant luminance.

-	BT.2020 [4] shall be signalled by setting colour_primaries to the value 9, transfer_characteristics to the value 14 and matrix_coeffs to the value 9. The chroma_sample_loc_type_top_field shall be set to 2.

A Receiver conforming to the H.265/HEVC UHD Operation Point shall be capable of decoding Bitstreams that use ITU-R BT.2020 [4] colorimetry. Such a Receiver should support ITU-R BT.2020 [4] signalling and provide an appropriate mapping of the signal to the supported colour space of the device.

=====  CHANGE  =====

#### 4.5.5.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC Full HD HDR Operation Point that uses PQ HDR shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to the value 16,

-	matrix_coeffs shall be set to the value 9.

-	The chroma_sample_loc_type_top_field shall be set to 2.

This signalling implies that BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.6.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC UHD HDR Operation Point that uses PQ HDR shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to the value 16,

-	matrix_coeffs shall be set to the value 9.

-	The chroma_sample_loc_type_top_field shall be set to 2.

This signalling implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Perceptual Quantization (PQ) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.7.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC Full HD HDR HLG Operation Point shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to either the value 18, or to the value 14. In the latter case, the Bitstream shall also contain the alternative_transfer_characteristics SEI message. The alternative_transfer_characteristics SEI message shall be inserted at each RAP, and its parameter preferred_transfer_characteristics shall be set to the value 18.

-	matrix_coeffs shall be set to the value 9.

-	The chroma_sample_loc_type_top_field shall be set to 2.

This signalling implies that BT.2020 [4] colorimetry in non-constant luminance and Hybrid Log Gamma (HLG) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.

=====  CHANGE  =====

#### 4.5.8.5	Colour information and HDR transfer characteristics

A Bitstream conforming to the H.265/HEVC UHD HDR HLG Operation Point shall comply with the following restrictions in the VUI:

-	colour_primaries shall be set to the value 9,

-	transfer_characteristics shall be set to either the value 18, or to the value 14. In the latter case, the Bitstream shall also contain the alternative_transfer_characteristics SEI message. The alternative_transfer_characteristics SEI message shall be inserted at each RAP, and its parameter preferred_transfer_characteristics shall be set to the value 18.

-	matrix_coeffs shall be set to the value 9.

-	The chroma_sample_loc_type_top_field shall be set to 2.

This signalling implies that Recommendation BT.2020 [4] colorimetry in non-constant luminance and Hybrid Log Gamma (HLG) electro-optical transfer function (EOTF) as defined in Recommendation ITU-R BT.2100 [11] are in use.