3GPP SA4#114-e meeting	S4-210798

May 18th – 28th 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.114 | CR |  | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | S4-210798 ITT4RT: Presentation Overlay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | ITT4RT |  |  |  |  |  | Date: |  |  | 2021-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | How to handle presentation type content in ITT4RT is on requrement of the work item and currently not sufficiently addresed in 26.114 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposed a new section Y.6.4.4. to add functionality and message flows to handle signalling, detection and replacement of presentation content in the 360-degree content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to handle presentation type content in ITT4RT is not poperly addressed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Y.6.4.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== 1st CHANGE  =====

## Y.6.4.4	ITT4RT presentation overlay (screen share)

To prevent degradation of presentation material (e.g., slides, video, notes) that may be captured from a display (screen or projector) with a 360-degree camera, the captured content in the 360-degree video can be replaced with the original presentation material. This replacement could either be performed in the client sending the 360-video or in the network (MRF/MCU)

.

When replacement is performed in the client sending the 360-video, the availability of the presentation content should be signalled by the source of the content to the client sending the 360-video using the SDP parameter “a=content:slides”[81].

When replacement is performed in the network (MRF/MCU)

the SDP offer from the sending 360-degree ITT4RT client and the SDP answer from the MRF/MCU should include the “a=3gpp_360video_overlayreplacement:true” attribute to indicate that content can be replaced.

The availability of the presentation content should be signalled by the source of the content to the MRF/MCU using the SDP parameter “a=content:slides”[81]. If no overlay parameters are given by the source of the presentation content (e.g., as Sphere-relative Overlay Configuration, see Y.6.4.3.2), the MRF/MCU should identify those parameters prior to the replacement.