3GPP TSG- S4 Meeting #116e	S4-211519

# Electronic Meeting, 10th November – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | <CR#> | Rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Clause 7: MBS User Service Provisioning and Ingest Procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_EXT |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The intention of this pCR is to start progressing the provisioning and ingest procedure. It is assumed, that the procedures need to be refined over multiple conf calls. Note, this pCR should not drive terminology / naming of features. The terminology in Clause 7 should follow the terminology of clause 4 and 5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 7	MBS User Services Procedures

## 7.1	General

Editor’s Note: for the AF/AS, it is better to hide the 5GC technology (for example TMGI, session, MBS bearer etc.), and just encapsulate them as the service, for example file download service, Live streaming service, Group Communication Service. The AF/AS will just configure the service(s) and service related properties to the MBSF.

## 7.2	User Service Provisioning Procedure

This clause describes the baseline user service provisioning procedure for MBS Service.

## Editor’s note: for the MBS user service provisioning request, it may include multiple requests/responses to provide the required properties for the ingestion. It should be depicted in the MBSF Services clause.

Figure 7.2- 1: MBS User Service Provisioning procedure

Prerequisites:

-	The AF has completed the SLA negotiation with Network Operator.

-    The AF has prepared the MBS user service configuration.

Steps:

- AF/AS authenticate itself in the 5MBS system.

- AF/AS sends the MBS User Service provisioning request to MBSF.

- MBSF should interact with MB-SMF for the resource allocation for the MBS user service. For the detailed procedure, refer to 3GPP TS 23.247 clause 7.1.1.

- MBSF generates the Service Announcement and updates the SA to UEs via MBS Session or unicast PDU Session if the required properties for SA are provided.

- MBSF selects the MBSTF. If the allocation of an ingress address was requested in step 2, the MBSF requests the MBSTF to reserve user plane ingress resources. If the allocation of an ingress transport address was not requested in step 2, the MBSF provides the source specific multicast address received in step 2 and requests the MBSTF to join the corresponding multicast tree from the AF.

- If ingress address requested, the MBSTF allocates an ingress address (IP address and port) and responses it to MBSF.

- MBSF sends the MBS User Service provisioning response to AF/AS. If the ingress address was requested in step 2, the MBSTF ingress address shall be included in the response.

## 7.3	Object Distribution User Service ingest procedure

This clause describes the baseline ingestion procedure for Object Distribution Service.

Editor’s Note: The following aspecs are considered in the call flow design

- Large scale deployments, i.e. multiple MBSTFs are needed to serve the target area

- It can make sense to reuse an TMGI for multiple (subsequent) distribution services, i.e. TMGI is already allocated at session creation

- Support of Dynamic PCC

Figure 7.3-1: Object Distribution User Service ingestion procedure

Prerequisites:

-	The AF has provisioned an MBS User Service for Object Distribution Method.

Steps:

1.	AF/AS may provision/update the object manifest of the MBS User Service, for example the files for the Pull Mode.

2.	[Condition] If all required properties for the object manifest are provided, MBSF may send the Nmb2_session_update request to change the object manifest of the session.

3.	[Pull Ingest] MBSTF pulls the object(s) from AF/AS when the PULL Mode and object URLs is configured. The objects could be pull from AF/AS after the session is activated and should be ready before the file distribution time.

4.	[Push Ingest] AF/AS pushes the object(s) to MBSTF when the PUSH Mode is configured. The objects could be continuously ingested after the session is activated.

5.	MBSTF performs the object segmentation, and AL-FEC encoding if FEC is configured.

6.	[Optional] MBSTF sends the status event(s) to MBSF if the status notification was subscribed.

7.	[Optional] MBSF generates the Service Announcement and updates the SA to UEs via MBS Session or unicast PDU Session if the required properties for SA are changed.

## 7.4	Packets Distribution User Service Ingestion procedure

This clause describes the baseline ingestion procedure for Packets Distribution Service.

Figure 7.4-1: Provision Packets Distribution Service Ingestion Procedure

Prerequisites:

-	The AF has provisioned an MBS User Service for Packets Distribution Method.

Steps:

1.	AF/AS may provision/update the packets distribution manifest of the MBS User Service.

2.	MBSF may send the Nmb2_session_update request to change the packets distribution manifest of the session.

3.	[Pull Ingest] MBSTF establishes the RTSP session with the AS if Pull Ingest is configured.

4.	[Pull Ingest] The AS pushes the UDP packets continuously to MBSTF.

5.	[Push Ingest] AS pushes the UDP packets continuously to MBSTF if Push Ingest is configured.

6.	[Condition] MBSTF sends the status event(s) to MBSF if the status notification was subscribed.

**** Last Change ****