3GPP TSG SA WG4#116e	S4-211557

# E-meeting, 10th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.116 | CR | 0017 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference Fixes to TV Video Profiles |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | 16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some bugs are identified - Wrong clauses referenced - Missing introduction to clause - Misleading clause headings |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) fix referencing 2) add sentence 3) update clause heading |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementers are confused |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2, 5.2.3, 5.3.2, 5.3.3, 5.4.2, 5.4.3, 5.5.2, 5.5.3, 5.6.2, 5.6.3, 5.7.2, 5.7.3, 5.8.2, 5.8.3, 5.9.2, 5.9.3, 5.10.2, 5.10.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.2.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.2.4 shall apply.

=====  CHANGE  =====

### 5.2.3	File Format Constraints

For Representations conforming to the H.264/AVC 720p HD Operation Point the following requirements apply.

The requirements as defined in clause 5.1.2 shall apply. In addition, the following shall apply.

The syntax and values for visual sample entry shall be set as follows:

-	It shall conform to AVCSampleEntry ('avc1') or AVCSampleEntry ('avc3') as defined in ISO/IEC 14496-15 [9].

-	If AVCSampleEntry ('avc3') is used the following requirements apply:

-	If the sample is a Sync Sample, all parameter sets needed for decoding that sample SHALL be included in the sample itself.

-	If the sample is at the start of a Segment or a Subsegment, (i.e. a random access point position) that is not a Sync Sample, all parameter sets needed for decoding that sample shall occur in one of the samples between the starting point and that sample inclusive.

-	The video track shall be encoded using the requirements and recommendations for H.264/AVC 720p HD Operation Point as defined in clause 4.4.2.

=====  CHANGE  =====

### 5.3.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.3.4 shall apply.

=====  CHANGE  =====

### 5.3.3	File Format Constraints

For Representations conforming to the H.264/AVC Full HD Operation Point the following requirements apply.

The requirements as defined in clause 5.2.3 shall apply. In addition, the video track shall be encoded using the requirements and recommendations for H.264/AVC Full HD Operation Point as defined in clause 4.4.3.

=====  CHANGE  =====

### 5.4.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.4.4 shall apply.

=====  CHANGE  =====

### 5.4.3	File Format Constraints

For Representations conforming to the H.265/HEVC 720p HD Operation Point the following requirements apply.

The requirements as defined in clause 5.1.3 shall apply. In addition, the following shall apply:

-	It shall conform to HEVCSampleEntry ('hvc1') or HEVCSampleEntry ('hev1') as defined in ISO/IEC 14496-15 [9];

-	If HEVCSampleEntry ('hev1') is used the following requirements apply:

-	If the sample is a Sync Sample, all parameter sets needed for decoding that sample shall be included in the sample itself.

-	If the sample is at the start of a Segment or a Subsegment, (i.e. a random access point position) that is not a Sync Sample, all parameter sets needed for decoding that sample occur in one of the samples between the starting point and that sample inclusive.

-	The video track shall be encoded using the requirements and recommendations for H.265/HEVC 720p HD Operation Point as defined in clause 4.5.2.

=====  CHANGE  =====

### 5.5.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.5.4 shall apply.

=====  CHANGE  =====

### 5.5.3	File Format Constraints

For Representations conforming to the H.265/HEVC Full HD Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. In addition, the video track shall be encoded using the requirements and recommendations for H.265/HEVC Full HD Operation Point as defined in clause 4.5.3.

=====  CHANGE  =====

### 5.6.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.6.4 shall apply.

=====  CHANGE  =====

### 5.6.3	File Format Constraints

For Representations conforming to the H.265/HEVC UHD Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC UHD Operation Point as defined in clause 4.5.4.

=====  CHANGE  =====

### 5.7.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.7.4 shall apply.

=====  CHANGE  =====

### 5.7.3	File Format Constraints

For Representations conforming to the H.265/HEVC Full HD HDR Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC Full HD HDR Operation Point as defined in clause 4.5.5.

If sample entry hvc1 is in use, then any possibly present Mastering display colour volume SEI message or any possibly present Content light level information SEI message shall be provided in the decoder configuration record and shall be constant for the entire file.

=====  CHANGE  =====

### 5.8.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.8.4 shall apply.

=====  CHANGE  =====

### 5.8.3	File Format Constraints

For Representations conforming to the H.265/HEVC UHD HDR Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC UHD HDR Operation Point as defined in clause 4.5.6.

If sample entry hvc1 is in use, then any possibly present Mastering display colour volume SEI message or any possibly present Content light level information SEI message shall be provided in the decoder configuration record and shall be constant for the entire file.

=====  CHANGE  =====

### 5.9.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.9.4 shall apply.

=====  CHANGE  =====

### 5.9.3	File Format Constraints

For Representations conforming to the H.265/HEVC Full HD HDR HLG Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC Full HD HDR HLG Operation Point as defined in clause 4.5.7.

=====  CHANGE  =====

### 5.10.2	MPD Signalling

The requirements as defined in clause 5.1.1 shall apply. In addition, the conditions in 5.10.4 shall apply.

=====  CHANGE  =====

### 5.10.3	File Format Constraints

For Representations conforming to the H.265/HEVC UHD HDR HLG Operation Point the following requirements apply.

The requirements as defined in clause 5.4.3 shall apply. The video track shall be encoded using the requirements and recommendations for H.265/HEVC UHD HDR HLG Operation Point as defined in clause 4.5.8.