3GPP TSG- S4 Meeting #116e	S4-211520

# Electronic Meeting, 10th November – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | <CR#> | Rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Clause 8: Network Function Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | The intention of this document is to start defining the MBSF and MBSTF Service APIs (Nmbsf and Nnmb2). It is assumed that the text needs to evolve over multiple conference calls. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 8	Network Function Services

## 8.1	General

The Network Function Services exposed by the MBSF and the MBSTF are defined in this clause.

Editor’s Note: This text assumes the re-use of an xMB-C profile for the Nmbsf API.

## 8.2	MBSF Services

### 8.2.1	General

The following table illustrates the set of NF services exposed by the MBSF.

Table 8.2-1: NF services provided by MBSF

| Service name | Service operation name | Operation semantics | Example consumer(s) |
| --- | --- | --- | --- |
| Nmbsf_MBSService | Create | Request/Response | AF, NEF |
|  | Retrieve | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Destroy | Request/Response | AF, NEF |
| Nmbsf_MBSSession | Create | Request/Response | AF, NEF |
|  | Retrieve | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Destroy | Request/Response | AF, NEF |
|  | StatusSubscribe | Subscribe/Notify | AF, NEF |
|  | StatusUnsubscribe |  | AF, NEF |
|  | StatusNotify |  | AF, NEF |

Editor’s Note: Service exposure to AF is FFS.

### 8.2.2	Nmbsf MBS Service operations

#### 8.2.2.1	Nmbsf_MBSService_Create service operation

Service operation name: Nmbsf_MBSService_Create

Description: Provision a new Multicast MBS Session or Broadcast MBS Session .

Input, Required: MBS Service ID.

Input, Optional: Service Name, service language, description, service type, Receive-Only Mode, Service Announcement mode, MBS service area, maximum bit rate, Consumption Reporting Configuration.

Output, Required: Result indication.

Output, Optional: Cause, service properties, Service Announcement fetch URL.

#### 8.2.2.2	Nmbsf_MBSService_Retrieve service operation

Service operation name: Nmbsf_MBSService_Retrieve

Description: Used by the NF/NEF to retrieve the properties of the MBS Service.

Input, Required: None

Input, Optional: MBS Service ID.

Output, Required: Result indication.

Output, Optional: Cause, properties of the MBS service(s)

#### 8.2.2.3	Nmbsf_MBSService_Update service operation

Service operation name: Nmbsf_MBSService_Update

Description: Update the established MBS Service properties.

Input, Required: MBS Service ID.

Input, Optional: Service properties.

Output, Required: Result indication.

Output, Optional: Cause.

#### 8.2.2.4	Nmbsf_MBSService_Destroy service operation

Service operation name: Nmbsf_MBSService_Destroy

Description: Release the resources of the associatedMBS Service.

Input, Required: MBS Service ID.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: Cause.

### 8.2.3	Nmbsf MBS Session operation

#### 8.2.3.1	Nmbsf_MBSSession_Create service operation

Service operation name: Nmbsf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS service configuration. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Service ID, MBS Session ID

Input, Optional: session start time, session stop time, MBS service area, maximum bit rate, max delay, Distribution Method, object delivery session, files, application session, packet delivery proxy mode session, packet distribution forward-only mode session, GC session.

Output, Required: Result indication.

Output, Optional: Cause, webdavUrl for push mode in object distribution session, webdavUrl for push mode in application session, MBSTF IP address for packet distribution method.

#### 8.2.2.2	Nmbsf_MBSSession_Retrieve service operation

Service operation name: Nmbsf_MBSSession_Retrieve

Description: This service operation is used by the AF/NEF to retrieve the properties of the session(s).

Input, Required: MBS Service ID

Input, Optional: MBS Session ID

Output, Required: Result indication.

Output, Optional: Cause, properties of the MBS Session(s)

#### 8.2.2.3	Nmbsf_MBSSession_Update service operation

Service operation name: Nmbsf_MBSSession_Update

Description: Update the established multicast session or broadcast session, e.g. prolong the MBS session duration, change files, change the MBS service area.

Input, Required: MBS Service ID, MBS Session ID.

Input, Optional: Session properties.

Output, Required: Result indication.

Output, Optional: Cause.

#### 8.2.2.4	Nmbsf_MBSSession_Destroy service operation

Service operation name: Nmbsf_MBSSession_Destroy

Description: Release the multicast session or broadcast session.

Input, Required: MBS Service ID, MBS Session ID.

Input, Optional: None.

Output, Required: Result indication.

Output, Optional: Cause.

#### 8.2.2.4	Nmbsf_MBSSession_Get service operation

Service operation name: Nmbsf_MBSSession_Get

Description: This service operation is used by the AF/NEF to retrieve the properties of the session(s).

Input, Required: MBS Service ID

Input, Optional: MBS Session ID

Output, Required: Result Indication.

Output, Optional: Cause, properties of the MBS Session(s)

#### 8.2.2.5	Nmbsf_MBSSession_StatusSubscribe operation

The Service Operation is used by AF/NEF to subscribe to a status(s) of one service, session, and/or files.

Service operation name: Nmbsf_MBSSession StatusSubscribe

Description: Invoked by AF/NEF on the MBSF, when it needs to create a subscription to monitor at least one event relevant to the service, session and/or file(s). The AF may subscribe to multiple events in a subscription.

Input, Required: MBS Service ID, Event ID(s), notification target address.

Input, Optional: MBS Session ID.

Output, Required: When the subscription is accepted: Subscription correlation ID.

#### 8.2.2.6	Nmbsf_MBSSession_StatusUnsubscribe operation

The Unsubscribe service operation is invoked by MBSF, towards the MBSTF, to remove an existing subscription previously created by itself at the MBSTF.

Service operation name: Nmbsf_MBSSession StatusUnsubscribe.

Description: The Unsubscribe service operation is to remove an existing subscription.

Input, Required: Subscription correlation ID.

Output, Required: Result Indication.

#### 8.2.2.7	Nmbsf_MBSSession_StatusNotify operation

The Notify service operation is invoked by the MBSTF, to send an event notification, towards the notification URI, when certain event included in the subscription has taken place.

Service operation name: Nmbsf_MBSSession_StatusNotify

Description: This service operation is used by the MBSTF to notify MBSF about the status change of the session or the status of the file.

Input, Required: MBS Service ID, Event ID(s).

Input, Optional: MBS Session ID, fileUrl, Status information.

Output, Required: Result Indication.

## 8.3	MBSTF Services

### 8.3.1	General

The following table illustrates the set of NF services exposed by the MBSTF.

Table 8.3-1: NF services provided by MBSTF

| Service name | Service operation name | Operation semantics | Example consumer(s) |
| --- | --- | --- | --- |
| Nmbstf_MBSSession | Create | Request/Response | MBSF |
|  | Retrieve | Request/Response | MBSF |
|  | Update | Request/Response | MBSF |
|  | Destroy | Request/Response | MBSF |
|  | StatusSubscribe | Subscribe/Notify | MBSF |
|  | StatusUnsubscribe |  | MBSF |
|  | StatusNotify |  | MBSF |

### 8.3.2	Nmbstf_MBSSession service

#### 8.3.2.1	Nmbstf_MBSSession_Create service operation

Service operation name: Nmbstf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session configuration.

Input, Required: MBS Session ID.

Input, Optional: MBS activation time, MBS termination time, QoS flow information, Bandwidth, unicast Address of MB-UPF, TSI if object delivery object is used, IP multicast address is sourced by MBSTF.

Output, Required: Result indication.

Output, Optional: Cause, Sourcing of IP Multicast if multicast is used.

#### 8.3.2.4	Nmbstf_MBSSession_Retrieve service operation

Service operation name: Nmbstf_MBSSession_Retrieve

Description: Retrieve an existing MBS Session.

Input, Required: None.

Input, Required: MBS Session ID.

Output, Required: Result indication.

Output, Optional: Cause, the properties of MBS session(s).

#### 8.3.2.2	Nmbstf_MBSSession_Update service operation

Service operation name: Nmbstf_MBSSession_Update

Description: Update an existing MBS Session, for example to change the session stop time, object delivery session, application session, packets delivery session, files, and ancillary information.

Input, Required: MBS Session ID.

Input, Optional: MBS termination time, unicast Address of MB-UPF list, object delivery session, application session, packets delivery session, ancillary info of the session.

Output, Required: Result indication.

Output, Optional: Cause, additional info (input parameters)

#### 8.3.2.3	Nmbstf_MBSSession_Destroy service operation

Service operation name: Nmbstf_MBSSession_Update

Description: delete a multicast session or broadcast session during MBS session configuration

Input, Required: MBS Session ID.

Output, Required: Result indication.

Output, Optional: Cause.

#### 8.3.2.5	Nmbstf_MBSSession_StatusSubscribe operation

The Status Subscription Operation is used by the MBSF to subscribe to a status(s) of one session, and/or files.

Service operation name: Nmbstf_MBSSession_StatusSubscribe

Description: The Subscribe service operation is invoked by MBSF, towards the MBSTF, when it needs to create a subscription to monitor at least one event relevant to the delivery session and/or file(s). The MBSF may subscribe to multiple events in a subscription.

Input, Required: MBS Session ID, Event ID(s), notification target address.

Output, Required: When the subscription is accepted: Subscription correlation ID.

#### 8.3.2.6	Nmbstf_MBSSession_StatusUnsubscribe operation

The Unsubscribe service operation is invoked by the MBSF, towards the MBSTF, to remove an existing subscription previously created by itself at the MBSTF.

Service operation name: Nmbstf_MBSSession_StatusUnsubscribe

Description: The Unsubscribe service operation is to remove an existing subscription.

Input, Required: Subscription correlation ID.

Output, Required: Result indication.

#### 8.3.2.7	Nmbstf_MBSSession_StatusNotify operation

The Notify service operation is invoked by the MBSTF to send an event notification towards the notification URI when an event included in the subscription has occurred.

Service operation name: Nmbstf_MBSSession_StatusNotify

Description: This service operation is used by the MBSTF to notify MBSF about the status change of the session or the status of the file.

Input, Required: MBS Session ID, Event ID(s).

Input, Optional: Status information.

Output, Required: Result indication.

**** Last Change ****