Agenda item: 	8.7

Source: 	Tencent Cloud

Title: 	Application context relocation by reference

Document for	Discussion and Agreement

# Introduction

At one of the recent SA4’s MBS ad-hoc calls, the SA6 liaison response was shared that confirmed the following two concepts:

- Each server/or server application that connects to an application client, is an EAS. Therefore, even if a container consists of multiple applications, each is considered a separate EAS.

- The relocation of an application context can only occur through EES, i.e. the location of the storage is transferred from S-EAS to S-EES and then to T-EES and finally to T-EAS (EELManagedACR procedure).

# Currently supported application context transfer

TS23.558 in its EELManagedACR provides the following for transferring the application context from S-EAS to T-EAS (TR23.558 clause 8.8.2.5):

“10.	The Application Context is transferred from S-EAS to the T-EAS at implementation specific time. In the case of EELManagedACR, the S-EES accesses the Application Context from the address as per step 1 and the S-EES and T-EES engage in the ACT from S-EAS to the T-EAS (obtained as per step 5) in a secure way. Further the T-EAS accesses the Application Context made available by the T-EES. If S-EAS performs the ACT directly with T-EAS, the specification of such process is out of scope of the present document.

NOTE 1:	The Application Context is encrypted and protected by the application layer. The S-EES and the T-EES engage in the packet level transport of the Application Context and they have no visibility to the content of the Application Context.”

One option is that while the S-EAS Application Context is accessible by S-EAS from its storage location, the actual transfer of data goes through the following path:

S-EASS-EES T-EEST-EAS

The other alternative is provided is the direct transfer of Application Context from S-EAS to T-EAS but this option is not part of the standard.

# Context transfer by reference use-case

One alternative would be to use the EELManagedACR to exchange the connection information needed for the direct transfer between S-EAS and T-EAS through S-EES and T-EES but leave the actual transfer from S-EAS to T-EAS outside of the scope of the standardization.

The connection information includes attributes that is needed for the direct transfer such as:

- Address of the application context storage

- Unique ID for application context

- Supported protocols for context transfer

- Security information needed

- Availability expiration time for transfer

An example of the call flow for this use case is shown in the following figure:

# Proposal

We propose the following:

- Send a liaison response to SA6, requesting to consider the context transfer by reference use case and addressing it.

- Include the following note in the dCR as the placeholder for further discussion at receiving the response from SA6:

Editor's Note: A liaison has been initiated with SA6 to discuss the possibility of extending the EELManagedACR procedure to support passing a reference to an offer of application context transfer. The parameters of this operation could include a unique transfer offer identifier, the endpoint address of the application context to be relocated, supported transfer protocols, security context, and expiration time of the transfer offer.

Send a liaison response to SA6, request considering the approach proposed in section 2.