3GPP TSG SA WG4 #116e	S4-211604

# E-meeting, 10-19 November, 2021	revision of S4-211530

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | 1 | Current version: | 0.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] HTTP/3 Deployment Architectures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-11-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Describe impact of HTTP/3 usage on deployment architectures |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

### 5.4.3	Deployment Architectures

#### 5.4.3.1	General

Editor’s Note: Clause 6.2.1.2 of TS 26.512 [15] describes the required and supported versions of HTTP for the 5GMS AF and 5GMS AS. For these functions to use HTTP/3, that clause must be updated in a separate contribution, to add HTTP/3 as a supported HTTP version. This clause will assume that HTTP/3 has been added to the same 5GMS functions, in the same way, as HTTP/2.

A minimal deployment architecture that would provide HTTP/3 as transport for downlink and uplink media streaming is shown in figure 5.4.3.1-1.

Figure 5.4.3.1-1: Deployment Architecture for HTTP/3

The addition of HTTP/3 as a supported protocol at reference points M2 and M4 in the 5G media streaming architecture has little effect on the overall architecture. For example, a 5G System using HTTP/3 would resolve Fully-Qualified Domain Names (FQDNs) in the same way as with previous versions of HTTP, as described in Annex B of TS 26.501 [15].

The biggest impact would be on implementations adjusting the endpoints they expose.

To use HTTP/3 for downlink or uplink media streaming, the 5GMS AS would expose HTTP/3 endpoints at reference points M2 and M4 . This is in addition to the HTTP/1.1 and (optionally) HTTP/2 endpoints, as described in clause 6.2.1.2 of TS 26.512 [15].

In the following discussion, the term HTTP/3 client is used for the entity that initiates an HTTP/3 connection, and the term HTTP/3 server is used for the entity that the HTTP/3 client wishes to communicate with.

#### 5.4.3.2	HTTP/3 deployment in downlink media streaming

To use HTTP/3 for pull-based content ingest, the 5GMSd Application Provider exposes an HTTP/3-based origin endpoint to the 5GMSd AS at reference point M2d. This is in addition to the HTTP/1.1 and (optionally) HTTP/2 endpoints, as described in clause 6.2.1.2 of TS 26.512 [15]. In this case, the 5GMS AS acts as an HTTP/3 client, the 5GMSd Application Provider acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2d.

To use HTTP/3 for push-based content ingest, the 5GMSd AS exposes an HTTP/3-based origin endpoint to the 5GMSd Application Provider at reference point M2d. The 5GMSd Application Provider acts as an HTTP/3 client, the 5GMS AS acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2d.

To use HTTP/3 between the 5GMSd Client and the 5GMSd AS at reference point M4d, the 5GMSd Client acts as an HTTP/3 client, and the 5GMS AS acts as an HTTP/3 server, so that HTTP operation would proceeds as usual at M4d.

Because, as described in clause 6.2.1.2 of TS 26.512 [15], the 5GMSd Application Provider may use any supported HTTP protocol version for push-based content ingest at interface M2d, and the Media Stream Handler may use any supported HTTP protocol version at interface M4d, no other changes to the architecture are needed.

#### 5.4.3.3	HTTP/3 deployment in uplink media streaming

In order to use HTTP/3 for uplink media streaming, the 5GMSu AS would expose an HTTP/3-based endpoint to the 5GMSu Client at interface M4u. Here, the 5GMSu Client acts as an HTTP/3 client, the 5GMS AS would act as an HTTP/3 server, and HTTP operation proceeds as usual at M4u.

Separately, the 5GMSu AS may exposes an HTTP/3-based endpoint to the 5GMSu Application Provider at reference point M2u for media egest.

The 5GMS AS here acts as an HTTP/3 client, the 5GMSu Application Provider acts as an HTTP/3 server, and HTTP operation proceeds as usual at M2u.

#### 5.4.3.4	Usage of HTTP/3 at reference point M5

Editor’s Note: This section should be filled in as other key issues (clause 5.3 on traffic identification, clause 5.12 on network slicing) progress.

===== END CHANGES =====