Source:	Ericsson LM

Title:	IVAS-7a - Audio format and designators

Document for:	Discussion and Agreement

Agenda Item:	7.5

# Introduction

As IVAS will provide support for numerous spatial audio formats, there is a need for the processing plan IVAS-7a [1] to define the input/output audio formats and suitable audio format designators.

This contribution provides an initial proposal for such definitions.

# Proposal

It is proposed that IVAS-7a is updated as suggested below. Please note that the proposal is based on some further proposed amendments, all collected in [2], and that numbering is not accurate and would need to be adjusted to current version and further updates based on the agreements in SA4.

The proposal suggests that input/output audio files are sample-wise interleaved 16-bit PCM files.

IVAS is envisioned to support several audio formats which comprise several channels/components, such as multi-channel audio for loudspeaker playback, Ambisonics or audio objects. Here, the term multi-track is used as a general term for all formats which include with more than one audio channel.

The proposal is further based on the draft IVAS Design Constraints (IVAS-4) [3], regarding supported audio formats and ambisonics channel ordering.

# References

- Pdoc IVAS-7a: Processing plan for selection phase, v.0.2.0

- S4-220166: Proposed updates for IVAS-7a, Ericsson LM

- Pdoc IVAS-4: IVAS Design Constraints, v.0.3.0

## Audio format

The format of input/output audio files is headerless PCM with samples stored in 16-bit 2’s complement little endian format. For multi-track audio, the audio tracks are sample-wise interleaved in the order specified by the corresponding configuration in Table 5.

## Audio track designators

Audio tracks are designated according to Table 3 with further specification according to Table 4.

Table 3: Track designators

| Track designator | Definition |
| --- | --- |
| L | Left channel |
| R | Right channel |
| M_<a> | Multi-channel middle layer at nominal azimuth direction <a> |
| U<e>_<a> | Multi-channel upper layer at nominal elevation direction <e> and nominal azimuth direction <a> |
| A_<n> | Ambisonics components with ACNindex <n> |
| O_<n> | Object ID <n> |

Table 4: Track specification

| Specification variable | Definition |
| --- | --- |
| <a> | Nominal azimuth direction (degrees): Left: L000-L180 Right: R000-R180 |
| <e> | Nominal elevation direction (degrees): Up: U00-U90 Down: D00-D90 |
| <n> | Index: 0, 1, … |

Editor’s Note: There are various conventions for naming of channels in multi-channel configurations. These conventions are typically associated with tolerances of loudspeaker positions. The notation proposed here is more explicit using nominal directions.

## Audio track configurations

Input/output audio shall follow configurations as specified in Table 5. Ambisonics components follow the ACN ordering where ACNindex = n2+n+m for real-valued spherical harmonics components of order n and degree m, where m=[-n,…,n].

Table 5: Audio track configurations

| Audio format | Number of tracks | Configuration (incl. ordering) |
| --- | --- | --- |
| Stereo/Binaural | 2 | L, R |
| Multi-channel 5.1 | 6 | M_L030, M_R030, …, TBD |
| Multi-channel 7.1 | 8 | TBD |
| Multi-channel 5.1+4 | 10 | TBD |
| Multi-channel 7.1+4 | 12 | TBD |
| FOA | 4 | A_0, A_1, A_2, A_3 |
| HOA<AO*> | AO+12 | A_0, A_1, …, A_<AO+12-1> |
| [Mono objects] | 1, …, TBD | O_0, …, O_<TBD> |

* AO = Ambisonics order