3GPP TSG SA WG4#116e	S4-220037

E-meeting, 14th – 23rd February 2022	revision of S4aV220862

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | draft | rev | 8 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 8K TV Scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Fraunhofer HHI, Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 8K_TV_5G |  |  |  |  |  | Date: |  |  | 04/02/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | See work item |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provides a new scenario for TR26.955 to document characterization of 8K TV. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item objectives not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2, 6.7 (new), Annex C.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The sequences are currently uploaded. Should take roughly one more week to complete |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aV220862 was agreed as the basis for future work. This version only removes any highlights and fixes references and cleans. Also the attachments are provided. Still missing for completion are the anchors and results. |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[X]	8K Association Resources, https://8kassociation.com/resources/

[Y]	Thierry Fautier, "New codecs,”, DASH-IF Workshop on Media Streaming and 5G, December 2019, accessible https://dashif.org/docs/workshop-2019/04-thierry%20fautier%20-%20Harmonic%20Codec%20Comparison%205G%20Media%20Workshop_Final%20v3.pdf

=====  CHANGE  =====

## 4.2	TV Video Profiles

The TV Video Profiles in TS 26.116 [3] address coded representations of TV distribution signals up to UHD-1 phase 2. Table 4.2-1 provides an overview of the TV relevant formats considered in the context of 3GPP TV Video Profiles.

In the context of TV Video Profiles, the following aspect are defined:

-	Bitstream: A media bitstream that conforms to a video encoding format and certain Operation Point.

-	Operation Point: A collection of discrete combinations of different content formats including spatial and temporal resolutions, colour mapping, transfer functions, etc. and the encoding format.

-	Receiver: A receiver that can decode and render any bitstream that is conforming to a certain Operation Point.

Table 4.2-1: TV over 3GPP services Video Profile Operation Points (TS 26.116 [3])

| Operation Point name | Resolution format | Picture aspect ratio | Scan | Max. frame rate | Chroma format | Chroma sub-sampling | Bit depth | Colour space format | Transfer Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.265/HEVC 720p HD | 1280 × 720 | 16:9 | Progressive | 30 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.264/AVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8 | BT.709 | BT.709 |
| H.265/HEVC Full HD | 1920 × 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 8; 10 | BT.709; BT.2020 | BT.709; BT.2020 |
| H.265/HEVC UHD | 3840 × 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2020 |
| H.265/HEVC Full HD HDR | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 PQ |
| H.265/HEVC UHD HDR | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 PQ |
| H.265/HEVC Full HD HDR HLG | 1920 x 1080 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 HLG |
| H.265/HEVC UHD HDR HLG | 3840 x 2160 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2100 HLG |
| H.265/HEVC 8K UHD | 7680 x 4320 | 16:9 | Progressive | 60 | Y'CbCr | 4:2:0 | 10 | BT.2020 | BT.2020 BT.2100 PQ BT.2100 HLG |

Each one of the Operation Points is associated with a video coding specification and a particular Profile, Level, and Tier (for HEVC). The combination of a specific profile, level, and tier indicate the maximum decoding capabilities, such as chroma format, resolution, frame rate, and bit depth,  that can be supported by a decoder conforming to such combination of parameters. An operation point is also associated with additional bitstream constraints defined in TS 26.116 [3], clause 4. Table 4.2-2 summarizes the video coding specification profile, tier, and level associated with each operation point.

Table 4.2-2: Operation point video codec Profile/Tier/Level

| Operation Point name | Video Codec | Profile | Tier | Level |
| --- | --- | --- | --- | --- |
| H.264/AVC 720p HD | AVC | High | - | 3.1 |
| H.265/HEVC 720p HD | HEVC | Main | Main | 3.1 |
| H.264/AVC Full HD | AVC | High | - | 4.2 |
| H.265/HEVC Full HD | HEVC | Main 10 | Main | 4.1 |
| H.265/HEVC UHD | HEVC | Main 10 | Main | 5.1 |
| H.265/HEVC Full HD HDR | HEVC | Main 10 | Main | 4.1 |
| H.265/HEVC UHD HDR | HEVC | Main 10 | Main | 5.1 |
| H.265/HEVC Full HD HDR HLG | HEVC | Main 10 | Main | 4.1 |
| H.265/HEVC UHD HDR HLG | HEVC | Main 10 | Main | 5.1 |
| H.265/HEVC 8K UHD | HEVC | Main 10 | Main | 6.1 |

For TV Video profiles, interoperability with ISO BMFF based systems and the DASH Streaming is of most relevance. Hence, for a codec to be used in the context of TV Video Profiles, the following is defined in terms of interoperability:

- The receiver requirements on elementary stream level

- The encapsulation of an elementary stream into an ISO Base Media File Format track

- The provisioning of the media as part of the DASH Adaptation Set to support seamless switching

- All MPD-level signalling for the codec to support capability discovery

For details, refer to TS 26.116 [3], clause 4 and clause 5.

=====  CHANGE  =====

## 6.7	Scenario 2: 8K-TV

### 6.7.1	Motivation

Since the initial development and last update of TV Video Profiles defined in 3GPP TS 26.116, TV and mobile device capabilities have improved and nowadays they support higher decoding capabilities. In particular, new TV sets and 5G mobile devices entering the market since 2020 support up to 8K video decoding as well as 8K display capabilities.

8K is recently trialed and introduced in several services. In addition, other ecosystem support is happening, such as 8K encoders are announced, 8K TV sets are shipped and content is produced in 8K. Some good information, background and motivation is for example collected by the 8K Association [X] . Furthermore, it is evident that distribution of 8K TV content is feasible with 5G. As an example, it was shown that with H.265/HEVC around 40 MBit/s on sports content can be achieved [Y]. Most of the existing 8K services are build on H.265/HEVC.

### 6.7.2	Description of the Anticipated Application

In the context of 3GPP services, 5G Media Streaming [13] as well as the TV Video Profiles [3] are specifications addressing this 8K-TV scenario. Both, 5G Media Streaming [13] and TV Video Profiles [3] build on CMAF-based Segment formats and DASH distribution. From 3GPP TS 26.116, the following operation point may be considered in scope of the 8K-TV Streaming Scenario (pending availability of appropriate test content):

-	H.265/HEVC 8K, see 3GPP TS 26.116 [3] clause 4.5.9.

Important aspects that are expected to be considered when evaluating a codec in the context of this 8K-TV scenario are similar to those of 4K-TV as defined in clause 6.3.2.

### 6.7.3	Source Format Properties

Table 6.7.3-1 provides an overview of the different source signal properties for 8K-TV. This information is used to select proper test sequences.

Table 6.7.3-1 8K-TV source format properties

| Source format properties | 8K-TV |
| --- | --- |
| Spatial resolution | 7680 × 4320 |
| Chroma format | Y’CbCr |
| Chroma subsampling | 4:2:0 |
| Picture aspec ratio | 16:9 |
| Frame rates | 24; 50; 60; 24/1.001; 60/1.001; [100; 120] Hz |
| Bit depth | 10 |
| Colour space formats | BT.2020 [15] |
| Transfer characteristics | BT.2020 [15], BT.2100 [16] (PQ & HLG) |
| Range | Standard range |

### 6.7.4	Encoding and Decoding Constraints

Table 6.7.4-1 provides an overview of encoding and decoding constraints for 8K-TV category using the H.265/HEVC codec. This will support the definition of detailed test conditions.

Table 6.7.4-1 Encoding and Decoding Configurations for 8K-TV with HEVC codec

| Encoding and Decoding Constraints | H.265/HEVC 8K-TV |
| --- | --- |
| Relevant Codec and Codec Profile/Levels according to TS26.116 and TS26.511. | H.265/HEVC Main 10 Profile Level 6.1 [8] |
| RAP period | Unconstrained, 1 sec |
| Bitrate parameters (CBR, VBR, CAE, HRD parameters) | QP = [22,27,32,37] VBR |
| Latency requirements and specific encoding settings | No latency requirements beyond RAP so picture reordering allowed |
| Encoding complexity context | real-time encoding, cloud-based encoding, offline encoding, etc. |
| Required decoding capabilities | H.265/HEVC Main 10 Profile Level 6.1 [8], and support for BT.2020 and BT.2100 |

### 6.7.5	Performance Metrics

Performance is assessed using BD-Rate computation, with PSNR, SSIM and VMAF metrics as objective quality criterion. Regarding complexity considerations, encoding/decoding runtime is provided.

All metrics as defined in clause 5.5 are expected to be reported.

For BD-Rate computation and SDR, only the following metrics are expected to be provided:

-	Peak-Signal to Noise Ratio PSNR(Y) of luma component as defined in clause 5.5.3,

-	Average colour component PSNR, PSNR over all colour components PSNR(i) as defined in clause 5.5.3,

-	Structural similarity metric MS-SSIM asdefined in clause 5.5.4.2,

-	Video Multimethod Assessment Fusion (VMAF) VMAF as defined in clause 5.5.3.

For BD-Rate computation and HDR, only the following metrics are expected to be provided:

-	the weighted Peak-Signal To Noise Ratio metric for the luminance colour components wPSNR(Y), as specified in clause 5.5.4 and 5.5.8,

-	Average colour component weighted PSNR, wPSNR over all colour components wPSNR(i), as specified in clause 5.5.4 and 5.5.8,

-	the PSNRL100 metric as specified in clause 5.5.4 and 5.5.8,

-	DE100 metric as specified in clause 5.5.4 and 5.5.8.

### 6.7.6	Interoperability Considerations

In order to use a codec in the context of 5G Media Streaming services in 3GPP TS 26.511 [13] and for TV Video profiles in 3GPP TS 26.116 [3], the same considerations for interoperability as for FullHD according to clause 6.2.6 apply.

For additional details, please refer to 3GPP TS 26.116 [3] and 3GPP TS 26.511 [13].

### 6.7.7	Reference Sequences

Table 6.7.7-1 provides the selected reference sequences for this scenario for SDR and Table 6.7.7-2 provides the selected reference sequences for this scenario for HDR. Keys are defined to refer to the sequences in the context of the scenario. The sequences are named and a reference to the details of the sequence is provided. Annex C.7 describes in detail the selection process conducted to build the test sequences considered for this scenario, the outcome of this process is reported in the table below.

Table 6.7.7-1 SDR Reference Sequences for 8K-TV scenario

| Key | Name | Reference | Resolution | Frame rate | Colour Gamut | Number of Frames | Scene Cut |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S6-R01 | BodeMuseum | C.7.1.1 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R02 | NeptuneFountain2 | C.7.1.2 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R03 | NeptuneFountain3 | C.7.1.3 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R04 | OberbaumSpree | C.7.1.4 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R05 | QuadrigaTree | C.7.1.5 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R06 | SubwayTree | C.7.1.6 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |
| S6-R07 | TiergartenParkway | C.7.1.7 | 7680 × 4320 | 60 | BT.2020 | 600 | 0 |

Table 6.7.7-2 HDR Reference Sequences for 8K-TV scenario

| Key | Name | Reference | Resolution | Frame rate | Colour Gamut | Number of Frames | Scene Cut |
| --- | --- | --- | --- | --- | --- | --- | --- |
| S6-R11 | BodeMuseum-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R12 | NeptuneFountain2-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R13 | NeptuneFountain3-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R14 | OberbaumSpree-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R15 | QuadrigaTree-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R16 | SubwayTree-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |
| S6-R17 | TiergartenParkway-PQ | C.7.2.1 | 7680 × 4320 | 60 | BT.2100 PQ | 600 | 0 |

### 6.7.8	Anchor Definition

#### 6.7.8.1	Overview

This clause provides details on how to generate the anchors for the 8K-TV scenario.

No H.264/AVC Anchors are defined.

H.265/HEVC Anchors are defined in clause 6.7.8.3.

#### 6.7.8.2	H.264/AVC Anchors

No H.264/AVC Anchors are defined for this scenario.

#### 6.7.8.3	H.265/HEVC Anchors

##### 6.7.8.3.1	Overview

Table 6.7.8.3.1-1 provides an overview of the H.265/HEVC anchor tuples. Keys are identified to refer to the anchors in the context of the scenario.

The details are also provided here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-6-8K/265/streams.csv.

Table 6.7.8.3.1-1 Anchor Tuple generation with H.265/HEVC for 8K-TV Scenario

| Key | Clause | Reference Sequence | Reference Encoder | Configuration | Variations | Anchor Key |
| --- | --- | --- | --- | --- | --- | --- |
| S6-A01-265 | 6.7.8.3.3 | S6-R01 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A1-265-<QP> |
| S6-A02-265 | 6.7.8.3.3 | S6-R02 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A2-265-<QP> |
| S6-A03-265 | 6.7.8.3.3 | S6-R03 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A3-265-<QP> |
| S6-A04-265 | 6.7.8.3.3 | S6-R04 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A4-265-<QP> |
| S6-A05-265 | 6.7.8.3.3 | S6-R05 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A5-265-<QP> |
| S6-A06-265 | 6.7.8.3.3 | S6-R06 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A6-265-<QP> |
| S6-A07-265 | 6.7.8.3.3 | S6-R07 | HM16.24 | S6-HM-01 | QP: [22,27,32,37] | S6-A7-265-<QP> |
| S6-A11-265 | 6.7.8.3.4 | S6-R11 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A11-265-<QP> |
| S6-A12-265 | 6.7.8.3.4 | S6-R12 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A12-265-<QP> |
| S6-A13-265 | 6.7.8.3.4 | S6-R13 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A13-265-<QP> |
| S6-A14-265 | 6.7.8.3.4 | S6-R14 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A14-265-<QP> |
| S6-A15-265 | 6.7.8.3.4 | S6-R15 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A15-265-<QP> |
| S6-A16-265 | 6.7.8.3.4 | S6-R16 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A16-265-<QP> |
| S6-A17-265 | 6.7.8.3.4 | S6-R17 | HM16.24 | S6-HM-02 | QP: [22,27,32,37] | S6-A17-265-<QP> |

##### 6.7.8.3.2	Common Parameters and Settings

To generate the anchor bitstreams, HM16.24 is used:

-	HM16.24 https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.24/

The common parameters are as follows:

- Profile: main10 (Main 10 Profile)

- DecodingRefreshType: 1 (CRA)

- SearchRange: 384

- InternalBitDepth: 10 (codec operating bit-depth where all sequences (including 8 bit sequences) are coded with an internal bitdeph of 10 in accordance with [44] and metrics are calculated in 10 bits).

- SEIMasteringDisplayColourVolumeSEI is not added. If it would be added, then the metadata in the json file may be used.

The following parameters need to be adapted for each sequence as follows using the JSON parameters of the reference sequence:

-	 IntraPeriod: Intra Period aligned with GOPSize such that approximately 1 second is achieved, i.e.

-	"frameRate": 23.98 or 24.0 or 25 or 30 => IntraPeriod set to 32,

-	"frameRate": 50.0 or 59.94 or 60 => IntraPeriod set to 64.

The following parameters are variables and triggered through updates of the config-file.

- QP: [22,27,32,37].

In cases where the anchor uses temporal filtering and the codec being tested does not, additional results may be included for information to show the comparison with temporal filtering turned off for the anchor. Alternatively, an external document, which details the improvement due to temporal filtering for the anchor, may be referenced.

##### 6.7.8.3.3	S6-HM-01: SDR Settings

The common parameters as defined in 6.7.8.3.2 apply.

In addition, the following parameters apply:

In addition, the following parameters apply:

- VuiParametersPresent: 1 (VUI present)

- ColourPrimaries: 9

- TransferCharacteristics: 14

- MatrixCoefficients: 9

- ChromaLocInfoPresent: 1

- ChromaSampleLocTypeTopField: 2

- ChromaSampleLocTypeBottomField: 2

- SEIDecodedPictureHash: 0 (md5 checksum absent)

The settings are defined in the attached configuration file s6-hm-01.cfg.

##### 6.7.8.3.4	S6-HM-02: HDR PQ Settings

The common parameters as defined in 6.7.8.3.2 apply.

In addition, the following parameters apply:

- VuiParametersPresent: 1 (VUI present)

- ColourPrimaries: 9

- TransferCharacteristics: 16

- MatrixCoefficients: 9

- ChromaLocInfoPresent: 1

- ChromaSampleLocTypeTopField: 2

- ChromaSampleLocTypeBottomField: 2

- SEIDecodedPictureHash: 0 (md5 checksum absent)

The settings are defined in the attached configuration file s6-hm-02.cfg.

### 6.7.9	Anchor Results

#### 6.7.9.1	H.264/AVC Anchors

No H.264/AVC anchors are provided.

#### 6.7.9.2	H.265/HEVC Anchors

HEVC anchor streams are provided according to the key system here:

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-6-8K/265/

HEVC anchor results are provided with the appropriate keys as defined in Table 6.7.8.3.1-1

- in the attached csv files

- https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/ Scenario-6-8K/265/Metrics/

Editor’s Note:

- Results for H.265/HEVC are still missing.

### 6.7.10	Additional Information and Performance Data

Tbd (with references).

=====  CHANGE  =====

7.3.2	Characterization against H.265/HEVC HM

For a codec to be fully characterized against H.265/HEVC HM, the following information should be provided:

1.	For scenario 1:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S1-HM-01 as defined in clause 6.2.8.3 with a comparable configuration and all SDR metrics defined clause 6.2.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S1-HM-02 as defined in clause 6.2.8.3 with a comparable configuration and all HDR metrics defined clause 6.2.5.

2.	For scenario 2:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S2-HM-01 as defined in clause 6.3.8.3 with a comparable configuration and all SDR metrics defined clause 6.3.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S2-HM-02 as defined in clause 6.3.8.3 with a comparable configuration and all HDR metrics defined clause 6.3.5.

3.	For scenario 3:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S3-HM-01 as defined in clause 6.4.8.3 with a comparable configuration and all SDR metrics defined clause 6.4.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S3-HM-02 as defined in clause 6.4.8.3 with a comparable configuration and all SDR metrics defined clause 6.4.5.

c)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S3-HM-03 as defined in clause 6.4.8.3 with a comparable configuration and all SDR metrics defined clause 6.4.5.

d)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S3-HM-04 as defined in clause 6.4.8.3 with a comparable configuration and all SDR metrics defined clause 6.4.5.

4.	For scenario 4:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S4-HM-01 as defined in clause 6.5.8.3 with a comparable configuration and all SDR metrics defined clause 6.5.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S4-HM-02 as defined in clause 6.5.8.3 with a comparable configuration and all SDR metrics defined clause 6.5.5.

5.	For scenario 5:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S5-HM-01 as defined in clause 6.6.8.3 with a comparable configuration and all SDR metrics defined clause 6.6.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S5-HM-02 as defined in clause 6.6.8.3 with a comparable configuration and all SDR metrics defined clause 6.6.5.

2.	For scenario 6:

a)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S6-HM-01 as defined in clause 6.7.8.3 with a comparable configuration and all SDR metrics defined clause 6.7.5.

b)	BD-Rate Gain table as defined in clause 5.7 for all H.265/HEVC reference sequences and configuration S6-HM-02 as defined in clause 6.7.8.3 with a comparable configuration and all HDR metrics defined clause 6.7.5.

Additional information such as Rate-Quality plots may be provided if considered insightful.

Partial characterization may be conducted as well.

=====  CHANGE  =====

# C.7	8K-TV Sequences

## C.7.1 	SDR Category

#### C.7.1.1	Bode Museum

The sequence provides a view of the Bode Museum in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Table C.7.1.1-1 HHI Generation Parameters

| Camera: | RED DSMC2 |
| --- | --- |
| Sensor: | Helium 8K S35 35.4 Megapixel CMOS Sensor 29.90 mm x 15.77 mm (Diagonal: 33.80 mm) |
| Lense: | Fujinon XK6x20 (20-120mm) |
| Recording resolution: | 7680x4320 @ 60fps |
| Recording format: | R3D raw (REDCODE 12:1) with RED Log3G10 log curve and REDWideGamutRGB color space |

Figure C.7.1.1-1 shows a screenshot of the Bode Museum test sequence.

Figure C.7.1.1-1: Screenshot of Bode Museum test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Bode Museum test sequence.

Table C.7.1.1-2 HHI Sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 7680 x 4320 |
| Scan | progressive |
| Frame Rate | 60 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT 2020 |
| Colour space | SDR |

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/BodeMuseum/BodeMuseum.json

The Bode Museum test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "BodeMuseum"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.2	Neptune Fountain 2

The sequence provides a view of the Neptune Fountain in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.2-1 shows a screenshot of the Neptune Fountain 2 test sequence.

Figure C.7.1.2-1: Screenshot of Neptune Fountain 2 test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Neptune Fountain 2 test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/NeptuneFountain2/ NeptuneFountain2.json

The Neptune Fountain 2 test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "NeptuneFountain2"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.3	Neptune Fountain 3

The sequence provides a different view of the Neptune Fountain in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.3-1 shows a screenshot of the Neptune Fountain 3 test sequence.

Figure C.7.1.3-1: Screenshot of Neptune Fountain 3 test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Neptune Fountain 3 test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/NeptuneFountain3/ NeptuneFountain3.json

The Neptune Fountain 3 test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "NeptuneFountain3"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.4	Oberbaum Spree

The sequence provides a view of the Oberbaum Spree in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.4-1 shows a screenshot of the Oberbaum Spree test sequence.

Figure C.7.1.4-1: Screenshot of Oberbaum Spree test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Oberbaum Spree test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/OberbaumSpree/ OberbaumSpree.json

The Oberbaum Spree test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "OberbaumSpree"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.5	Quadriga Tree

The sequence provides a view of the Quadriga Tree in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.5-1 shows a screenshot of the Quadriga Tree test sequence.

Figure C.7.1.5-1: Screenshot of Quadriga Tree test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Quadriga Tree test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/QuadrigaTree/ QuadrigaTree.json

The Quadriga Tree test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "QuadrigaTree"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.6	Subway Tree

The sequence provides a view of a Subway Tree in Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.6-1 shows a screenshot of the Subway Tree test sequence.

Figure C.7.1.6-1: Screenshot of Subway Tree test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Subway Tree test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/SubwayTree/ SubwayTree.json

The Subway Tree test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "SubwayTree"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

#### C.7.1.7	Tiergarten Parkway

The sequence provides a view of a parkway in the Tiergarten Berlin. The content generation parameters are provided in Table C.7.1.1-1.

Figure C.7.1.7-1 shows a screenshot of the Tiergarten Parkway test sequence.

Figure C.7.1.7-1: Screenshot of Tiergarten Parkway test sequence

The HHI test sequence properties as provided in Table C.7.1.1-2 apply to the Tiergarten Parkway test sequence.

The sequence can be accessed at

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TiergartenParkway/ TiergartenParkway.json

The Tiergarten Parkway test sequence is made available under the following copyright disclaimer.

Digital Video Sequence "TiergartenParkway"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

7680x4320p, YCbCr 4:2:0, BT.2020, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/

## C.7.2 	HDR Category

#### C.7.2.1	HHI Sequences converted by Interdigital

All 7 HHI SDR 8K sequences in clause C.7.1 were regenerated in PQ version by the following process:

- Use the same raw camera material as SDR sequences

- Tune grading setting per sequence (color, exposure, tone mapping)

- Use the raw development tool provided by camera manufacturer, using above-mentionned settings and PQ transfer curve, generating 16-bit RGB .tiff output

- Convert to YCbCr 4:2:0 10-bit using ffmpeg and BT.2020 matrix, limited range, with top-left chroma location

- Overlay the HHI logo, previously converted to YCbCr 4:2:0, BT.2020, PQ, at 400 nits

The test sequence properties as provided in Table C.7.2.1-2 apply to all HHI/Interdigital PQ test sequences.

Table C.7.2.1-1 HHI/Interdigital PQ Sequence properties

| Parameter | Value |
| --- | --- |
| Resolution | 7680 x 4320 |
| Scan | progressive |
| Frame Rate | 60 |
| Bit Depth | 10 |
| Length | 600 frames |
| YUV format | 4:2:0 |
| Colour components | ITU-T BT 2100 PQ |
| Colour space | HDR |

Based on this, the following HDR sequences are provided:

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/BodeMuseum/BodeMuseum_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/NeptuneFountain2/ NeptuneFountain2_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/NeptuneFountain3/ NeptuneFountain3_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/OberbaumSpree/ OberbaumSpree_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/QuadrigaTree/ QuadrigaTree_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/SubwayTree/ SubwayTree_PQ.json

-	https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/ReferenceSequences/TiergartenParkway/ TiergartenParkway_PQ.json

The HDR sequences test sequences are made available under the following copyright disclaimer.

Digital Video Sequence "<name>"

Copyright (C) 2019 Fraunhofer HHI

Author: Björn Kowalewsky

HDR grading by Philippe de Lagrange (C) 2022 Interdigital R&D France

7680x4320p, YCbCr 4:2:0, BT.2100 PQ, 10 bit, narrow range, 60 fps, 600frames.

COPYRIGHT AND LICENSE INFORMATION

November, 2021

Fraunhofer Heinrich Hertz Institute (HHI)

Video Coding & Analytics Department

Einsteinufer 37

10587 Berlin, Germany

The video sequences provided above and all intellectual property rights therein remain the property of Fraunhofer HHI. This video sequence is licensed under the Creative Commons Attribution-NonCommercial-NoDerivatives 4.0 International License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nc-nd/4.0/