3GPP TSG SA WG4#119e	S4-220610

E-meeting, 11th – 20th May 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | pseudo | rev | - | Current version: | 1.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5G_Video] Initial Conclusions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated (as Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 04/05/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 9	Gaps and Optimization Potential

## 9.1	Identified Gaps and Deficiencies with Existing Codecs

Based on the scenarios that were defined in this Technical Report, the existing 3GPP codecs H.264/AVC and H.265/HEVC had been benchmarked and evaluated.

H.264/AVC is still applicable and widely used in services and due to its install base and ubiquity remains a fallback solution for these basic use cases. However, H.264/AVC clearly lacks compression efficiency and flexibility to address more advanced use cases, such as HDR, gaming sequences and screen content sharing.

H.265/HEVC provides, at least for the considered scenarios, a full feature set and is broadly and versatily applicable. As the codec is part of 3GPP specifications since quite some time, its install base has matured and is almost ubiquituous. Based on this, a successor of H.265/HEVC would be interesting if it provides at least the same flexibility with improved compression efficiency.

The performance of H.265/HEVC for higher spatial resolutions, such as 8K, has not been evaluated.

## It is encouraged to study more details of gaps and deficiencies of existing codecs.

## 9.2	Potential Requirements for New Codecs

From the collected scenarios, no explicit new requirements for new codecs have been identified.

However, flexibility to different applications and compression efficiency are they key functionalities for a codec in 3GPP. A new codec is expected to differentiate from H.265/HEVC in at least one, preferably several dimensions.

It is encouraged to study more details of potential requirements for new codecs.

# 10	Conclusions and Proposed Next Steps

The Technical Report provides a full characterization framework for video codecs in the context of 5G services. This framework permits to evaluate the performance of existing codecs, and also permits to identify benefits of potential new codecs. The framework fulfils the following aspects

<summarize what we have, scenarios, sequences, anchors, metrics>

The framework provides a comprehensive set of scenarios.

The framework clearly has deficiencies, for example encoder configurations for scenarios may have not been stringent enough in the definition, leading to results that may not be fully comparable. Results in this document should always be taken with care, and the reader should understand how these results were derived. The framework does not include subjective evaluation, it is purely based on objective metrics.

Results are complete for 3GPP codecs H.264/AVC and H.265/HEVC for all scenarios.

For new codecs, only initial results are provided and need to be taken with care, as for example the encoders used have different maturity and features.

The framework and the initial results are not mature enough to come up with concrete proposed next steps, for example in terms of benefits of one or multiple new codecs in 3GPP services. H.265/HEVC is prominent and addresses many use cases. It is recommended to use these initial results and the framework in potential follow-up studies. Such potential studies may for example include:

- A refinement and pruning of the framework for codec evaluation

- A more detailed study on potential deficiencies of existing codecs

- Potential requirements for new codecs in terms of efficiency, functionality and interoperability

- An evaluation of benefits and issues of introducing one or several new codecs for 3GPP services

# A potential timeline for the needs of new codecs