# 3GPP TSG-WG SA4 Meeting #119	S4-220689

# Online,  11th to 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.805 | CR |  | rev | - | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_NPN4AVProd]: Proposal of a study conclusion clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NPN4AVProd |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Study on Media Production via 5G NPN is finalizing. This document proposes a conclusion, some recommendations for further work and some suggestions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The conclusion section is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 7	Conclusions, Recommendations and Suggestions

## 7.1	General

The technical report contains an elaborative description on different protocols and workflows, used in today’s media production. A number of different standardization and industry foras are defining Ethernet and IP based protocols for different media production purposes. The study has not identified a technical area. It is therefore recommended to not start normative work within 3GPP SA4 at this time. Instead, a number of consideration suggestions are identified, which are intended to support media production device manufactures and media producers to leverage different 3GPP features foir their purposes.

The capabilities of media production devices, more specifically video camera, are studied extensively. One observation is that the 5G Media Uplink Streaming Architecture is relevant, but typically deployed together with 5G Media Downlink Streaming components for example to support return video, etc. It seems unlikely, that the 5G Media Uplink Streaming Architecture is deployed alone, i.e., without any downlink streaming architecture component.

Media production devices like cameras include support for speech for Intercom / Push to Talk use-cases. 3GPP defined solutions might be relevant, but this has not been studied in detail.

Editor’s Note:

-	Multicamera production solutions. E.g. adding a use-case example to the Program / Standby Cameras KI.

-	AES 67 redundancy removale

## 7.3	Suggestions to Device Manufactures and Media Producers

-	Avoid multiplexing of different media component: 3GPP network support different traffic priorization schemes. Key Issue #2 discusses, how to use 3GPP QOS and Network Slicing for prioritizing different media components of a production device. It is suggested to separate media componens into separate IP / UDP flows for separate prioritization.

-	It is generally suggested to use e.g. a native RTP payload format such as defined in RFC7798 (HEVC payload format)

-	When MPEG2-TS is used, then the following suggestions should be considered

-	Extra jitter buffer for PCR timeline: 3GPP Networks change the radio MCS depending on the current channel conditions of a device, resulting in a varying bitrate. MPEG2-TS is designed for a constant bandwidth channel and extra care needs to be taken to maintain the PCR timeline.

-	Null packets: It is a common practize to add null packets (padding to make the stream a constant bitrate stream) into the MPEG2-TS stream in order to create a stream for a constant bandwidth channel. Null packets are ignored by the receiver. In a 5G System, these null packets take away capacity for lower priority traffic and is unnecessarily introducing radio interference. It is suggested to use a setup which does not insert null packets

- 	For higher tier productions, there is a strong desire to use a constant quality stream, i.e. no dynamic bitrate adaptation applied. In order to secure a constant quality signal, it is recommended to use a GBR type QoS flow and to provision the system in such a way, that the bitrate can be provided in all cases. That also means, that a very strict admission control should be applied.

-	For lower tier productions or when admission control cannot be very strict, it is recommended to employ a bitrate adaptation scheme (see Key Issue #5) to prevent congestion related packet losses.

-	For remote production scenarios, it is suggested to QoS provision the full end to end path and to account for a bitrate adaptation scheme and packet loss recovery.

-	Wireless networks are often shielded toward public networks using a Firewall. Usage of a NAT introduces independence around the IP address allocation withing the 5G network. Media production devices are often remotely configured and controlled, so that the user of the 5G Device (e.g. cameraman) can concetrate on the creative capturing. Specifically, in remote production scenarios, it is recommended to use client side initiated protocols for remote configuration and control, such as MQTT in order to mitigate issues with NATs and Firewalls.

-	Time Synchronization: Media Production often needs precise time synchronization of wireless devices. SMPTE 2059-2 (PTP provide for Broadcast) is supported by 23.501 from Release 17 onwards.

-	Device On-Boarding, SA2 NPN / eNPN study.

- 	Audio legacy systems may not be connected to the 5G-system directly. An adapter function is needed to convert from circuit switching to packet based audio transmission. Different state-of-the-art adapters from professional audio manufactors exist. The usage of packet based audio transmission is preferred. The 5G System acts as a transparent transport layer. Audio over IP (AoIP) solutions are supported natively when the 5G System is configured and optimized for the traffic specifica. For compatibility it is suggested to use only one AoIP impementation with a defined parameter set. Mixing different parameter sets and AoIP standards may introduce unknown side effects. Details of implementation depend on device manufactor and system provider.

- 	….

**** Next Change ****

# Annex X: Summary of 5G-MAG workshops (information)

## X1. General

5G-MAG organized several workshops to reach out various media producers and device manufactures wrt 5G Network usages. This annex contains a summary of the workshops, including links to the 5G-MAG site.

## X.2	First 5G-MAG Workshop (21st April 2021)

The first 5G MAG workshop around Media Production over 5G Non-Public Networks aimedto gathering input from different stakeholders in the media and ICT industries around the objectives of the new study in 3GPP.

Three sessions tackled different aspects of Media Production over 5G NPNs, with the final one organized as an interactive exchange. The main objectives of the workshop are:

1.	To collect input on the objectives of the Study Item from the media vertical around relevant media production use cases, media device and network orchestration solutions and other media-related aspects of Media Production use cases.

2.	To introduce relevant 5G System features coming from 3GPP and collect input around e.g. NPNs, Network Slicing, QoS classes, network event reporting and assistance, etc. that may be useful for media production.

3.	To facilitate collaboration and discussion around reference architectures for media production (including media and control flows), relevant QoS requirements and KPIs, protocols, codecs and service layers, etc.

Detailed agenda, presentations and recordings of the session can be found here: https://www.5g-mag.com/post/5g-mag-workshop-media-production-over-5g-npn

# X.3	Preparation session for a second 5G-MAG Workshop (15th of December 2021)

The 5G Media Action Group (5G-MAG) has organizing the workshop “Media Production and 5G Non-Public Networks: Deep dive into media production protocols” on 15th of December.

This interactive workshop aimed at gathering input from different stakeholders in the media and ICT industries around the work in the 3GPP SA4 study. In particular, we were interested in discussing how the diverse set of media production and transport protocols that may have to interface and be supported by the 5G System.

Agenda

- Welcome Ian Wagdin (CP-C Chair, BBC)

- The technical activities in 5G-MAG Thibaud Biatek (CP-T Chair, ATEME)

- Technical Report on Media Production and 5G NPNs Thorsten Lohmar (3GPP SA4 Study Item NPN4AVPROD Rapporteur, Ericsson)

- QoS and Prioritization David Butler, Ivan Hassan (BBC)

Linked to this workshop, 5G-MAG members prepared a questionnaire for attendees in order to get their view around several aspects on the workflows supporting wireless connectivity. The outcome is presented in Section XX.

Recordings and presentations are available at https://www.5g-mag.com/post/5g-mag-workshop-media-production-over-5g-npn-deep-dive-into-protocols

# X.3	Second 5G-MAG Workshop (19th of January 2022)

5G-MAG requested input from media companies in order to present their workflows and relevant trials linked to the use of wireless connections.

A total of 8 requests for presentations were received as follows:

| Vislink | David Edwards | Download Presentation Slides |
| --- | --- | --- |
| NuLink | Laurent Zwahlen | Download Presentation Slides |
| Qualcomm | Yiqing Cao | Download Presentation Slides |
| NRK | Erik Vold | Download Presentation Slides |
| France TV | Jacques Donat-Bouillud and Samy Nicolas Bouchalat | Download Presentation Slides |
| Ross Video | Tom Crocker | Download Presentation Slides |
| Agile Content | Johan Bilon | Download Presentation Slides |
| BBC | Sam Hurst | Download Presentation Slides |

Recordings and presentations are available at https://www.5g-mag.com/post/follow-up-workshop-media-production-over-5g-npn-deep-dive-into-protocols

# X.3	Summary of the Workshops

Summary of the Presentations:

- Vislink highlighted the main critical advantages of an SNPN deployment: security, constant bandwidth availability, predictable latency, in general non-contention with other users. In contrast, adaptive bitrate is currently used in cellular bonding scenarios to when non-contention cannot be guaranteed. Vislink provided a proof-of-concept to compare 5G and OFDM finding similar latencies with a small (but acceptable) difference between the two. The latency of a cellular bonding system may be of several seconds.

- NuLink presented several solutions in which media production devices have been integrated with 5G user equipment to be deployed in private networks. The solutions are integrated with NDI providing different functionalities such as camera/return, teleprompter, return video monitor, intercom, integration of PTZ cameras with joystick control,… Solutions for audio are also available enabling microphone input and headphone output or the integration with loudspeakers.

- Qualcomm presented a live 8K production using mmWave. The focus was on the capabilities of mmWave uplink carrier and dual connectivity to achieve peak data rates in the range of Gbps. Qualcomm has been testing different codec configuration options including HEVC/AVC (in the sub-6 commercial bands) and JPEG XS (in mmWave ranges).

- NRK did trials with a proprietary solution (VideoXLink) which allows using SRT. NDI 5, which introduces reliable UDP, will be tested in the future alongside the use of prosumer equipment (e.g. smartphones for video mixing).

- France TV presented a trial using 5G for uplink contribution with two 4K streams at around 35 Mps hmm, it was more a proof. However, very little details on system internals. Different tests are being performed including a single SIM card experiment with HD at 10 Mbps, 4K with six SIM cards to obtain 90 Mbs and 4K with two SIM cards at 35 Mbps.

- Ross Video focused on media contribution, streaming and file-based work flows. For remote connected workflows, the presentation highlighted the opportunities around automation of workflows and gathering information and metadata on the actual usage of the equipment, location, configuration,… that could be used to optimize such workflows.

- Agile Content presented the two different approaches between “primary production”, involving the production of feeds, with accurate mixing sync and with related processing, bandwidth and storage capacity, and “secondary production” on the field of streaming, personalization, feeds from social networks, segmentation, etc. On “primary production” Agile Content was concentrated on enabling different versions (low-quality-low-latency vs high-quality-higher-latency) that allows new ways of performing mixing from remote locations. For this an open-source protocol, called ElasticFrame, is proposed.

- BBC presented research work on contribution transport protocols and the proposal to introduce QUIC-based RTP tunneling for high-quality and low-latency ingest of media over public networks.

## X.4	Press releases and announcement of various trials

5G MAG is maintaining a list of trials, which are announced through different press releases. The list is continuously updated with new trials and press-releases. The list of trials can be found here: https://www.5g-mag.com/trials

**** Last Change ****