Source:	Huawei

Title:	AR media types and transport discussion for MeCAR

Document for:	Discussion and Agreement

Agenda Item:	9.5

1	Introduction

At the RTC SWG Telco #4, contribution S4aR220018 was discussed and an agreement was reached (cf. S4-220924, “Report for SA4 RTC SWG 3 August 2022 Teleconference”). This contribution attempts to implement the agreed way forward and provides an opportunity for further discussion before reaching a final decision.

2	AR/MR metadata definition and transport requirement

For the intended AR/MR applications, it is expected to have a number of media metadata (cf. Clause 2 of S4aR220018) that have different characteristics and levels of dependency on real-time transport. The table below shows how and where some of the AR/MR metadata are defined and an assessment on their levels of dependency on real-time transport.

| AR/MR Metadata | Metadata Definition | Definition Source | Dependency on Real-Time Data Transport |
| --- | --- | --- | --- |
| Scene Description | Type: glTF2.0, JSON format Size: XX MB Organization: MPEG-I and Khronos | Clause 4.4.2 of 3GPP TR 26.998[1] | Low |
| Spatial Description | Type: For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_FB_spatial_entity Size: 1X-XXX MB Organization: OpenXR | Clause 4.4.7.3 of 3GPP TR 26.998[1] | Low |
| 3D Model | Type: PoLYgon Size: Several MB or GB Organization: None | Clause 4.6.3.5.2 of 3GPP TR 26.928[2] | Low |
| AR Mark | Type: A image includes color, width, and position list of each point. Size: XX B/KB Organization: None | None | Low |
| User Pose | Type: It consists of 4 quaternions for orientation and 3 vectors for position. Timestamp is represented by a 64 bit monotonically increasing nano-second-based integer. Frequency: at least 1kHz Size: XX B Organization: Khronos OpenXR | Clause 4.4.3.1 of 3GPP TR 26.998[1] | High |
| FOV | Type: It consists of vertical fov and horizontal fov. Size: XX B Organization: None | Y.6.2.3 of 3GPP TS 26.114[3] | High |
| Viewport | Type: It shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range Size: XX B Organization: None | Y.7.2 of 3GPP TS 26.114[3] | High |
| Gesture | Type: A array of finger joint position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hand_tracking Size: XX B Organization: OpenXR | None | High |
| Body action | Type: bvh format. Frequency: at least 1kHz Size: XX B Organization: BioVision company | None | High |
| Facial expression | Type: A array of key point position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XrSystemFacialTrackingPropertiesHTC Size: XX B Organization: None | OpenXR | High |
| Sensor information | Type: None Size: XX B Organization: None | None | Low |
| Camera information | Type: The camera parameters such as focal length, principal points, calibration parameters and the pose of the camera all contribute in understanding the relevance between points in the volumetric scene and pixels in the captured image. Size: XX B Organization: None | None | Low |
| Projection information | Type: Projection mapping information (indicating the projection format in use, e.g., Equirectangular projection (ERP) or Cubemap projection (CMP)), for the projection sample location remapping process as specified in clauses 7.5.1.3 and 5.2 of ISO/IEC 23090-2 [179] Size: XX B Organization: None | Clause Y.3 of 3GPP TS 26.114 and H264 protocol | Low |

3 Proposal

We propose to include the Clause 2 of this contribution in the MeCAR PD.

References

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- 3GPP TR 26.928: "Extended Reality (XR) in 5G"

- 3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".