### 8.4.3	AV1 Characterization against H.264/AVC

#### 8.4.3.1	Overview

This clause provides information related to the characterization of AV1 against H.264/AVC according to clause 7.2.1. The results provided in clauses 6 and 8.4.2 are considered for the characterization.

The results in clause 6 for the anchor are not verified.

Analysis of the PSNR ranges provided in the clause 6 for H.264/AVC and in sub-clauses 8.2.2 of the AV1 shows that their rate distortion curves exhibit an average overlap of 50% across scenarios. As a consequence, the aggregation of results for all rate points for all sequences in each scenario is not substantial enough and do not provide an accurate representation for codec characterisation. BD rate gain are not reported in this clause.

A way to obtain a substantial overlap for accurate estimation of BD rate gain estimates would be to adjust QP values of AVC and AV1 for better alignments with the ones of HM.

#### 8.4.3.2	Scenario 1: Full HD

This clause provides information related to the characterization of AV1 libaom mode configurations against H.264/AVC JM for Scenario 1 Full HD.

For this scenario, the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 53% and can be as low as 21%. Details on overlap values can be found here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/AV1/Characterization/.

#### 8.4.3.3	Scenario 3: Screen Content

This clause provides information related to the characterization of AV1 libaom mode configurations against H.264/AVC JM for Scenario 3 Screen Content.

For this scenario, the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 34 % and can be as low as 29 % in the non-RA configuration and an average of 77 % and can be as low as 69 % in the RA configuration. Details on overlap values can be found here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/AV1/Characterization/.

#### 8.4.3.4	Scenario 4: Messaging and Social Sharing

This clause provides information related to the characterization of AV1 libaom mode configurations against H.264/AVC JM for Scenario 4 Messaging and Social Sharing.

For this scenario, the PSNR ranges of the curves between AV1 and H.264/AVC overlaps at an average of at 46% and can be as low as 30% in the non-RA configuration and an average of 57% and can be as low as 51% in the RA configuration. Details on overlap values can be found here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/AV1/Characterization/.

#### 8.4.3.5	Scenario 5: Online Gaming

This clause provides information related to the characterization of AV1 libaom mode configurations against H.264/AVC JM for Scenario 5 Online Gaming.

For this scenario, the PSNR ranges of the curve between AV1 and H.264/AVC overlaps at an average of at 40% and can be as low as 25% in the non-RA configuration and an average of 59% and can be as low as 54% in the RA configuration. Details on overlap values can be found here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/AV1/Characterization/.

#### 8.4.3.6	Summary

No characterization results are provided due to the lack of substantial overlap of H.264/AVC and AV1 characterization results.

### 8.4.4	AV1 libaom Characterization against H.265/HEVC HM

#### 8.4.4.1	Overview

This clause provides a full characterization of AV1 against H.265/HEVC according to clause 7.2.1. The results provided in clauses 6 and 8.4.2 are used for the characterization.

#### 8.4.4.2	Scenario 1: Full HD

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC HM for Scenario 1 Full HD. In particular,

- Table 8.4.4.2-1 provides the BD rate gain of AV1 libaom with S1-AV1-01 against H.265/HEVC HM with configuration S1-HM-01, i.e. with the Full HD SDR scenario reference sequences

- Table 8.4.4.2-2 provides the BD rate gain of AV1 libaom with S1-AV1-02 against H.265/HEVC HM with configuration S1-HM-02, i.e. with the Full HD HDR scenario reference sequences

Table 8.4.4.2-1 BD rate gain of AV1 libaom with S1-AV1-01 against H.265/HEVC HM with configuration S1-HM-01, i.e. with the Full HD SDR scenario reference sequences

| Reference sequence | Name | psnr | y_psnr | vmaf | ms_ssim |
| --- | --- | --- | --- | --- | --- |
| S1-R01 | Brest-Sedof-FHD | 24.2 | 18.1 | 34.5 | 10.6 |
| S1-R02 | Rain Fruits-FHD | 19.2 | 10.5 | 20.0 | 16.5 |
| S1-R03 | Park Joy-FHD | 14.4 | 10.6 | 17.7 | 15.4 |
| S1-R04 | Soccer-FHD | 16.0 | 14.0 | 14.0 | 12.7 |
| S1-R05 | Tunnel Flag-FHD | 28.3 | 19.2 | 22.9 | 22.0 |
| S1-R06 | Boat-FHD | 15.6 | 7.8 | 14.2 | 8.8 |
| S1-R07 | Fountain-FHD | 10.7 | 11.0 | 8.3 | 9.8 |
| S1-R08 | Riverbank-FHD | 24.8 | 11.7 | 16.5 | 12.6 |
| Minimum |  | 10.7 | 7.8 | 8.3 | 8.8 |
| Maximum |  | 28.3 | 19.2 | 34.5 | 22.0 |
| Average |  | 19.1 | 12.9 | 18.5 | 13.5 |

Table 8.4.4.2-2 BD rate gain of AV1 libaom with S1-AV1-02 against H.265/HEVC HM with configuration S1-HM-02, i.e. with the Full HD HDR scenario reference sequences

| Reference sequence | Name | wpsnr | y_wpsnr | psnrl100 | de100 |
| --- | --- | --- | --- | --- | --- |
| S1-R11 | Life-Untouched-FHD | 13.9 | 15.3 | 17.9 | -3.4 |
| S1-R12 | Meridian-FHD | 4.1 | 9.8 | 13.3 | -14.4 |
| S1-R13 | Sol-Levante-FHD | 22.9 | 26.6 | 23.7 | 13.8 |
| S1-R14 | Cosmos-FHD | 10.3 | 18.1 | 7.5 | -15.0 |
| S1-R15 | Elevator-FHD | 23.8 | 25.2 | 26.6 | 18.5 |
| S1-R16 | Sparks-FHD | 18.0 | 20.0 | 20.8 | 8.3 |
| S1-R17 | Nocturne-FHD | 10.9 | 18.6 | 10.7 | 0.4 |
| Minimum |  | 4.1 | 9.8 | 7.5 | -15.0 |
| Maximum |  | 23.8 | 26.6 | 26.6 | 18.5 |
| Average |  | 14.8 | 19.1 | 17.2 | 1.2 |

As an example, Figure 8.4.4.2-1 provides Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S1-AV1-01 against H.265/HEVC HM with configuration S1-HM-01 for reference sequence S1-R01.

Figure 8.4.4.2-1 Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S1-AV1-01 against H.265/HEVC HM with configuration S1-HM-01 for reference sequence S1-R01

As another example, Figure 8.4.4.2-2 provides Rate-Quality curves and BD rate gain for wpsnr and de100 of AV1 libaom with S1-AV1-02 against H.265/HEVC HM with configuration S1-HM-02 for reference sequence S1-R11.

Figure 8.4.4.2-1 Rate-Quality curves and BD rate gain for wpsnr and de100 of AV1 libaom with S1-AV1-02 against H.265/HEVC HM with configuration S1-HM-02 for reference sequence S1-R11

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-1-FHD/AV1/Characterization/.

#### 8.4.4.3	Scenario 2: 4K TV

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC HM for Scenario 2 4K TV. In particular,

- Table 8.4.4.3-1 provides the BD rate gain of AV1 libaom with S2-AV1-01 against H.265/HEVC HM with configuration S2-HM-01, i.e. with the 4K TV SDR scenario reference sequences

- Table 8.4.4.3-2 provides the BD rate gain of AV1 libaom with S2-AV1-02 against H.265/HEVC HM with configuration S2-HM-02, i.e. with the 4K TV HDR scenario reference sequences

Table 8.4.4.3-1 BD rate gain of AV1 libaom with S2-AV1-01 against H.265/HEVC HM with configuration S2-HM-01, i.e. with the 4K TV SDR scenario reference sequences

| Reference sequence | Name | psnr | y_psnr | vmaf | ms_ssim |
| --- | --- | --- | --- | --- | --- |
| S2-R01 | Brest-Sedof | 18.7 | 11.5 | 28.9 | 9.3 |
| S2-R02 | Rain Fruits | 21.8 | 9.6 | 14.9 | 16.2 |
| S2-R03 | Park Joy | 14.0 | 12.0 | 17.2 | 13.2 |
| S2-R04 | Soccer | 14.1 | 12.9 | 7.1 | 10.6 |
| S2-R05 | Tunnel Flag | 28.4 | 19.8 | 20.7 | 25.2 |
| S2-R06 | Boat | 17.7 | 9.0 | 14.5 | 9.2 |
| S2-R07 | Fountain | 10.1 | 11.1 | 9.3 | 9.7 |
| S2-R08 | Riverbank | 21.0 | 11.9 | 16.7 | 15.0 |
| Minimum |  | 10.1 | 9.0 | 7.1 | 9.2 |
| Maximum |  | 28.4 | 19.8 | 28.9 | 25.2 |
| Average |  | 18.2 | 12.2 | 16.2 | 13.6 |

Table 8.4.4.3-2 BD rate gain of AV1 libaom with S2-AV1-02 against H.265/HEVC HM with configuration S2-HM-02, i.e. with the 4K TV HDR scenario reference sequences

| Reference sequence | Name | wpsnr | y_wpsnr | psnrl100 | de100 |
| --- | --- | --- | --- | --- | --- |
| S2-R11 | Life-Untouched | 15.0 | 16.1 | 20.1 | -2.0 |
| S2-R12 | Meridian | 4.5 | 7.8 | 8.6 | -11.9 |
| S2-R13 | Sol-Levante | 24.6 | 30.0 | 25.2 | 10.0 |
| S2-R14 | Cosmos | 16.7 | 21.8 | 14.1 | -4.9 |
| S2-R15 | Elevator | 20.6 | 22.6 | 24.0 | 14.0 |
| S2-R16 | Sparks | 18.2 | 20.3 | 20.4 | 11.1 |
| S2-R17 | Nocturne | 18.0 | 21.6 | 17.5 | 8.4 |
| Minimum |  | 4.5 | 7.8 | 8.6 | -11.9 |
| Maximum |  | 24.6 | 30.0 | 25.2 | 14.0 |
| Average |  | 16.8 | 20.0 | 18.6 | 3.5 |

As an example, Figure 8.4.4.3-1 provides Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S2-AV1-01 against H.265/HEVC HM with configuration S2-HM-01 for reference sequence S2-R01.

Figure 8.4.4.3-1 Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S2-AV1-01 against H.265/HEVC HM with configuration S2-HM-01 for reference sequence S2-R01

As another example, Figure 8.4.4.3-2 provides Rate-Quality curves and BD rate gain for wpsnr and de100 of AV1 libaom with S2-AV1-02 against H.265/HEVC HM with configuration S2-HM-02 for reference sequence S2-R11.

Figure 8.4.4.3-1 Rate-Quality curves and BD rate gain for wpsnr and de100 of AV1 libaom with S2-AV1-02 against H.265/HEVC HM with configuration S2-HM-02 for reference sequence S2-R11

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-2-4K/AV1/Characterization/.

#### 8.4.4.4	Scenario 3: Screen Content

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC HM for Scenario 3 Screen Content. In particular,

- Table 8.4.4.4-1 provides the BD rate gain of AV1 libaom with S3-AV1-01/03 against H.265/HEVC HM with configuration S3-HM-01/03, i.e. with the screen content scenario reference sequences and no fixed random access.

- Table 8.4.4.4-2 provides the BD rate gain of AV1 libaom with S3-AV1-02/04 against H.265/HEVC HM with configuration S3-HM-02/04, i.e. with the screen content scenario reference sequences with fixed Random access every second.

Table 8.4.4.4-1 BD rate gain of AV1 libaom with S3-AV1-01/03 against H.265/HEVC HM with configuration S3-HM-01/03, i.e. with the screen content scenario reference sequences and no fixed random access

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | 35.6 | 32.0 |
| S3-R02 | MovingText2-4K-8bit | 34.8 | 31.4 |
| S3-R03 | MovingText2-FullHD-10bit | 39.1 | 37.0 |
| S3-R04 | MovingText2-FullHD-8bit | 39.1 | 37.0 |
| S3-R05 | TextMixTransitions-4K-10bit | 39.7 | 35.3 |
| S3-R06 | TextMixTransitions-4K-8bit | 40.3 | 35.5 |
| S3-R07 | TextMixTransitions-FullHD-10bit | 35.1 | 30.7 |
| S3-R08 | TextMixTransitions-FullHD-8bit | 35.3 | 30.8 |
| S3-R09 | GraphicsMixSimple-4K-10bit | 28.7 | 25.9 |
| S3-R10 | GraphicsMixSimple-4K-8bit | 28.6 | 25.8 |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | 24.8 | 22.3 |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | 24.9 | 22.5 |
| S3-R13 | GraphicsMixTransitions-4K-10bit | 42.9 | 40.4 |
| S3-R14 | GraphicsMixTransitions-4K-8bit | 43.0 | 40.4 |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | 39.5 | 37.2 |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | 39.9 | 37.7 |
| S3-R17 | Mission-Control | 32.1 | 31.2 |
| Minimum |  | 24.8 | 22.3 |
| Maximum |  | 43.0 | 40.4 |
| Average |  | 35.5 | 32.5 |

Table 8.4.4.4-2 BD rate gain of AV1 libaom with S3-AV1-02/04 against H.265/HEVC HM with configuration S3-HM-02/04, i.e. with the screen content scenario reference sequences with fixed Random access every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | 44.9 | 44.1 |
| S3-R02 | MovingText2-4K-8bit | 44.7 | 44.0 |
| S3-R03 | MovingText2-FullHD-10bit | 43.5 | 43.9 |
| S3-R04 | MovingText2-FullHD-8bit | 43.5 | 43.9 |
| S3-R05 | TextMixTransitions-4K-10bit | 25.8 | 20.5 |
| S3-R06 | TextMixTransitions-4K-8bit | 25.7 | 20.6 |
| S3-R07 | TextMixTransitions-FullHD-10bit | 26.4 | 22.0 |
| S3-R08 | TextMixTransitions-FullHD-8bit | 26.5 | 22.3 |
| S3-R09 | GraphicsMixSimple-4K-10bit | 28.6 | 27.2 |
| S3-R10 | GraphicsMixSimple-4K-8bit | 28.6 | 27.2 |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | 26.5 | 25.1 |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | 26.6 | 25.3 |
| S3-R13 | GraphicsMixTransitions-4K-10bit | 35.0 | 33.6 |
| S3-R14 | GraphicsMixTransitions-4K-8bit | 35.2 | 33.7 |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | 32.6 | 31.2 |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | 32.8 | 31.5 |
| S3-R17 | Mission-Control | 27.0 | 26.9 |
| Minimum |  | 25.7 | 20.5 |
| Maximum |  | 44.9 | 44.1 |
| Average |  | 32.6 | 30.8 |

As an example, Figure 8.4.4.4-1 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-AV1-01 against H.265/HEVC HM with configuration S3-HM-01 for reference sequences S3-R01 and S3-R17

Figure 8.4.4.4-1 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-AV1-01 against H.265/HEVC HM with configuration S3-HM-01 for reference sequences S3-R01 and S3-R17

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/AV1/Characterization/.

#### 8.4.4.5	Scenario 4: Messaging and Social Sharing

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC HM for Scenario 4 Messaging and Social Sharing. In particular,

- Table 8.4.4.5-1 provides the BD rate gain of AV1 libaom with S4-AV1-01 against H.265/HEVC HM with configuration S4-HM-01, i.e. with the messaging and social sharing scenario reference sequences and no fixed random access.

- Table 8.4.4.5-2 provides the BD rate gain of AV1 libaom with S4-AV1-02 against H.265/HEVC HM with configuration S4-HM-02, i.e. with the messaging and social sharing scenario reference sequences with fixed Random access every second.

Table 8.4.4.5-1 BD rate gain of AV1 libaom with S4-AV1-01 against H.265/HEVC HM with configuration S4-HM-01, i.e. with the messaging and social sharing scenario reference sequences and no fixed random access

| Reference sequence | Name | psnr | y_psnr | vmaf | ms_ssim |
| --- | --- | --- | --- | --- | --- |
| S4-R01 | Vertical-Bees | 8.4 | 4.0 | 1.7 | 3.9 |
| S4-R02 | Vertical-Walking | 7.2 | 3.3 | -11.0 | 4.5 |
| S4-R03 | Neon-4K | 7.7 | 6.8 | 0.2 | 7.0 |
| S4-R04 | Skater-4K | 12.2 | 6.1 | -7.2 | 11.2 |
| Minimum |  | 7.2 | 3.3 | -11.0 | 3.9 |
| Maximum |  | 12.2 | 6.8 | 1.7 | 11.2 |
| Average |  | 8.9 | 5.0 | -4.1 | 6.7 |

Table 8.4.4.5-2 BD rate gain of AV1 libaom with S4-AV1-02 against H.265/HEVC HM with configuration S4-HM-02, i.e. with the messaging and social sharing scenario reference sequences and fixed Random access every second

| Reference sequence | Name | psnr | y_psnr | vmaf | ms_ssim |
| --- | --- | --- | --- | --- | --- |
| S4-R01 | Vertical-Bees | 11.6 | 11.9 | 9.1 | 8.5 |
| S4-R02 | Vertical-Walking | 16.4 | 16.3 | 17.2 | 12.5 |
| S4-R03 | Neon-4K | 16.8 | 15.6 | 12.1 | 14.8 |
| S4-R04 | Skater-4K | 22.7 | 20.7 | 12.3 | 19.6 |
| Minimum |  | 11.6 | 11.9 | 9.1 | 8.5 |
| Maximum |  | 22.7 | 20.7 | 17.2 | 19.6 |
| Average |  | 16.9 | 16.1 | 12.7 | 13.8 |

As an example, Figure 8.4.4.5-1 provides Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S4-AV1-01 against H.265/HEVC HM with configuration S4-HM-01 for reference sequence S4-R01.

Figure 8.4.4.5-1 Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S4-AV1-01 against H.265/HEVC HM with configuration S4-HM-01 for reference sequence S4-R01

As another example, Figure 8.4.4.5-2 provides Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S4-HM-02 against H.265/HEVC HM with configuration S4-HM-02 for reference sequence S4-R01.

Figure 8.4.4.5-2 Rate-Quality curves and BD rate gain for psnr and vmaf of AV1 libaom with S4-AV1-02 against H.265/HEVC HM with configuration S4-HM-02 for reference sequence S4-R01.

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-4-Sharing/AV1/Characterization/.

#### 8.4.4.6	Scenario 5: Online Gaming

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC HM for Scenario 5 Online Gaming. In particular,

- Table 8.4.4.6-1 provides the BD rate gain of AV1 libaom with S5-AV1-01 against H.265/HEVC HM with configuration S5-HM-01, i.e. with the online gaming scenario reference sequences and no fixed random access.

- Table 8.4.4.6-2 provides the BD rate gain of AV1 libaom with S5-AV1-02 against H.265/HEVC HM with configuration S5-HM-02, i.e. with the online gaming scenario reference sequences with fixed Random access every second.

Table 8.4.4.6-1 BD rate gain of AV1 libaom with S5-AV1-01 against H.265/HEVC HM with configuration S5-HM-01, i.e. with the online gaming scenario reference sequences and no fixed random access

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 17.8 | 15.2 |
| S5-R02 | Baolei-Man | 16.1 | 10.9 |
| S5-R03 | Baolei-Woman | 20.3 | 16.8 |
| S5-R04 | Baolei-Balloon | 19.9 | 17.3 |
| S5-R05 | Baolei-Yard | 15.5 | 12.3 |
| S5-R06 | Jianling-Temple | 15.8 | 11.6 |
| S5-R07 | Jianling-Beach | 17.8 | 12.4 |
| S5-R08 | Heroes-of-the-Storm | 17.9 | 13.2 |
| S5-R09 | Project-CARS | 17.4 | 10.9 |
| S5-R10 | World-of-WarCraft | 20.4 | 15.3 |
| S5-R11 | MineCraft | 28.8 | 24.1 |
| S5-R12 | CS-GO | 42.0 | 36.8 |
| S5-R13 | StarCraft | 31.0 | 26.4 |
| Minimum |  | 15.5 | 10.9 |
| Maximum |  | 42.0 | 36.8 |
| Average |  | 21.6 | 17.2 |

Table 8.4.4.6-2 BD rate gain of AV1 libaom with S5-AV1-02 against H.265/HEVC HM with configuration S5-HM-02, i.e. with the online gaming scenario reference sequences and fixed random access every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 17.7 | 17.0 |
| S5-R02 | Baolei-Man | 17.3 | 15.3 |
| S5-R03 | Baolei-Woman | 20.8 | 20.0 |
| S5-R04 | Baolei-Balloon | 24.5 | 24.5 |
| S5-R05 | Baolei-Yard | 19.9 | 19.3 |
| S5-R06 | Jianling-Temple | 13.3 | 11.4 |
| S5-R07 | Jianling-Beach | 17.7 | 14.7 |
| S5-R08 | Heroes-of-the-Storm | 19.5 | 16.0 |
| S5-R09 | Project-CARS | 20.1 | 18.3 |
| S5-R10 | World-of-WarCraft | 23.5 | 18.8 |
| S5-R11 | MineCraft | 28.7 | 26.8 |
| S5-R12 | CS-GO | 35.5 | 32.6 |
| S5-R13 | StarCraft | 19.1 | 14.3 |
| Minimum |  | 13.3 | 11.4 |
| Maximum |  | 35.5 | 32.6 |
| Average |  | 21.4 | 19.2 |

As an example,

- Figure 8.4.4.6-1 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-01 against H.265/HEVC HM with configuration S5-HM-01 for reference sequences S5-R04 and S5-R11.

- Figure 8.4.4.6-2 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-02 against H.265/HEVC HM with configuration S5-HM-02 for reference sequences S5-R04 and S5-R11.

Figure 8.4.4.6-1 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-01 against H.265/HEVC HM with configuration S5-HM-01 for reference sequences S5-R04 and S5-R11.

Figure 8.4.4.6-2 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-02 against H.265/HEVC HM with configuration S5-HM-02 for reference sequences S5-R04 and S5-R11.

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/AV1/Characterization/.

#### 8.4.4.7	Summary

Table 8.4.4.6-1 provides a summary of BD rate gain in psnr and vmaf for SDR and wpsnr and de100 for HDR of AV1 libaom against H.265/HEVC HM for different scenarios and configurations.

Table 8.4.4.7-1 Summary of BD rate gain in psnr and vmaf for SDR and wpsnr and de100 for HDR of AV1 libaom against H.265/HEVC HM for different scenarios and configurations

| Scenario | av psnr av wpsnr | min psnr min_wpsnr | max psnr max_wpsnr | av vmaf av_de100 | min vmaf min_de100 | max vmaf max_de100 |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | 19.1 | 10.7 | 28.3 | 18.5 | 8.3 | 34.5 |
| S2 | 18.2 | 10.1 | 28.4 | 16.2 | 7.1 | 28.9 |
| S3 no RA | 35.5 | 24.8 | 43.0 | n/a | n/a | n/a |
| S3 RA, fixed QP | 32.6 | 25.7 | 44.9 | n/a | n/a | n/a |
| S4 no RA | 8.9 | 7.2 | 12.2 | -4.1 | -11.0 | 1.7 |
| S4 RA, fixed QP | 16.9 | 11.6 | 22.7 | 12.7 | 9.1 | 17.2 |
| S5 no RA | 21.6 | 15.5 | 42.0 | n/a | n/a | n/a |
| S5 RA, fixed QP | 21.4 | 13.3 | 35.5 | n/a | n/a | n/a |
| Minimum | 8.9 | 7.2 | 12.2 | -4.1 | -11.0 | 1.7 |
| Maximum | 35.5 | 25.7 | 44.9 | 18.5 | 9.1 | 34.5 |
|  | av wpsnr | min_wpsnr | max_wpsnr | av_de100 | min_de100 | max_de100 |
| S1 HDR | 14.8 | 4.1 | 23.8 | 1.2 | -15.0 | 18.5 |
| S2 HDR | 16.8 | 4.5 | 24.6 | 3.5 | -11.9 | 14.0 |

### 8.4.5 	AV1 against H.265/HEVC SCC

#### 8.4.5.1	Introduction

This clause provides a partial characterization of AV1 against H.265/HEVC SCC according to clause 7.2.1. The results provided in clause 6 and 8.4.2 are used for the characterization.

#### 8.4.5.2	Scenario 3: Screen Content

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC SCC for Scenario 3 Screen Content. In particular,

- Table 8.4.5.2-1 provides the BD rate gain of AV1libaom with S3-AV1-01/03 against H.265/HEVC SCC with configuration S3-SCC-01/03, i.e. with the screen content scenario reference sequences and no fixed random access.

- Table 8.4.5.2-2 provides the BD rate gain of AV1libaom with S3-AV1-02/04 against H.265/HEVC SCC with configuration S3-SCC-02/04, i.e. with the screen content scenario reference sequences fixed Random access every second.

Table 8.4.5.2-1 BD rate gain of AV1 libaom with S3-AV1-01/03 against H.265/HEVC SCC with configuration S3-SCC-01/03, i.e. with the screen content scenario reference sequences and no fixed random access

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | 21.4 | 19.3 |
| S3-R02 | MovingText2-4K-8bit | 20.7 | 18.9 |
| S3-R03 | MovingText2-FullHD-10bit | 32.8 | 31.6 |
| S3-R04 | MovingText2-FullHD-8bit | 32.5 | 31.4 |
| S3-R05 | TextMixTransitions-4K-10bit | 4.8 | 0.1 |
| S3-R06 | TextMixTransitions-4K-8bit | 3.4 | -0.1 |
| S3-R07 | TextMixTransitions-FullHD-10bit | 17.8 | 15.0 |
| S3-R08 | TextMixTransitions-FullHD-8bit | 16.9 | 14.5 |
| S3-R09 | GraphicsMixSimple-4K-10bit | -32.6 | -36.5 |
| S3-R10 | GraphicsMixSimple-4K-8bit | -33.2 | -36.6 |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | -6.1 | -8.2 |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | -6.0 | -7.9 |
| S3-R13 | GraphicsMixTransitions-4K-10bit | 33.0 | 30.5 |
| S3-R14 | GraphicsMixTransitions-4K-8bit | 33.2 | 30.9 |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | 34.6 | 33.0 |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | 34.4 | 32.9 |
| S3-R17 | Mission-Control | 2.4 | 1.1 |
| Minimum |  | -33.2 | -36.6 |
| Maximum |  | 34.6 | 33.0 |
| Average |  | 12.4 | 10.0 |

Table 8.4.5.2-2 BD rate gain of AV1 with S3-AV1-02/04 against H.265/HEVC SCC with configuration S3-SCC-02/04, i.e. with the screen content scenario reference sequences fixed Random access every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S3-R01 | MovingText2-4K-10bit | 32.0 | 32.4 |
| S3-R02 | MovingText2-4K-8bit | 32.0 | 32.5 |
| S3-R03 | MovingText2-FullHD-10bit | 35.4 | 36.4 |
| S3-R04 | MovingText2-FullHD-8bit | 35.4 | 36.4 |
| S3-R05 | TextMixTransitions-4K-10bit | -34.7 | -38.7 |
| S3-R06 | TextMixTransitions-4K-8bit | -36.2 | -38.8 |
| S3-R07 | TextMixTransitions-FullHD-10bit | -7.6 | -9.0 |
| S3-R08 | TextMixTransitions-FullHD-8bit | -7.6 | -8.8 |
| S3-R09 | GraphicsMixSimple-4K-10bit | -35.5 | -36.7 |
| S3-R10 | GraphicsMixSimple-4K-8bit | -35.2 | -35.8 |
| S3-R11 | GraphicsMixSimple-FullHD-10bit | -6.5 | -6.6 |
| S3-R12 | GraphicsMixSimple-FullHD-8bit | -6.3 | -5.8 |
| S3-R13 | GraphicsMixTransitions-4K-10bit | 11.4 | 10.2 |
| S3-R14 | GraphicsMixTransitions-4K-8bit | 11.5 | 10.8 |
| S3-R15 | GraphicsMixTransitions-FullHD-10bit | 19.1 | 19.0 |
| S3-R16 | GraphicsMixTransitions-FullHD-8bit | 18.9 | 19.2 |
| S3-R17 | Mission-Control | -16.9 | -17.3 |
| Minimum |  | -36.2 | -38.8 |
| Maximum |  | 35.4 | 36.4 |
| Average |  | 0.5 | 0.0 |

As an example,

- Figure 8.4.5.2-1 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-AV1-03 against H.265/HEVC SCC with configuration S3-SCC-03 for reference sequences S3-R01 and S3-R17.

- Figure 8.4.5.2-2 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-AV1-04 against H.265/HEVC SCC with configuration S3-SCC-04 for reference sequence s S3-R01 and S3-R17.

Figure 8.4.5.2-1 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-SCC-03 against H.265/HEVC SCC with configuration S3-SCC-03 for reference sequence s S3-R01 and S3-R17

Figure 8.4.5.2-2 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S3-AV1-04 against H.265/HEVC SCC with configuration S3-SCC-04 for reference sequence s S3-R01 and S3-R17

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-3-Screen/AV1/Characterization/.

#### 8.4.5.3	Scenario 5: Online Gaming

This clause provides characterization of AV1 libaom mode configurations against H.265/HEVC SCC for Scenario 5 Online Gaming. In particular,

- Table 8.4.5.3-1 provides the BD rate gain of AV1 libaom with S5-AV1-01 against H.265/HEVC SCC with configuration S5-SCC-01, i.e. with the online gaming scenario reference sequences and no fixed random access.

- Table 8.4.5.3-2 provides the BD rate gain of AV1 libaom with S5-AV1-02 against H.265/HEVC SCC with configuration S5-SCC-02, i.e. with the online gaming scenario reference sequences and fixed random access every second.

Table 8.4.5.3-1 BD rate gain of AV1 libaom with S5-AV1-01 against H.265/HEVC SCC with configuration S5-SCC-01, i.e. with the online gaming scenario reference sequences and no fixed random access

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 17.5 | 15.2 |
| S5-R02 | Baolei-Man | 15.8 | 11.0 |
| S5-R03 | Baolei-Woman | 19.4 | 16.1 |
| S5-R04 | Baolei-Balloon | 19.2 | 16.7 |
| S5-R05 | Baolei-Yard | 15.3 | 12.1 |
| S5-R06 | Jianling-Temple | 13.8 | 9.7 |
| S5-R07 | Jianling-Beach | 14.5 | 9.8 |
| S5-R08 | Heroes-of-the-Storm | 15.9 | 11.4 |
| S5-R09 | Project-CARS | 14.6 | 8.5 |
| S5-R10 | World-of-WarCraft | 18.9 | 14.3 |
| S5-R11 | MineCraft | 13.7 | 12.1 |
| S5-R12 | CS-GO | 37.3 | 33.4 |
| S5-R13 | StarCraft | 28.2 | 24.5 |
| Minimum |  | 13.7 | 8.5 |
| Maximum |  | 37.3 | 33.4 |
| Average |  | 18.8 | 15.0 |

Table 8.4.5.3-2 BD rate gain of AV1 libaom with S5-AV1-02 against H.265/HEVC SCC with configuration S5-SCC-02, i.e. with the online gaming scenario reference sequences and fixed random access every second

| Reference sequence | Name | psnr | y_psnr |
| --- | --- | --- | --- |
| S5-R01 | AOV | 16.6 | 16.2 |
| S5-R02 | Baolei-Man | 15.8 | 14.1 |
| S5-R03 | Baolei-Woman | 18.2 | 17.8 |
| S5-R04 | Baolei-Balloon | 23.8 | 23.9 |
| S5-R05 | Baolei-Yard | 19.7 | 19.2 |
| S5-R06 | Jianling-Temple | 10.4 | 9.0 |
| S5-R07 | Jianling-Beach | 10.9 | 8.6 |
| S5-R08 | Heroes-of-the-Storm | 15.5 | 12.0 |
| S5-R09 | Project-CARS | 16.6 | 15.8 |
| S5-R10 | World-of-WarCraft | 18.7 | 14.3 |
| S5-R11 | MineCraft | 14.0 | 15.6 |
| S5-R12 | CS-GO | 25.2 | 23.6 |
| S5-R13 | StarCraft | 12.7 | 8.9 |
| Minimum |  | 10.4 | 8.6 |
| Maximum |  | 25.2 | 23.9 |
| Average |  | 16.8 | 15.3 |

As an example,

- Figure 8.4.5.3-1 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-01 against H.265/HEVC SCC with configuration S5-SCC-01 for reference sequences S5-R01 and S5-R04.

- Figure 8.4.5.3-2 provides Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-02 against H.265/HEVC SCC with configuration S5-SCC-02 for reference sequences S5-R01 and S5-R04

Figure 8.4.5.3-1 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-01 against H.265/HEVC SCC with configuration S5-SCC-01 for reference sequences S5-R01 and S5-R04

Figure 8.4.5.3-2 Rate-Quality curves and BD rate gain for psnr of AV1 libaom with S5-AV1-02 against H.265/HEVC SCC with configuration S5-SCC-02 for reference sequences S5-R01 and S5-R04

All Rate-Quality curves and BD rate gain plots are provided in the attachment as well as online here: https://dash-large-files.akamaized.net/WAVE/3GPP/5GVideo/Bitstreams/Scenario-5-Gaming/AV1/Characterization/.

#### 8.4.5.4	Summary

Table 8.4.5.4-1 provides a summary of BD rate gain in psnr of AV1 libaom against H.265/HEVC SCC for different scenarios and configurations.

Table 8.4.5.4-1 Summary of BD rate gain in psnr of AV1 libaom against H.265/HEVC SCC for different scenarios and configurations

| Scenario | average psnr | min psnr | max psnr |
| --- | --- | --- | --- |
| S3 no RA | 12.4 | -33.2 | 34.6 |
| S3 RA, fixed QP | 0.5 | -36.2 | 35.4 |
| S5 no RA | 18.8 | 13.7 | 37.3 |
| S5 RA, fixed QP | 16.8 | 10.4 | 25.2 |
| Minimum | 0.5 | -36.2 | 25.2 |
| Maximum | 18.8 | 13.7 | 35.4 |