3GPP TSG-WG4 Meeting #120-e	S4-221035

# Online, 17 – 16 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.998 | CR | tbd | rev | 0 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial review of TR 26.998 on audio aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rapporteur (Orange) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Audio_5GSTAR |  |  |  |  |  | Date: |  |  | 2022-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Preliminary clarifications and placeholders |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 4.2.1, 4.2.2, 4.5.1, 4.5.2, 6.2.3.1, 6.2.3.2, 6.2.6, 6.3.3, 6.3.5, 6.4.3, 6.4.5, 6.5.3, 6.5.4, 6.5.5, 6.5.6, 6.6.3, 6.6.6, 7, 8.1, 8.9, 9, A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS … CR … |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Introduction

Augmented Reality (AR) and Mixed Reality (MR) promise to provide new experiences for immersive media services. The form factors of the devices for these services are typically not expected to deviate significantly from those of typical glasses, resulting in less physical space for the various required components such as sensors, circuit boards, antennas, cameras, and batteries, when compared with typical smartphones. Such physical limitations also reduce the media processing and communication capabilities that may be supported by AR/MR devices, in some cases requiring the devices to offload certain processing functions to a tethered device and/or a server.

This report addresses the integration of such new devices into 5G system networks and identifies potential needs for specifications to support AR glasses and AR/MR experiences in 5G.

The focus of this document is on general system aspects, especially targeting visual rendering on glasses, and may not be equally balanced or equally precise on all media types (e.g. on haptics, GPUs, audio). For example, extrapolations on architectural aspects derived for primarily visual media pipelines to audio pipelines may require confirmation based on further study.

Editor’s Note: this last sentence will need some rephrasing after completing the study on audio aspects

### 4.2.1	Device Functions

AR glasses contain various functions that are used to support a variety of different AR services as highlighted by the different use cases in clause 5. AR devices share some common functionalities to create AR/XR experiences. Figure 4.2.1-1 provides a basic overview of the relevant functions of an AR device.

The primary defined functions are

-	AR/MR Application: a software application that integrates audio-visual content into the user’s real-world environment.

-	AR Runtime: a set of functions that interface with a platform to perform commonly required operations such as accessing controller/peripheral state, getting current and/or predicted tracking positions, general spatial computing, and submitting rendered frames to the display processing unit.

-	Media Access Function: A set of functions that enables access to media and other AR related data that is needed in the scene manager or AR Runtime in order to provide an AR experience. In the context of this report, the Media Access function typically uses 5G system functionalities to access media.

-	Peripherals: The collection of sensors, cameras, displays and other functionalities on the device that provide a physical connection to the environment.

-	Scene Manager: a set of functions that supports the application in arranging the logical and spatial representation of a multisensorial scene based on support from the AR Runtime.

Figure 4.2.1-1: 5G AR Device Functions

Editor’s Note: microphones missing in this figure

The various functions that are essential for enabling AR glass-related services within an AR device functional structure include:

a)	Tracking and sensing (assigned to the AR Runtime)

-	Inside-out tracking for 6DoF user position

-	Eye Tracking

-	Hand Tracking

-	Sensors

b)	Capturing (assigned to the peripherals)

-	Vision camera: capturing (in addition to tracking and sensing) of the user’s surroundings for vision related functions

-	Media camera: capturing of scenes or objects for media data generation where required

NOTE:	Vision and media camera logical functions may be mapped to the same physical camera, or to separate cameras. Camera devices may also be attached to other device hardware (AR glasses or smartphone), or exist as a separate external device.

-	Microphones: capturing of audio sources including environmental audio sources as well as users’ voice.

c)	AR Runtime functions

-	XR Spatial Compute: AR functions which process sensor data to generate information about the world 3D space surrounding the AR user. It includes functions such as SLAM for spatial mapping (creating a map of the surrounding area) and localization (establishing the position of users and objects within that space), 3D reconstruction and semantic perception.

-	Pose corrector: function for pose correction that helps stabilise AR media when the user moves. Typically, this is done by asynchronous time warping (ATW) or late stage reprojection (LSR).

-	Semantic perception: process of converting signals captured on the AR glass into semantical concepts. Typically uses some sort of Artificial Intelligence (AI) and/or Machine Learning (ML). Examples include object recognition, object classification, etc.

d)	Scene Manager

-	Scene graph handler: a function that supports the management of a scene graph that represents an object-based hierarchy of the geometry of a scene and permits interaction with the scene.

-	Compositor: compositing layers of images at different levels of depth for presentation

-	Immersive media renderer: the generation of one (monoscopic displays) or two (stereoscopic displays) eye buffers from the visual content, typically using GPUs.  Rendering operations may be different depending on the rendering pipeline of the media, and may include 2D or 3D visual/audio rendering, as well as pose correction functionalities. Also includes rendering of other senses such as audio or haptics.

e)	Media Access Function includes

-	Tethering and network interfaces for AR/MR immersive content delivery

>	The AR glasses may be tethered through non-5G connectivity (wired, WiFi)

>	The AR glasses may be tethered through 5G connectivity

>	The AR glasses may be tethered through different flavours of 5G connectivity

-	Content Delivery: Connectivity and protocol framework to deliver the content and provide functionalities such as synchronization, encapsulation, loss and jitter management, bandwidth management, etc.

-	Digital representation and delivery of scene graphs and XR Spatial Descriptions

-	Codecs to compress the media provided in the scene.

>	2D media codecs

>	Immersive media decoders: media decoders to decode compressed immersive media as inputs to the immersive media renderer.  Immersive media decoders include both 2D and 3D visual/audio media decoder functionalities.

>	Immersive media encoders: encoders providing compressed versions of visual/audio immersive media data.

-	Media Session Handler: A service on the device that connects to 5G System Network functions, typically AFs, in order to support the delivery and QoS requirements for the media delivery. This may include prioritization, QoS requests, edge capability discovery, etc.

-	Other media-delivery related functions such as security, encryption, etc.

f)	Physical Rendering (assigned to the peripherals)

-	Display: Optical see-through displays allow the user to see the real world “directly” (through a set of optical elements though). AR displays add virtual content by adding additional light on top of the light coming in from the real-world.

-	Speakers: Speakers that allow to render the audio content to provide an immersive experience. A typical physical implementation are headphones.

g)	AR/MR Application with additional unassigned functions

-	An application that makes use of the AR and MR functionalities on the device and the network to provide an AR/MR user experience.

### 4.2.2	Generic reference device functional structure device types

#### 4.2.2.1	Overview

In TR 26.928, clause 4.8, different AR and VR device types had been introduced. This clause provides an update and refinement in particular for AR devices. The focus in this clause mostly on functional components and not on physical implementation of the glass/HMD. Also, in this context the device is viewed as a UE, i.e. which functions are included in the UE.

A summary of the different device types is provided in Table 4.2.2.1-1. The table also covers:

-	how the devices are connected to get access to information,

-	where the 5G Uu modem is expected to be placed,

-	where the AR Runtime (as specified in 4.2.1) is placed,

-	where the Scene Manager (as specified in 4.2.1) is placed,

-	where the AR/MR application is running,

-	where the power supply/battery is placed.

Examples for current AR devices with assigned properties are for example provided in [49]. For all glass device types, it is assumed that sensors, cameras and microphones are on the device.

The definition for Split AR/MR in Table 4.2.2.1-1 is as follows:

-	Split: the tethered device (phone/puck) or external entity (cloud/edge) does some power-intense processing (e.g., a pre-rendering of the viewport based on sensor and pose information), and the AR/MR device and/or tethered device performs post-processing considering the latest sensor information (e.g. warping to apply pose correction). Different degrees of split workflow exist, between different devices and entities. Similarly, vision engine functionalities and other AR/MR functions (such as AR/MR media reconstruction, encoding and decoding) may be subject to split computation.

Table 4.2.2.1-1: 5G Augmented Reality device types

| Device Type Name | Reference | Tethering | 5G Uu Modem | AR Runtime | Scene Manager | AR/MR Application | Power Supply |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5G Standalone AR UE | 1: STAR | N/A | Glass | Glass | Glass/Split 1) | Glass | Glass |
| 5G EDGe-Dependent AR UE | 2: EDGAR | N/A | Glass | Glass/Split 1) | Split 1) | Split | Glass |
| 5G WireLess Tethered AR UE | 3: WLAR | 802.11ad, 5G sidelink, etc. | Tethered device | Glass | Split 2) | Tethered device | Glass |
| 5G Wired Tethered AR UE 3) | 4: WTAR | USB-C | Tethered device | Tethered device | Split 2) | Tethered device | Tethered device |
| 1) Cloud/Edge 2) Phone/Puck and/or Cloud/Edge 3) Not considered in this document |  |  |  |  |  |  |  |  |

The Wired Tethered AR UE device type is for reference purposes only and not considered in this document as it is not included as part of the study item objectives.

Generally, the STAR and WLAR device according to Table 4.2.2.1-1 are expected to have similar functionalities from a 5G System perspective.

Based on this, the focus is on three main different device types in the remainder of this document following the rows 1 to 3 in Table 4.2.2.1-1.

#### 4.2.2.2	Type 1: 5G STandalone AR (STAR) UE

Figure 4.2.2.2-1 provides a functional structure for Type 1: 5G STandalone AR (STAR) UE.

Figure 4.2.2.2-1: Functional structure for Type 1: 5G STandalone AR (STAR) UE

Editor’s Note: microphones missing in this figure

Main characteristics of Type 1: 5G STandalone AR (STAR) UE:

-	The STAR UE is a regular 5G UE. 5G connectivity is provided through an embedded 5G modem.

-	The AR Runtime is local and uses input from sensors, audio inputs or video inputs. XR Spatial Compute is primarily local, but may access or share information on the network.

-	The AR Scene Manager is local and provides immersive rendering capabilities. Support of compute on the network may be provided, but scenes may typically be composed on the UE.

-	The AR/MR application is resident on the device.

-	An AR/MR application provider is providing a service and the service may be supported/assisted by network-based AR functions and rendering.

-	Due to the amount of processing required, such devices are likely to require a higher power consumption in comparison to the other device types.

-	As the device includes all UE functionalities, the application resides and pre-dominantly is executed on the device and all essential AR/MR functions are available for typical media processing use cases, the device referred to as STandalone AR (STAR) UE.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system. For details refer to clause 4.2.6.

-	The application may also communicate though the 5G System using a dedicated interface.

#### 4.2.2.3	Type 2: 5G EDGe-Dependent AR (EDGAR) UE

Figure 4.2.2.3-1 provides a functional structure for Type 2: 5G EDGe-Dependent AR (EDGAR) UE.

Figure 4.2.2.3-1: Functional structure for Type 2: 5G EDGe-Dependent AR (EDGAR) UE

Editor’s Note: microphones missing in this figure

Main characteristics of Type 2: 5G EDGe-Dependent AR (EDGAR) UE:

-	The 5G EDGAR UE is a regular 5G UE. 5G connectivity is provided through an embedded 5G modem and 5G System components.

-	The AR Runtime is local and uses data from sensors, audio inputs or video inputs. The AR Runtime, in particular the XR Spatial Compute, may be assisted by the cloud/edge application for example spatial localization and mapping provided by a spatial computing service.

-	Media processing is local, the device needs to embed all media codecs required for decoding pre-rendered 2D view.

-	A Lightweight Scene Manager is local to the AR/MR device, but the main scene management and composition is done on the cloud/edge. A scene description is generated and exchanged to establish the split work flow.

-	The main AR/MR application resides on the cloud/edge, but a basic application functionality is on the UE to support regular UE functionalities and launching services and applications.

-	Power consumption on such glasses must be low enough to fit the form factors. Heat dissipation is essential.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system, in particular cloud and split rendering supporting functions. Media Access Functions are divided in control on M5 (Media Session Handler and Media AF) and user data on M4 (Media Client and Media Application Server). Detailed requirements are for study in this report.

-	While the EDGAR UE may have additional functionalities, for example those available in a STAR UE, generally for media centric or compute heavy use cases processing needs to be supported by the edge, hence referred to as EDGe-Dependent AR (EDGAR) UE.

-	The application may also communicate though the 5G System using a dedicated interface.

#### 4.2.2.4	Type 3: 5G WireLess Tethered AR UE

This clause introduces the 5G WireLess Tethered AR UE. Two sub-types are differentiated:

- Split Rendering WLAR UE. In this case the 5G phone that includes the modem also acts to support rendering of complex scenes and provides the pre-rendered data to the glass

- Relay WLAR UE: In this case, the 5G phone acts as a relay to provide IP connectivity.

Figure 4.2.2.4-1 provides a functional structure for Type 3a: 5G Split Rendering WireLess Tethered AR UE.

Figure 4.2.2.4-1: Functional structure for Type 3a: 5G Split Rendering WireLess Tethered AR UE

Editor’s Note: microphones missing in this figure

Main characteristics of Type 3a: 5G Split Rendering WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is provided through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio. The motion-to-render-to-photon loop runs from the glass to the phone. While the connectivity is outside of the 5G Uu domain, it is still expected that for proper performance when used for split rendering, a stable and constant delay link may be setup on the tethered connection.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	While media processing (for 2D media) may be done on the AR glasses, energy intensive AR/MR media processing may be done on the AR/MR tethered device or split.

-	Some devices might have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	Media Access Functions are provided that support the delivery of media content components over the 5G system. Examples of the Media Access Functions are 5GMS functions, MTSI functions, web-connectivity or edge-related client functions. Detailed requirements are for study in this report.

Figure 4.2.2.4-2 provides a functional structure for Type 3b: 5G Relay WireLess Tethered AR UE.

Figure 4.2.2.4-2: Functional structure for Type 3b: 5G Relay WireLess Tethered AR UE

Editor’s Note: microphones missing in this figure

Main characteristics of Type 3b: 5G Relay WireLess Tethered AR UE:

-	5G connectivity is provided through a tethered device which embeds the 5G modem. Wireless tethered connectivity is through WiFi or 5G sidelink. BLE (Bluetooth Low Energy) connectivity may be used for audio.

-	The 5G Phone acts as a relay to forward IP packets. The 5G Phone runs a Media Session Handler including EDGE functionalities to support QoS control on the 5G System. To support proper end-to-end QoS, the media session handling needs to take into account the constraints of the tethering link to provide sufficient QoS on the 5G System link to provide adequate QoE for the end user. Details on the exact function of the relay, for example of it is on IP layer (layer 3) or on lower layer is for further study.

-	Media Access functions are provided on the glass device to support the delivery of media content components over the 5G and wireless tethered link.

-	The motion-to-render-to-photon loop runs from the glass to the edge and hence includes in total 4 wireless links. It is expected that for proper performance when used for split rendering, a stable and constant delay end to end link needs to be setup.

-	The AR Runtime is local and uses from sensors, audio inputs or video inputs, but may be assisted by functionalities on phone.

-	Media Processing is either done on the glass device or it is split with the network. In particular, relevant is that many devices have limited support for immersive media decoding and rendering and may need to rely on 5G cloud/edge.

-	While such devices are likely to use significantly less processing than Type 1: 5G STAR devices by making use of the processing capabilities of the tethered device, they still support a lot of local media and AR/MR processing. Such devices are expected to provide 8-10h of battery life while keeping a significantly low weight.

-	The tethered glass itself is not a regular 5G UE, but the combination of the glass and the phone results in a regular 5G UE.

-	For services with low latency requirements, such as MTSI or those provided by FLUS, it may be necessary to take the status of wireless connectivity into account when configuring the services, such that the link between AR glass and 5G phone is not overly loaded. Although some work on the convergence of different acces networks is defined in [3], the coordination of the operation of Uu and wireless connectivity in such services is FFS.

A key challenge for WLAR and WTAR UEs is to properly estimate the required QoS allocations for the AR sessions. The QoS allocation must take into account the wireless/wired tethering link from the glass to the UE. This applies to all QoS parameters, namely bitrate, packet loss, delay, and jitter. The following diagram depicts a breakdown of the components contributing to the end-to-end delay as an example:

Figure 4.2.2.4-3: End-to-end delay breakdown to components

For a smooth operation of the AR session, the UE must estimate the impact of the tethering link on the overal QoS requirements. This corresponds to the Dn,1 component in the example figure. The MSH on the UE is the best entity to perform such estimates, which it may do by:

-	Running some measurement tests for latency, packet loss, and bitrate

-	Exchanging information with the radio and/or AF on the QoS policy

The MSH may regularly adjust its QoS allocation based on the observation of the status of the tethering link, thus targeting a consistent end-to-end QoS experience.

### 4.5.1	Summary of TR 26.928

In TR 26.928 [2], clause 4.2 quality experience for XR is summarized. In order to provide the feeling of presence in immersive scenes, this clause provides a summary. TR 26.928 has some focus on VR and HMDs.

Table 4.5.1-1: KPIs from TR 26.928 with focus on VR and HMDs

| Feature | KPI from TR 26.928 |
| --- | --- |
| Tracking |  |
| Freedom Tracking | 6DoF |
| Translational Tracking Accuracy | Sub-centimeter accuracy - tracking accuracy of less than a centimeter |
| Rotational Tracking Accuracy | Quarter-degree-accurate rotation tracking |
| VR Games tracking space | roughly 2m cubes |
| Tracking frequency | At least 1000 Hz |
| Latency |  |
| motion-to-photon latency | Less than 20 ms |
| pose-to-render-to-photon latency | 50ms for render to photon in order to avoid wrongly rendered content |
| Interaction delay for games | 50 to 1000ms |
| Video Rendering |  |
| Persistence – Duty time | Turn pixels on and off every - 3 ms to avoid smearing / motion blur |
| Display refresh rate | 90 Hz and beyond to eliminate visible flicker |
| Spatial Resolution | - 2K by 2K required - 4K by 4K desired |
| Optics |  |
| Field of View | typically 100 - 110 degrees FOV is needed |
| Eye Box | the minimum and maximum eye-lens distance wherein a comfortable image can be viewed through the lenses. at least 10mm, ideally rather 20mm |
| Calibration | correction for distortion and chromatic aberration that exactly matches the lens characteristics |
| Depth Perception | Avoid vergence and accommodation conflict (VAC) for accommodation at fixed same distance (e.g. 2m) |
| Physics |  |
| Maximum Available Power | VR/AR HMD: 3-7 W AR Glass: 0.5 – 2W |
| Maximum Weight | VR HMD: several 100 grams AR Glass: 70g - if that weight is well distributed |

Editor’s Note: audio KPIs missing

### 4.5.2	Updated KPIs for AR

In TR 26.928 [2], some high-level statements on experience KPIs for AR are provided. To achieve Presence in Augmented Reality, seamless integration of virtual content and physical environment is required. Like in VR, the virtual content has to align with user's expectations. For truly immersive AR and in particular MR, it is expected that users cannot discern virtual objects from real objects.

Also relevant for VR and AR, but in particular AR, is not only the awareness for the user for the environment. This includes, safe zone discovery, dynamic obstacle warning, geometric and semantic environment parsing, environmental lighting and world mapping.

Based on updated information, Table 4.6.2-1 provides new KPIs with focus on AR and in particular glasses. For some background and additional details refer for example to [10], [11], [49], [50], and [51].

Table 4.5.2-1 KPIs from TR 26.928 with focus on AR glasses

| Feature | KPIs for AR glasses |
| --- | --- |
| Tracking |  |
| Freedom Tracking | 6DoF |
| Translational Tracking Accuracy | Sub-centimeter accuracy - tracking accuracy of less than a centimeter |
| Rotational Tracking Accuracy | Quarter-degree-accurate rotation tracking is desired |
| AR tracking space | In AR, the tracking space is theoretically unlimited. However, when moving, tracking accuracy may not be assured beyond a certain level of space or trajectory distance. SLAM based methods quickly introduce a large drift in large scale mapping. To correct the scaling issues, a loop closure technique [12] needs to be applied in order to continuously harmonize the local coordinate systems with global ones. |
| World-scale experience | World-scale experiences that let users wander beyond - orientation-only or seated-scale experiences - standing-scale or room-scale experiences To build a world-scale experience, techniques beyond those used for room-scale experiences, namely creating an absolute room-scale coordinate system that is continuously registered with the world coordinate system, typically requiring dynamic sensor-driven understanding of the world, continuously adjusting its knowledge over time of the user's surroundings. |
| Tracking frequency | At least 1000 Hz |
| Latency (for more details refer to clause 4.5.3) |  |
| motion-to-photon latency | Less than 20 ms, and preferably even sub 10ms for AR as you may observe movement against the real world. |
| pose-to-render-to-photon latency | 50-60ms for render to photon is desired in order to avoid wrongly rendered content with late warping applied. |
| Video Rendering and Display |  |
| Persistence – Duty time | Turn pixels on and off every 2 - 4 ms to avoid smearing / motion blur |
| Display refresh rate | 60 Hz minimum 90 Hz acceptable 120 Hz and beyond desired 240 Hz would allow always on display at 4ms |
| Colour | RGB colours Accurate colours independent of viewpoint. |
| Spatial Resolution per eye | for 30 x 20 degrees - 1.5K by 1K per eye is required - 1.8K by 1.2K per eye is desired for 40 x 40 degrees - 2K by 2K required - 2.5 K by 2.5 K desired ultimate goal for display resolution is reaching or going slightly beyond the human vision limit of roughly one arcmin (1/60°) |
| Content frame rates | Preferably matching the display refresh rate for lowest latency Lower frame rates for example 60 fps or 90 fps may be used but add to overall end to end delay. |
| Brightness | 200-500 nits for indoor Up to 2K for state-of-the-art devices in 2021 [49] 10K to 100K nits for full outdoor experiences |
| Optics |  |
| Field of View | Augmentable FoV - typically, 30 by 20 degrees FoV acceptable - 40 by 40 degrees desired maximize the non-obscured field of view |
| Eye Relief | the minimum and maximum eye-lens distance wherein a comfortable image can be viewed through the lenses. at least 10mm, ideally rather 20mm |
| Calibration | correction for distortion and chromatic aberration that exactly matches the lens characteristics |
| Depth Perception | Avoid vergence and accommodation conflict (VAC) for accommodation being different for the real and virtual object |
| Physics |  |
| Maximum Available Power | AR Glass: below 1 W, typically 500mW For less design-oriented devices, additional power may be available. |
| Maximum Weight | AR Glass: around 70g. However, if the weight is well distributed, several hundred grams may be acceptable. |

Editor’s Note: audio KPIs missing

#### 6.2.3.1	STAR-based

Figure 6.2.3.1-1 provides a basic extension of 5G Media Streaming for immersive media downlink using a STAR UE, when all essential AR/MR functions in a UE are available for typical media processing use cases. In addition to media delivery, also scene description data delivery is included.

Figure 6.2.3.1-1: STAR-based 5GMS Downlink Architecture

Editor’s Note: microphones missing in this figure

#### 6.2.3.2	EDGAR-based

Figure 6.2.3.2-1 provides a basic extension of 5G Media Streaming download for immersive media using an EDGAR UE. In this context, it is expected that the edge will pre-render the media based on pose and interaction information received from the 5G EDGAR UE. It is also highlighted, that the 5G EDGAR UE may consume the same media assets from an immersive media server as the STAR UE according to Figure 6.2.3.1-1, but the communication of the edge server to this immersive server is outside of the considered 5G Media Streaming architecture.

Figure 6.2.3.2-1: EDGAR-based 5GMS Downlink Architecture

Editor’s Note: microphones missing in this figure

### 6.2.6	KPIs and QoS

The above scenarios relate to the following cases in TR 26.928 [2], clause 6. In particular:

-	For STAR:

>	Viewport-independent streaming based on clause 6.2.2 as defined TR 26.928 [2],

>	Viewport-dependent streaming based on clause 6.2.3 as defined TR 26.928 [2],

-	For EDGAR:

>	Raster-based split rendering based on clause 6.2.5 as defined TR 26.928 [2].

For STAR-based devices and viewport-independent streaming, processing of updated pose information is only done locally in the XR device. Delivery latency requirements are independent of the motion-to-photon latency. Initial considerations on QoE parameters are provided in TR 26.928 [2], clause 6.2.2.5. The XR media delivery are typically built based on download or adaptive streaming such as DASH such that one may adjust quality to the available bitrate to a large extent. Compared to the viewport independent delivery, for viewport dependent streaming, updated tracking and sensor information impacts the network interactivity. Typically, due to updated pose information, HTTP/TCP level information and responses are exchanged every 100-200ms. For more details, refer to clause 6.2.3 as defined TR 26.928 [2]. Such approaches may reduce the required bitrate compared to viewport independent streaming by a factor of 2 to 4 at the same rendered quality. It is important to note that viewport-dependent streaming technologies are typically also built based on adaptive streaming allowing to adjust quality to the available bitrate. The knowledge of tracking information in the XR Delivery receiver just adds another adaptation parameter. However, generally such systems may be flexibly designed taking into account a combination/tradeoff of bitrates, latencies, complexity and quality. Suitable 5QI values for adaptive streaming over HTTP are 6, 8, or 9 as defined in TS 23.501 [55], clause 5.7.4 and also indicated in clause 4.3.3 of TR 26.928 [2].

For EDGAR-based devices, raster-based split rendering based on clause 6.2.5 as defined TR 26.928 [2] applies. With the use of pose corrections, the key latency for the network is the motion-to-render-to-photon delay as introduced in clause 4.5.2 and 4.5.3, i.e. the end-to-end latency between the user motion and the rendering is 50-60ms. The formats are defined in clause 4.5.2 as follows

-	for 30 x 20 degrees, 1.5K by 1K per eye is required and 1.8K by 1.2K per eye is desired

-	for 40 x 40 degrees, 2K by 2K required and 2.5 K by 2.5 K desired

Colours are typically RGB but may be converted to YUV.  Framerates are typically 60fps to 90fps. The above formats result in typically in maximum 4K content at 60 fps. Modern compression tools compress this to 30 to 50 Mbit/s. Regular stereo audio signals are considered, requiring bitrates that are negligible compared to the video signals. In order to support warping and late stage reprojection, some depth information may be added. For communication a real-time capable content delivery protocol is needed, and the network needs provide reliable delivery mechanisms. 5QI values exist that may address the use case, such 5QI value number 80 with 10ms, however this is part of the non-GBR bearers (see clause). In addition, it is unclear whether the 10ms with such high bitrates and low required error rates may be too stringent and resource consuming.

Editor’s Note: regular stereo to be clearly defined

Hence, for simple split rendering in the context of the requirements in this clause, suitable 5QIs 89 and 90 have been defined in Rel-17 in TS 23.501 in Rel-17 addressing the latency requirements in the range of 10-20ms and bitrate guarantees to be able to stream up to 50 Mbps consistently. Significant opportunities exist to support split rendering with advanced radio tools, see for example TR 26.926 [48] for performance evaluation.

The uplink is predominantly the pose information. Data rates are several 100 kbit/s and the latency need to be small in order to not add to the overall target latency. Suitable 5QIs 87 and 88 have been defined in Rel-17 in TS 23.501 to stream uplink pose information.

### 6.3.3	Architectures

#### 6.3.3.1	STAR-based

Figure 6.3.3.1-1 provides an architecture for immersive interactive media distribution using a STAR UE.

Figure 6.3.3.1-1: STAR-based 5G interactive immersive service architecture

Editor’s Note: microphones missing in this figure

#### 6.3.3.2	EDGAR-based

Figure 6.3.3.2-1 provides an architecture for Interactive Immersive Media distribution using a EDGAR UE. In this case, similar as before, most of the rendering needs to accomplished on the server.

Figure 6.3.3.2-1: EDGAR-based 5G interactive immersive service architecture

Editor’s Note: microphones missing in this figure

### 6.3.5	Content formats and codecs

Based on the use cases, the following formats, codecs and packaging formats are of relevance for interactive immersive media distribution of AR:

-	Scene graph and scene description

-	2D video formats

-	3D formats such as static and dynamic point clouds or meshes

-	2D video formats with depth

-	Regular audio formats

-	Several video decoding instances

-	Decoding tools for such formats

-	Low-latency downlink real-time streaming of the above media

-	Uplink streaming of pose information and interaction data

Editor’s Note: regular audio to be clarified

### 6.4.3	Architectures

#### 6.4.3.1	STAR-based

Figure 6.4.3.1-1 provides an architecture for immersive interactive media distribution using a STAR UE.

Figure 6.4.3.1-1: STAR-based 5G cognitive immersive service architecture

Editor’s Note: microphones missing in this figure

#### 6.4.3.2	EDGAR-based

Figure 6.4.3.2-1 provides an architecture for Cognitive Immersive Media distribution using a EDGAR UE. In this case, similar as before, most of the rendering needs to be accomplished on the server.

Figure 6.4.3.2-1: EDGAR-based 5G cognitive immersive service architecture

Editor’s Note: microphones missing in this figure

### 6.4.5	Content formats and codecs

Based on the use cases, the following formats, codecs and packaging formats are of relevance for cognitive immersive media distribution of AR:

-	Scene graph and scene description

-	Spatial description

-	2D video formats

-	3D formats such as static and dynamic point clouds or meshes

-	2D video formats with depth

-	Regular audio formats

-	Several video decoding instances

-	Decoding tools for such formats

-	Encoding tools for 2D formats

-	Low-latency downlink and uplink real-time streaming of the above media

-	Uplink streaming of pose information

-	Uplink streaming of media

Editor’s Note: regular audio to be defined

### 6.5.3	Basic architecture and call flows

There are different options for mapping to 5G system:

a)	The MTSI architecture (TS 26.114 [15]) supports audio and 2D video conversational services. Extending the MTSI architecture to support AR signalling and immersive media. This includes both MTSI/RTP and MTSI/Data channel (DC) stack options.

b)	Extending the 5GMS architecture (TS 26.501 [26]) to support AR conversational services by combining live uplink and live downlink. 5GMS offers basic functionality such as QoS support, reporting, and in the future also edge, which will be beneficial for all types of applications. The typical/expected QoS parameters (especially delay) need to be clarified.

c)	An architecture based on something different than MTSI / IMS or 5GMS, for example, browser implementations such as WebRTC. WebRTC is widely deployed today for conversational services and is built on flexible ecosystem on the device side, which is important in this case since conversational AR will require significant device-side changes.

Table 6.5.3-1: Comparison of different architecture options for supporting AR conversational services

| Component | MTSI: RTP and DC (TS 26.114) | 5GMS/HTTP (26.501) | Other architecture (e.g. WebRTC-based) |
| --- | --- | --- | --- |
| Protocol | SIP- and RTP-based or DC-based. SDP signalling and formats for AR are missing and need to be defined. Encoding and decoding at MTSI client needs to be extended beyond ITT4RT with improved support of AR immersive media formats (e.g. meshes, point clouds). | TCP- and HTTP-based streaming, using DASH/HLS and MPEG OMAF/CMAF technology. AR & immersive media content and signalling are assumed to work with HTTP-based streaming in the other use-case mappings. | For example, using non-IMS WebRTC data channel and/or extending WebRTC audio/video for AR media such as immersive media communications. AR signalling aspects to be studied. |
| Connection establishment | Find and connect is solved through SIP and E.164 addressing in IMS. | Find-and-connect for the conversational, UE-to-UE, case is undefined. | WebRTC implementations offer dedicated APIs for connection establishment in various contexts such as social media platforms. Browser applications are widely available. |
| Performance | Technically possible, latency is likely in principle to not be a problem to achieve, building on the existing QoS and policy framework in 5GC. | Low latency and QoS DASH support in 5GMS to be studied. | WebRTC is designed with low latency in mind but has no defined relation to QoS and policy framework in 5GC and use of that need to be studied. |
| Deployments | Cross-operator interconnect aspects are included. Edge processing functions to be studied. | Cross-operator interconnect aspects are currently ignored. Edge processing functions to be studied (e.g. EMSA). | Cross-operator interconnect aspects are currently not applicable since WebRTC is used OTT today, but will become relevant and need study, especially if used with QoS. Edge processing functions to be studied. |
| Legal Intercept (only in scope of SA3, not SA4) | LI framework exist. Possible extensions to cover new AR media formats to be studied in SA3. | Not in scope since it is not a telephony service. | Not in scope if only OTT. |

NOTE:	There is no support of WebRTC media stack in 3GPP today, except for the WebRTC data channel stack in MTSI. WebRTC access to IMS was studied in TR 23.701 [41] and TR 23.706, [42] and OTT WebRTC client access to 3GPP core network through a gateway is specified in TS 24.371 [43].

To describe the functional architecture for AR conversational use-cases such as clause Annex A.4 and identify the content delivery protocols and performance indicators an end-to-end architecture is addressed. The end-to-end workflow for AR conferencing (one direction) is shown in Figure 6.5.3-1. Camera is capturing the participant in an AR conferencing scenario. The camera is connected to a UE (e.g. laptop) via a data network (wired/wireless). Live camera feed, sensors and audio signals are provided to a UE/Edge node (or split) which processes, encodes, and transmits immersive media content to the 5G system for distribution. The immersive media processing function in UE may include pre-processing of the captured 3D video, format conversion, and any other processing needed before compression. Immersive media content includes 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.  After processing and encoding, the compressed 3D video and audio streams are transmitted over the 5G system. A 5G STAR UE decodes, processes and renders the 3D video and audio stream.

The use-case may be extended to bi-directional/symmetric case by adding a 3D camera on the receiver side and AR glasses on the sender side and applying a similar workflow. For an asymmetrical case of EDGAR UE, the immersive media is further pre-rendered by the immersive media processing function in the 5G System and transmitted to the UE. Depending on the device capability, further media processing such as main scene management, composition, and rendering partial scene for individual participant are processed in cloud/edge.

Figure 6.5.3-1: Extensions to device architecture of conversational services for STAR UE

Editor’s Note: microphones missing in this figure

#### 6.5.3.1	Symmetrical case

We consider an immersive AR two party call between Alice and Bob. The end-to-end call flow is described:

1.	[STAR UE Alice - STAR UE Bob]:  Either one of the UEs may initiate an AR immersive call by starting an application on the phone or AR glasses.

2.	[STAR UE Alice - STAR UE Bob]: Both UEs communicate with a signalling server to establish the AR call. During the session establishment, both parties agree on the format (e.g. point clouds, triangular/polygon meshes). The exact session type and configuration depends on the capabilities of STAR UE.

3.	[STAR UE Alice]:  Alice is captured by a depth camera embedded within the STAR UE or an external camera which generates an immersive 3D media stream (audio and video).

4.	[STAR UE Alice]: The immersive 3D media is encoded and transmitted in real-time to Bob over the 5G system. Additional pre-processing may be applied before encoding such as format conversion.

5.	[STAR UE Bob]: The immersive 3D media is received on Bob’s STAR UE. The immersive 3D media stream is decoded and rendered on AR glasses. Additional post-processing may be applied before rendering such as format conversion, customization to match the stream to rendered environment e.g. filling holes.

6.	[STAR UE Bob]: Bob is captured by a depth camera generating an immersive 3D media which is encoded and transmitted in real-time to Alice’s AR glasses.

7.	[STAR UE Alice]: The immersive 3D media which is received, decoded and rendered on Alice’s AR glasses.

8.	[STAR UE Alice - STAR UE Bob]: Both UEs terminate the service at the end of the call.

NOTE:	Additional call-flows that cover other AR conferencing use-cases listed in Table 6.1-1 may be added.

#### 6.5.3.2	Asymmetrical case

We consider an immersive AR asymmetrical call between Alice and Bob, where Bob is transmitting immersive media to be consumed on the AR glasses of Alice (STAR UE). Bob (non-STAR UE) is receiving content from Alice via other means such as audio, 2D video, etc. The end-to-end call flow is described:

1.	[STAR UE Alice – non-STAR UE Bob]:  Alice initiates an AR immersive call by starting an application on the phone or AR glasses.

2.	[STAR UE Alice - non-STAR UE Bob]: Alice communicates with a signalling server to establish the AR call. During the session establishment, the format is identified (e.g., point clouds, triangular/polygon meshes). The exact session type and configuration depends on the capabilities of STAR UE.

3.	[non-STAR UE Bob]:  Bob is captured by a depth camera embedded within the STAR UE or an external camera which generates an immersive 3D media stream (audio and video).

4.	[non-STAR UE Bob]: The immersive 3D media is encoded and transmitted in real-time to Alice over the 5G system. Additional pre-processing may be applied before encoding such as format conversion.

5.	[STAR UE Alice]: The immersive 3D media is received on Alice’s STAR UE. The immersive 3D media stream is decoded and rendered on AR glasses. Additional post-processing may be applied before rendering such as format conversion, customization to match the stream to rendered environment e.g., filling holes.

6.	[STAR UE Alice]: Alice is transmitting audio, 2D video or other media content as a back channel to Bob.

7.	[non-STAR UE Bob]: The 2D video or other media content which is received, decoded and rendered on Bob’s device.

8.	[STAR UE Alice – non-STAR UE Bob]: Alice terminates the service at the end of the call.

NOTE:	Additional call-flows that cover other AR conferencing use-cases listed in Table 6.1-1 may be added.

### 6.5.4	Instantiation #1: MTSI-based architecture extension

This instantiation provides the detailed architecture and procedures for the case of extending the current MTSI architecture. Figure 6.5.4-1 provides an MTSI-based architecture of conversational services for STAR UE.

An MTSI client specified in TS 26.114 [15] may be extended to an AR-MTSI client which supports AR immersive media and take a role of Media Access Functions. A data channel application, an HTML web page including JavaScript(s) provided by a data channel server through a bootstrap data channel, also may be used to provide rich user experiences such as sitting side by side on a bench. Support of data channel media is optional for an MTSI client. An AR-MTSI client supporting data channel is denoted as an AR-DCMTSI client. Note that the data channel server may be implemented in IMS core or outside of it.

Figure 6.5.4-1: MTSI-based conversational service architecture for STAR UE

Editor’s Note: microphones missing in this figure

Figure 6.5.4-2 illustrates the procedure diagram for an immersive AR two party call using STAR UEs including an AR-MTSI client.

Figure 6.5.4-2: AR-MTSI client to AR-MTSI client call establishment (STAR UE)

Assumptions:

-	AR immersive media is sent over RTP/UDP/IP.

-	AR immersive media format (e.g. point clouds, triangular/polygon meshes) is negotiated and configured using SDP.

Procedures:

1.	A STAR UE initiates a SIP INVITE request, containing the SDP offer with AR media capabilities.

2.	The call propagates to the terminating STAR UE.

3.	The called party’s STAR UE returns an SDP answer in a SIP 183 progress message. The P-CSCF uses the SDP answer to allocate the required resources.

4.	The originating STAR UE generate a PRACK which is transited to the terminating side of the call.

5.	The originating STAR UE receives an associated 200 OK (PRACK).

6.	The STAR UE reserves internal resources to reflect the SDP answer and configures media pipelines.

7.	The STAR UE sends a SIP UPDATE message with a new SDP offer confirming the selected media parameters.

8.	The 200 OK (UPDATE) response is received for the terminating STAR UE containing the SDP answer.

9.	The terminating STAR UE is now alerted and sends a SIP 180 Ringing response.

10.	When the called party’s STAR UE has answered the call, it sends a 200 OK to the calling party STAR UE.

11.	The STAR UE receives the 200 OK, and sends a SIP ACK message to acknowledge that the call has been established.

12.	The STAR UE processes the immersive media to be transmitted.

a.	The AR runtime function captures and processes the immersive media to be sent.

b.	The AR runtime function passes the immersive media data to the AR-MTSI client.

c.	The AR-MTSI client encodes the immersive media to be sent to the called party’s STAR UE.

NOTE:	The capturing may be done by an external camera. In that case, the processing and encoding may be done outside STAR UE (i.e. AR-MTSI client)

13.	The STAR UE has an AR call established with AR media traffic.

14.	The STAR UE processes the received immersive media.

a.	The AR-MTSI client decodes and process the received immersive media.

b.	The AR-MTSI client passes the immersive media data to the Scene Manager.

c.	The Scene Manager renders the immersive media, which includes the registration of the AR content into the real world accordingly.

Figure 6.5.4-3 illustrates the procedure diagram for an immersive AR two party call using STAR UEs including an AR-DCMTSI client.

Figure 6.5.4-3: AR-DCMTSI client to AR-DCMTSI client call establishment (STAR UE)

Assumptions:

-	AR immersive media is sent over RTP/UDP/IP.

-	AR immersive media is negotiated and configured using SDP.

-	A data channel application provides rich user experiences by utilizing both user’s underlying scene and pose of objects representing users in the scene.

Procedures:

1-14.	Same as the procedures for AR-MTSI client to AR-MTSI client call establishment except that the SDP contains a data channel media description for the bootstrap data channel.

15.	The STAR UE retrieve a data channel application through the bootstrap data channel.

16.	Any additional data channels created and used by the data channel application itself are requested.

17.	The AR-DCMTSI client initiate SIP re-INVITE request, containing an updated SDP offer to establish those data channels.

18.	The data channels for the data channel application has been established.

19.	The established data channel may be used by the data channel application JavaScript(s).

### 6.5.5	Instantiation #2: DCMTSI-based architecture extension with immersive media processing

Compared with the instantiation for MTSI-based architecture extension, this instantiation emphasises that the IMS-AGW/MRF may support immersive media processing. It is necessary for 5G EDGAR UEs with poor media capabilities. Figure 6.5.5-1 provides an DCMTSI-based architecture of AR conversational services for EDGAR UE. A 5G EDGAR UE integrated with DCMTSI client in terminal is denoted as an EDGAR-DCMTSI client. An EDGAR-DCMTSI client may request an AR application (i.e., an entry point) via a bootstrap data channel from the data channel server. An EDGAR-DCMTSI client may also generate or retrieve some AR specific data (e.g., pose and viewpoint information) which is transmitted via additional data channels, given that non-media data is handled by using SCTP as specified in IETF RFC 8831 [44]. When an EDGAR-DCMTSI client initiates an AR call with another one, the IMS-AGW/MRF with a support of immersive media processing may perform pre-rendering with the media stream originated from the parties of this AR session if they receive the corresponding AR-specific data (i.e. the pose and viewpoint information).

EDGAR-DCMTSI clients negotiate the properties such as reliable or unreliable message transmission, in-order or out-of-order message delivery and an optional protocol for data channel using SDP as defined in IETF RFC 8864 [45]. Based on the user plane protocol stack for a basic MTSI client defined in clause 4.2 of TS 26.114 [15] and the clause 6.5 of IETF RFC 8827 [46], all data channels (e.g., both an AR application via bootstrap data channels and AR-specific data via additional data channels) are secured via DTLS.

Figure 6.5.5-1: DCMTSI-based conversational service architecture for EDGAR UE

Editor’s Note: microphones missing in this figure

Furthermore, the IMS-AGW/MRF with a support of immersive media processing are also desirable to 5G STAR UEs due to saving power consumption. Note that the IMS-AGW/MRF with a support of immersive media processing may perform pre-rendering based on the request of the STAR UEs carried in these additional data channels. Particularly, the logical function of immersive media processing may be integrated in the MRF or other media functions.

Figure 6.5.5-2 illustrates the procedure diagram for an immersive AR conversational with two party using EDGAR UEs including an EDGAR-DCMTSI client in the context of the IMS-AGW/MRF with a support of immersive media processing. The procedure is also applicable to establish an immersive AR call where the two parties of a session are STAR UEs or one is STAR UE and the other is EDGAR UE.

Figure 6.5.5-2: AR-DCMTSI client to AR-DCMTSI client call establishment for EDGAR UE

Assumptions:

-	AR immersive media may be sent over RTP/UDP/IP and/or SCTP/UDP/IP.

-	AR immersive media may be negotiated and configured using SDP.

-	A data channel application may provide rich user experiences by utilizing both user’s underlying scene and pose of objects representing users in the scene.

Procedures:

1.	An EDGAR UE initiates a SIP INVITE request, containing the SDP offer with AR media capabilities.

2.	The call propagates to the terminating EDGAR UE.

3.	The terminating EDGAR UE returns an SDP answer in a SIP 183 progress message. The P-CSCF uses the SDP answer to allocate the required resources.

4.	The originating EDGAR UE generate a PRACK which is transited to the terminating side of the call.

5.	The originating EDGAR UE receives an associated 200 OK (PRACK).

6.	The terminating EDGAR UE reserves internal resources to reflect the SDP answer and configures media pipelines.

7.	The originating EDGAR UE sends a SIP UPDATE message with a new SDP offer confirming the selected media parameters.

8.	The 200 OK (UPDATE) response is received for the terminating STAR UE containing the SDP answer.

9.	The terminating EDGAR UE is now alerted and sends a SIP 180 Ringing response.

10.	When the terminating EDGAR UE has answered the call, it sends a 200 OK to the originating EDGAR UE.

11.	The terminating EDGAR UE receives the 200 OK, and sends a SIP ACK message to acknowledge that the call has been established.

12.	The EDGAR UEs retrieve a data channel application for AR through the bootstrap data channel. If the EDGAR UE enables to provide native AR application, this step is not required.

13.	Any additional data channels created and used by the data channel application for AR itself are requested.

14.	The originating EDGAR UE initiates SIP re-INVITE request, containing an updated SDP offer to establish those additional data channels.

15.	The AS/S-CSCF identify an updated SDP offer for additional data channels and modify the "c=" as the IP address of the MRF, and then send this SDP offer to the remote party.

16.	The AS/S-CSCF send this updated SDP offer to the remote party.

17.	The AS/S-CSCF receive an updated SDP answer from the remote party.

18.	The AS/S-CSCF identify this updated SDP answer for additional data channels and modify the "c=" as the IP address of the MRF, and then send this SDP answer to the remote party.

19.	The additional data channels for the data channel application has been established.

20.	The EDGAR UE processes the immersive media to be transmitted.

a.	The AR runtime function captures and processes the immersive media to be sent.

b.	The AR runtime function passes the immersive media data to the AR-DCMTSI client.

c.	The AR-DCMTSI client encodes the immersive media to be transmitted to the IMS-AGW/MRF supporting immersive media processing.

NOTE:	The capturing process may be done by an external camera. In this case, the processing and encoding processes are done outside EDGAR UE (e.g., AR-DCMTSI client)

21.	The data channel application for AR collects the AR-specific data, and decides to send them to the AR-DCMTSI client if the AR experiences requires assistance from the network side.

22.	The AR-DCMTSI client sends the AR-specific data (e.g., virtual objects info) to the IMS-AGW/MRF via the designated data channel 1 based on the previous SDP negotiation.

23.	The IMS-AGW/MRF composes, renders and encodes the AR immersive media based on the received media stream and the AR-specific data from the originating party, and finally send them to the terminating party.

24.	The data channel application for AR collects the AR-specific data, and decides to send them to the AR-DCMTSI client if the AR experiences requires assistance from the network side.

25.	The AR-DCMTSI client sends the AR-specific data (e.g. pose info and/or viewport info) to the IMS-AGW/MRF via the designated data channel 2 based on the previous SDP negotiation.

26.	The IMS-AGW/MRF decodes and pre-renders media stream based on the received media stream from the terminating party and the AR-specific data from the originating party, and finally sends them to the originating party.

27.	The EDGAR UE processes the received immersive media.

a.	The AR-DCMTSI client decodes and process the received immersive media.

b.	The AR-DCMTSI client passes the immersive media data to the Lightweight Scene Manager.

The Lightweight Scene Manager renders the immersive media, which includes the registration of the AR content into the real world accordingly.

### 6.5.6	Content formats and codecs

Based on the use cases, the following formats, codecs, and packaging formats are of relevance for AR conversational:

-	General

>	2D Video Formats and video compression codecs

>	Regular Audio Formats and audio compression codecs

Editor’s Note: regular audio to be defined

-	In addition, for downlink

>	Immersive media 3D Formats such as static and dynamic point clouds or meshes

>	Spatial Audio Formats

>	Decoding tools for such formats

>	Composed Scene Graph and Scene Description

-	In addition, for uplink

>	Immersive media 2D Video Formats with depth

>	Immersive media 3D Formats such as static and dynamic point clouds or meshes

>	Spatial Audio Formats

>	Encoding tools for such formats

>	Streaming of sensor information (e.g., gyroscope, accelerometer) as well as pose information

NOTE 1:	Details on uplink delivery of immersive media 3D formats are for further study, to take into account the specific latency requirements of each conversational use case.

NOTE 2:	It is not necessary to support all media formats listed, depending on the device type and/or application.

### 6.6.3	Basic architecture

To describe the functional architecture for shared AR conversational experience use case such as clause Annex A.7 and identify the content delivery protocols and performance indicators, an end-to-end architecture is addressed. The end-to-end architecture for shared AR conferencing (one direction) is shown in Figure 6.6.3-1. To simplify the architecture, only 5G STAR UE is considered in this figure.

Figure 6.6.3-1: Shared AR conversational service for STAR UE.

Editor’s Note: microphones missing in this figure

Camera(s) are capturing the participant(s) in an AR conferencing scenario. The camera(s) for each participant are connected to a UE (e.g. laptop or mobile phone or AR glasses) via a data network (wired/wireless). Live camera feeds, sensors, and audio signals are provided to a UE which processes, encodes, and transmits immersive media content to the 5G system for distribution.  In multi-party AR conversational services, the immersive media processing function on the cloud/network receives the uplink streams from various devices and composes a scene description defining the arrangement of individual participants in a single virtual conference room.  The scene description as well as the encoded media streams are delivered to each receiving participant. A receiving participant’s 5G STAR UE receives, decodes, and processes the 3D video and audio streams, and renders them using the received scene description and the information received from its AR Runtime, creating an AR scene of the virtual conference room with all other participants.

Also note that if the format conversion is desired, the immersive media processing function on the cloud may optionally use media services such as pre-processing of the captured 3D video, format conversion, and any other processing before compression of immersive media content including 3D representation, such as in form of meshes or point clouds, of participants in an AR conferencing scenario.

NOTE:	As an example of the composite scene generation, the immersive media processing function may take the input from the participants’ physical constraints, so that the generated scene is consistent with every participants’ environment and can be rendered at each device consistently.

Figure 6.6.3-2 illustrates the architecture for shared AR conversational experience use case when an 5G EDGAR UE (receiver) is used. While the functionalities of the sender and the network/cloud shown in Figure 6.6.3-1 are identical in the STAR and EDGAR devices, an EDGAR device uses a split-rendering function on Cloud/Edge.

Figure 6.6.3-2: Shared AR conversational service for EDGAR UE and cloud/edge pre-rendering.

Editor’s Note: microphones missing in this figure

The AR session management may be done by AR/MR application on device. In this case, it is a responsibility of the device to connect and to acquire an entry point for edge/cloud during the session management. AR Scene Manager on cloud/edge generates the lightweight scene description and simple format of AR media that match AR glass display capabilities of the individual participant’s 5G EDGAR device. The lightweight scene description and encoded rendered scene are delivered to the UE. The UE receives the simple format of AR media and audio streams, decodes and renders them using the received lightweight scene description and the information received from its AR Runtime, creating an AR scene of the virtual conference room with all other participants.

### 6.6.6	Content formats and codecs

Based on the use cases, the following formats, codecs and packaging formats are of relevance for Media Streaming of AR:

-	Scene Graph/Description

-	2D Video Formats

-	3D Formats such as static and dynamic point clouds or meshes

-	Animated 3D meshes

-	2D Video Formats with depth

-	2D, stereo, and spatial audio formats

Editor’s Note: 2D seems to be superfluous here for audio

-	Several video decoding instances

-	Decoding tools for such formats

-	Encoding tools for 2D formats

-	Low-latency downlink and uplink real-time streaming of the above media

-	Uplink streaming of pose information

-	Uplink streaming of media

# 7	Considerations on Devices Form-factor

## 7.1	General

The components of AR glasses are same or similar with those of mobile phones which may launch and execute AR/MR applications. However, AR glasses have rather different requirements and limitations compared with mobile phones.

From a form factor perspective, AR glasses have several different design considerations. For example, AR glasses have two separate see-through displays for each eye. They also usually include more than two vision cameras which are spatially separated in order to achieve better disparity for depth estimation. In addition, AR glasses are worn and closely attached to a user’s face and contain IMU sensors to estimate where the user’s focal point is. Most of the included components are designed and placed in order to meet requirements which differ to those for mobile phones.

From a media processing perspective, AR/MR applications consume far more energy than non-AR/MR applications [27]. Multiple, as well as different types of cameras are always turned on to track the features detected in 2D and 3D video every second. In the case when AR/MR objects are augmented into the real world, the objects need to be rendered frame by frame with different view frustum positions and directions. In the case when the AR/MR objects are rendered in a server, the AR/MR device is expected to upload the user's pose in a millisecond frequency, then download, decode, correct, and composite the pre-rendered image sequences streamed from the server.

Besides, from an ergonomics perspective, restrictions need to be considered to place the components of the AR glasses in a limited space and under the manageable range of user neck joint torque.

This clause addresses form-factor related issues from the components of AR glasses device architectures, such as battery/power consumption, camera, display, heat dissipation, and weight.

## 7.2	Battery/Power consumption

The run time of a typical battery is proportional to its physical size, capacity, and weight, while they are proportional to user discomfort and neck torque. A study on the characteristics of AR applications [27] measured battery consumption of commercially available applications on AR, streaming, and social networking which shows that AR applications consume around at least 46% more energy than non-AR applications. The capacity of the battery needs to be designed to support a fair amount of running time for the everyday use of AR/MR applications. The amount of running time could be from tens of minutes for shopping of products via AR remote advertising in Annex A.2, 1-2 hours for streaming of volumetric video in Annex A.3, or even several hours for AR gaming in Annex A.6. However, as capacity is typically proportional to weight, and as the AR glasses is expected to be worn and equipped under the consideration of human ergonomics such as neck strain, there are clear limitations on extending the capacity of the battery. Such limitations may be relaxed by dynamically offloading some energy-intensive workloads to 5G cloud/edge. In this case, local processing power consumption is exchanged with power consumption for 3GPP/non-3GPP connectivity and an always on connectivity as well. For connectivity Discontinuous Reception (DRX) and Reduced Capability (RedCap) may be one of examples looking for lower power consumption for the radio for AR/MR application.

The following KPI is related with battery and power consumption and listed in clause 4.5.2.

-	Maximum Available Power

## 7.3	Camera

Augmented reality may be realized by SLAM. To understand the physical world through SLAM, various types of multiple cameras need to be continuously turned on and always need to be acquiring image sequences.

Among the various components contributing to heat, such as CPU, GPU, camera and display, it is measured that the cameras are one of major sources of heat dissipation for AR applications [27]. AR/MR applications may need to be aware of the available run time remaining, and the amount of heat dissipation felt by the user.

In addition, as multiple cameras may be equipped in AR glasses for various purposes, they need to be designed and placed optimally to process the required functions in AR Runtime. Camera related parameters, such as for calibration, pose correction, Vision Engine, SLAM etc. are expected to have a big impact on the quality of service for AR glasses. AR/MR applications may need to be aware of intrinsic and extrinsic parameters for the cameras to properly process the required functions. Such parameters may be delivered to the server whenever there is any change in camera configurations.

The following KPI is related with camera and listed in clause 4.5.2

-	Maximum Available Power

## 7.4	Display

There is at least one display for each eye on a pair of immersive AR glasses. The AR glasses estimates the position of each eye then presents pixels of the rendered AR/MR objects on the display in order to combine the ray of light reflected from the surface of real-world objects with each pixel. A renderer in the AR scene manager may take into consideration the shape and optical distortion characteristics of the displays, pixel arrangements, and the estimated position of each eye of the user. At least one of the view frustum models that represents either an AR glasses, each display, or each eye, with a 3D map of the surroundings may be provided to the AR scene manager in order to minimize the post processing of customizing a generic rendered image to fit to a certain pair of AR glasses.

The following KPI(s) are related with display and listed in clause 4.5.2.

-	Maximum Available Power

-	Persistence – Duty time

-	Display refresh rate

-	Spatial Resolution per eye

-	Content frame rates

-	Brightness

-	Field of View

-	Eye Relief

-	Calibration

-	Depth perception

## 7.5	Heat dissipation

It has been studied that AR applications may generate 4-5 degrees (in Celsius) higher heat than non-AR applications on the same device [27]. Another study shows that a user’s heat sensation and discomfort increase with temperature. Overheated components have not only degraded performance but also power leakage through thermal throttling [28].

The following KPI may be related with heat dissipation and listed in clause 4.5.2.

-	Maximum Available Power

## 7.6	Weight

AR glasses consists of displays, sensors, cameras, batteries and so on. The weight of AR glasses puts constant pressure on a user’s skin and changes the amount of torque applied to the neck joints and muscles in a neutral posture.

A study shows that a user’s posture may be changed from a neutral to a look-up posture, a look-down posture, or a body-bending posture because of the relative placement of virtual objects [29]. Those different postures increase the moment arm between the Centre of Mass (CoM) of the wearable device and the neck joint.

There are different characteristics between HMD type and glasses type devices, as the CoM of glasses type devices is biased towards the front of the device, by design. As a result, AR/MR applications need to consider the issues due to the differences in the ergonomics between the two different types of wearable devices.

The following KPI is related with weight and listed in clause 4.5.2.

-	Maximum Weight

Editor’s Note: subclause for audio to be inserted?

## 8.1	General

This clause documents and clusters potential new work and study areas identified in the context of this Technical Report. In particular, two areas have been identified as crucial for supporting AR type of services and applications that impact network and terminal architectures:

-	5G Generic Architecture for Real-Time Media Delivery as introduced in clause 8.2.

-	Support for Media Capabilities for Augmented Reality Glasses as introduced in clause 8.5.

In order to separate the work areas of these potential work topics, Figure 8.1-1 and Figure 8.1-2 provides the high-level scope of these two work topics for STAR-based and EDGAR-based UEs, respectively.

Figure 8.1-1: Work topic separation between AR media capabilities, terminal architecture and network architecture for STAR-type devices.

Editor’s Note: microphones missing in this figure

Figure 8.1-2: Work topic separation between AR media capabilities, terminal architecture and network architecture for EDGAR-type devices.

Editor’s Note: microphones missing in this figure

## 8.9	Audio Media Pipelines for AR Experiences

Editor’s Note: this subclause should be replaced after completing the study on audio aspects

The current focus of this document is on general system aspects, especially targeting visual rendering on glasses. As such it may lack accuracy on audio media type. For example, extrapolations on architectural aspects derived for primarily visual media pipelines to audio pipelines need confirmation and further considerations. In particular, the following aspects may require further study:

-	In device functional architecture, the type of audio capture and playback and the related system integration need to be defined.

-	In 5G AR device types, the functional structures identified in this TR may be differentiated for immersive media types, e.g. operating immersive audio standalone while immersive video functions are split, involving tethered and/or cloud/edge entities.

-	In the 5G system architecture mapping, the split of codecs and rendering assumed for video may not be appropriate for audio.

# 9	Conclusions

AR/MR experiences involve augmenting visual/auditory contents into the real world to improve the user’s experience with better immersiveness, unlike VR, which provides an entirely virtual world. To realize these experiences, glass-type AR/MR devices may be a good candidate device, easily combining the lights from the real world and those from the display without a need of holding a device in one’s hand.

In this study, the generic finding for eXtended Reality (XR) in TR 26.928 [2] have been further analysed with specific focus on Augmented Reality (AR) experiences and in particular also with a new device type, AR glasses. Different device centric functions of AR glasses are defined, and different device types are defined. Of particular relevance are 5G STandalone AR (STAR) UEs, i.e. devices that have sufficient capabilities to render rich AR experiences on the device as well as 5G EDGe-Dependent AR (EDGAR) UEs for which edge-based rendering support is a must to provide rich AR experiences. Three basic functions are introduced, the AR Runtime, the Scene Manager and the 5G Media Access Function. Basic AR processes are defined, and a comprehensive summary of AR related media formats is provided. The relevant work in external organizations is summarized.

Based on core use cases, different scenarios are mapped to the 5G System architecture, namely (i) Immersive media downlink streaming (ii) Interactive immersive services (iii) 5G cognitive/spatial computing immersive services as well (iv) AR conversational services. Potential normative work is identified and summarized in clause 8.

Based on the details in the report, the following next steps are proposed.

In the short-term:

-	Document the relevant 5G generic architecture for real-time media delivery based on the 5GMS architecture as addressed in clause 8.2.

-	Establish the concept of 5G media service enablers as introduced in clause 8.3 and make use of the concept to define relevant AR media service enablers.

-	Define a 5G real-time communication media service enabler to support different low-latency streaming and conversational AR related services based on the considerations in clause 8.4.

-	Define media capabilities for AR glasses in a service-independent manner based on the considerations in clause 8.5. The outcomes may affect the other items, especially the 5G real-time communication media service enabler and the IMS-based conversational services.

-	Based on the work on above, define a split rendering media service enabler to support EDGAR devices, as addressed in clause 8.6.

-	Study options for smartly tethering AR glasses based on the discussion in clause 8.7.

-	Develop the extension of IMS-based AR conversational services and shared AR experiences, including an extended MTSI terminal architecture, as addressed in clause 8.8.

-	Complement this TR with the relevant audio aspects in a follow-up study based on the considerations in clause 8.9.

Editor’s Note: this bullet should be replaced after completing the study on audio aspects

In the mid-term:

-	Add issues around semantical perception and spatial mapping to an AI/ML study, taking into account the findings in clause 4.2.3 and 4.2.5 as well as TR 22.874.

All work topics will benefit to be carried out in close coordination with other groups in 3GPP on 5G System and radio related matters, edge computing and rendering as well in communication with experts in MPEG on the MPEG-I project as well as with Khronos on their work on OpenXR, glTF and Vulkan/OpenGL. A follow-up workshop based on the information in clause 4.6.9 may be conducted in order to explore additional synergies and complementary work in different organizations in the XR/AR domain.

# A.1	Use Case 16: AR remote cooperation

Use Case Name

AR remote cooperation

Description

As described in Annex A.9 of 3GPP TR26.928[2], a remote expert makes AR actions (e.g. overlaying graphics and drawing of instructions) to the received local video streams. This use case highlights that both parties can share their own video streams and overlay 2D/3D objects on top of these video streams compared with the scenario from TR 26.928. For example, a car technician contacts the technical support department of the car components manufacture by phone when he has some difficulty in repairing a consumers' car. The technical support department can arrange an engineer to help him remotely via real-time communication supporting AR. The car technician makes a video call with the remote engineer, uses his camera to capture the damaged parts of the car and shares them with the remote engineer in-call. And he marks possible points of failure by drawing instructions on the top of these video contents in order that the remote engineer can see the marks and make a detailed discussion. Also, they have respectively FOVs on their sides to check the failure. Likewise, the remote engineer can also overlay graphics and animated objects based on these shared video contents to adjust or correct the technician's operations. Furthermore, if the maintenance procedures are complex, the remote engineer can show the maintenance procedures step by step which are captured in real-time to the local technician. Therefore, the local technician can follow the operations. Finally, they find out the problems and fix them. It looks like that the remote engineer is beside the technician, discusses and solves the problems together. In the extension to this use case, it the remote engineer enables front-facing and back-facing cameras at the same time, the car technician can see a small video stream, which is captured by the front-facing camera of the remote engineer to achieve more attentive experiences.

Categorization

Type: AR, MR Degrees of Freedom: 3DoF+, 6DoF Delivery: Interactive, Conversational Device: XR5G-P1, XR5G-A2, XR5G-A3, XR5G-A4, XR5G-A5, others

Preconditions

<provides conditions that are necessary to run the use case, for example support for functionalities on the end device or network> Both parties on the device with the following features - Support for conversational audio and video - Collect and delivery of AR actions and viewer information - Enabling of the front-facing and back-facing cameras at the same time The network with the following features - Rendering of overlying AR actions and viewer information - Rendering of virtual and real superposition of different video contents

Requirements and QoS/QoE Considerations

<provides a summary on potential requirements as well as considerations on KPIs/QoE as well as QoS requirements> QoS: - conversational QoS requirements - sufficient bandwidth to delivery compressed 2D/3D objects QoE: - Synchronized rendering of overlay AR actions and pose information - Synchronized rendering of audio and video - Fast and accurate positioning information

Feasibility and Industry Practices

<How could the use case be implemented based on technologies available today or expected to be available in a foreseeable timeline, at most within 3 years? - What are the technology challenges to make this use case happen? - Do you have any implementation information? - Demos - Proof of concept - Existing services - References - Could a reduced experience of the use case be implemented in an earlier timeframe or is it even available today? > Enhancements in media processing for multiple video streams both from different parties and/or the same party together with all kinds of AR actions may be performed in the network (e.g. by a media gateway) and in order to enable richer real-time experiences. Accordingly, the extensive hardware capabilities (e.g. multi-GPU) are required.

Potential Standardization Status and Needs

<identifies potential standardization needs> - MTSI regular audio and video call between both parties - Standardized format for AR actions (e.g. static and/or dynamic 2D/3D objects) and posture information - Delivery protocols for AR actions and posture information - Rendering of more than one video stream

Editor’s Note: regular audio to be clarified