3GPP TSG SA WG4#120e	S4-220951

E-meeting, 17th – 26th August 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0034 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMSA_Ph2] 5GMS over 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMSA_Ph2 |  |  |  |  |  | Date: |  |  | 11/08/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The work item in SP-220614 asks among others for the following: 3. 5GMS over 5MBS: - Adding call flows and procedures to support carriage of 5GMS streaming sessions over 5MBS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The CR addresses the above objectives by adding - Architecture for 5GMS via MBS - Reference Points |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Item objectives not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This a starting point by re-using the specification for 5GMS via eMBMS and needs work and discussion. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[X]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[Y]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

=====  CHANGE =====

## 4.X	5G Downlink Media Streaming via MBS

### 4.X.1	Architecture for 5G Downlink Media Streaming over MBS

Figure 4.X.1-1 below depicts an architecture for downlink 5G Media Streaming via MBS as defined in TS 26.502 [X] that combines the functions and reference points of the 5GMS System with those of the MBS System.

Figure 4.X.1-1: Architecture for 5G Media Streaming over MBS

This arrangement allows 5GMS-based downlink media streaming to be deployed as an MBS-Aware Application on top of MBS as defined in TS 26.502 [X].

In this case:

- The 5GMSd AF configures the delivery of 5GMSd content to an MBS Client in the UE by provisioning an MBS User Service as defined in clause 4.5.1 of TS 26.502. In order to additionally deliver this content over one or more MBS Distribution Session, the 5GMSd AF invokes Nmb10 control plane procedures on the MBSF as specified in clause 7.2 of TS 26.502 and, as a result, content is ingested by the MBSTF from the 5GMSd AS using the pull-]based object ingest procedures at reference point Nmb8 specified in clause 6.1 of TS 26.502using pull-]based object ingest.

NOTE 1: Push-based ingest is not enabled as it would unnecessarily modify the 5GMS AS operation.

NOTE 2: 5GMSd AS and MBS AS share similar functions and may be deployed in a single physical location.

2.	The 5GMSd Client acts as MBS-Aware Application (as defined in TS 26.502 [X]) for the MBS Client. Thus, the MBS Client is controlled by the 5GMSd Client via the MBS-6 API (as defined in TS 26.517 [Y]).

3.	The MBSTF Client receives media and other objects from the MBSTF according to the Object Distribution Method specified in clause 6.1 of TS 26.502 [X]. If an uplink is available to the MBS Client, and if Object Repair as specified in clause 4.2.6 of TS 26.502 [X] is available in the MBS System, the MBS Client uses the unicast repair delivery procedures to recover damaged media objects received from the MBSTF by interacting at reference point MBS-4-UC.

4.	The Media server subfunction of the MBSTF Client provides a media server interface and shall expose the content received (and possibly repaired) by the MBS Client to the 5GMSd Client via the HTTP client-to-application interface. (This interaction is labelled MBS-7 in figure 4.X.1-1 above.)

5.	The Media Player sends requests according to the signalled object availability times in the manifest. In case a media object transmitted via an MBS Distribution Session is not received by the MBS Client by the object availability times, or if it cannot be repaired in time for consumption by the 5GMS Client, the Media Server in the MBS Client returns an error or a partial object in response to the Media Player's request for the media object, and the Media Player may instead attempt to retrieve the media object, or ranges of it, from the 5GMSd AS at reference point M4d, if available. The object shall be available for download from the 5GMSd AS for a well-defined time duration.

NOTE 3:	Details on determining the availability time requirements of the application are deferred to stage 3.

The usage of existing reference points to support these scenarios is documented in the following clauses. Procedures for 5GMS via MBS are defined in clause 5.X.

### 4.X.2	Usage of 5GMS reference points for MBS-based delivery

#### 4.X.2.1	Usage of M1d

Reference point M1d is used as defined in clauses 4.1 to 4.4.

In addition, the content provider shall authorize via M1d that 5GMS content may be distributed via MBS.

The translation of M1d information to MBS delivery provisioning is left to implementation.

NOTE:	The 5GMS Application Provider may provision specific use-cases (high velocity, specific reception area, indoor/outdoor/mobile users) at reference point M1d. These service requirements are translated by the 5GMSd AF into specific Nmb10 calls to provision the MBSF with a service that has the correct parameters for a specific location.

#### 4.X.2.2	Usage of M2d

Reference point M2d is be used as defined in clauses 4.1 to 4.4.

#### 4.X.2.3	Usage of M3d

Reference point M3d is used as defined in clauses 4.1 to 4.4.

#### 4.X.2.4	Usage of M4d

Reference point M4d is used as defined in clauses 4.1 to 4.4.

#### 4.X.2.5	Usage of M5d

Reference point M5d is used as defined in clauses 4.1 to 4.4.

In addition, for 5GMS content to be distributed via MBS:

-	The 5GMS Service Access Information shall include the relevant information of the MBS Service Announcement in order to bootstrap reception of the MBS service, typically via a service identifier (i.e., the serviceId attribute of the bundleDescription.userServiceDescription element of the USD - see TS 26.517 [Y]). This is passed by the Media Session Handler to the MBS Client via reference point MBS-6.

When this information is present in the Service Access Information and when the UE is MBS-capable, the 5GMSd Client shall invoke the MBS Client to initiate reception of the corresponding MBS User Service.

-	The 5GMS Service Access Information shall include relevant information from the MBS Service Announcement in order for the Media Session Handler to:

i)	Collect metrics of the MBS User Service from the MBS Client and report them to the 5GMSd AF using an appropriate metrics reporting scheme.

ii)	Collect media consumption information from the MBS Client and submit it to the 5GMSd AF in 5GMS consumption reports.

#### 4.X.2.6	Usage of M6d

Reference point M6d is used as defined in clauses 4.1 to 4.4.

#### 4.X.2.7	Usage of M7d

Reference point M7d is used as defined in clauses 4.1 to 4.4.

#### 4.X.2.8	Usage of M8d

Reference point M8d is used as defined in clauses 4.1 to 4.4.

### 4.X.3	Usage of MBS reference points and interfaces

#### 4.X.3.1	Usage of Nmb10

The 5GMSd AF provisions MBS User Services in the MBSF as defined in clauses 5.3 of TS 26.502 [X].

#### 4.X.3.2	Usage of Nmb8

The MBSTF ingests content from the 5GMSd AS using the pull-based ingest method.

#### 4.X.3.3	Usage of MBS User Services and Distribution Methods

The MBS User Service Announcement as defined in TS 26.502 is used to advertise the availability of 5GMS content delivered via MBS.

A Generic application service (as defined in clause 5.7 of TS 26.346 [19]) is provisioned in the BM-SC and the application service entry point instance is a downlink 5GMS streaming manifest, for example a DASH MPD or HLS playlist.

NOTE:	The support of multiple manifests for the same media streaming session is not covered in 5GMS. Hybrid DASH/HLS is supported in eMBMS. Usage together with 5GMS is left to implementation.

#### 4.X.3.4	Usage of MBS-6

The MBSF Client operates according to the procedures defined in clause 6.3 of TS 26.347 [20] at reference point MBMS-API-C when communicating with the 5GMSd Client.

The MBSF Client exposes information to the Media Session Handler to manage the reception of MBS User Services.

The Media Session Handler configures the MBSF Client for reception reporting.

The MBSF Client provides reception reports to the Media Session Handler.

#### 4.X.3.5	Usage of MBS-7

The MBSTF Client operates according to the procedures defined in clause 7 of TS 26.347 [20] at reference point MBMS-API-U when communicating with the 5GMSd Client.

The MBSTF Client provides the streaming manifest, as well as updates of the manifest, to the 5GMSd Client and implements policies for hybrid services based on clause 7 of TS 26.347 [20].

The MBSTF Client exposes fully- and partially-received media objects to the Media Player in the 5GMSd Client.