Source:		Qualcomm Inc., Lenovo

Title:	Media type and metadata categories and characteristics

Agenda Item:	9.5

Document for:	Discussion and Agreement

# Introduction

At the 3GPP-SA4#120-e, the Permanent Document for MeCAR v3.0.0 [1] was approved. The Permanent Document specifies AR/MR media type definition as below.

- Scene Description

- Spatial Description

- 3D Model

- AR Anchor

- User Pose

- FOV

- Viewport

- Gesture

- Body action

- Facial expression

- Sensor information

- Camera information

- Projection information

Some media types indicate the device capabilities, some media types are the description of the media content consumed by the UEs, and some media types are the user inputs or haptics.

This contribution proposes to use “metadata” instead of “media type” in the PD as “media type” is usually defined for a specific media content consumed by the UEs such as audio, video, image, text, message, 3D graph and volumetric data.

# Metadata categories and characteristics

Based on the metadata definition and the description specified in MeCAR PD, the AR/MR metadata may be classified into 3 categories:

- Device capability: the metadata specifying the device capabilities and features such as camera information and projection information. The information is usually available at the beginning of an AR/MR session and may not change within an AR/MR session.

- Media description: the metadata describing the space or object media content such as scene description, spatial description, and 3D visual mode to be consumed by the UEs. The data size may be large (>10MB) and the information usually does not change frequently and may be event driven.

- Interaction: the metadata representing the user inputs or haptics such as user pose, viewport, gesture, body action, facial expression, FOV and AR anchor point. The interaction may trigger the event at the receiver side and the sender may expect low-latency response (50~1000ms). The interaction data size may be small, but the frequency could be high (>1KHz). The data transport may be continuously or may require burst transport. Depending on the applications, the interactive metadata may be synchronized to each other, or synchronized to other media streams, and the reliability may be strict or not strict.

Table 1 is the updated AR/MR metadata definition in PD subclause 3.7.2.

Table 1 – AR/MR Metadata definitions

| AR/MR category | Metadata | Definition | Media Type Description (Examples) |
| --- | --- | --- | --- |
| Device capability | Camera information | TBD | Type: The camera parameters such as focal length, principal points, calibration parameters and the pose of the camera all contribute in understanding the relevance between points in the volumetric scene and pixels in the captured image. Organization: None |
|  | Projection information | Clause Y.3 of 3GPP TS 26.114 and H264 protocol Parameters associated to the projection from the 3D scene to the user display | Type: Projection mapping information (indicating the projection format in use, e.g., Equirectangular projection (ERP) or Cubemap projection (CMP)), for the projection sample location remapping process as specified in clauses 7.5.1.3 and 5.2 of ISO/IEC 23090-2 [179] Organization: None |
| Media description | Scene Description | Clause 4.4.2 of 3GPP TR 26.998[1] Scene description is used to describe the composition of a 3D scene, referencing and positioning the different 2D and 3D assets in the scene typically using a tree or a graph structure. | Type: glTF2.0, JSON format Organization: MPEG-I and Khronos |
|  | Spatial Description | Clause 4.4.7.3 of 3GPP TR 26.998[1] Visual features, keyframes, and spatial maps are used for mapping the real world, typically as part of the SLAM process. | Type: For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_FB_spatial_entity Organization: OpenXR |
|  | 3D Visual Model | Clause 4.6.3.5.2 of 3GPP TR 26.928[2] 3D visual object description as a list of vertices, faces and other elements, along with associated attributes. | Type: PoLYgon Organization: None |
| Interaction | AR Anchor | The AR anchor is meant to identify a point in the user space to be used to anchoring a visual object (2D or 3D) | Type: Metadata allowing accurate overlaying/rendering of text, graphics or video contents to support Use Case 8 of TR 26.928. Organization: None |
|  | User Pose | Clause 4.4.3.1 of 3GPP TR 26.998[1] Representation of the user position and orientation | Type: It consists of a quaternion for orientation and a 3D vector for position. Timestamp is represented by a 64 bit monotonically increasing nano-second-based integer. Organization: Khronos OpenXR |
|  | FOV | Y.6.2.3 of 3GPP TS 26.114[3] The Field of View (FOV) is the extent of observable world at any given moment | Type: It consists of vertical fov and horizontal fov. Organization: None |
|  | Viewport | Y.7.2 of 3GPP TS 26.114[3] The viewport corresponds to the projection of the user View onto a target display | Type: It shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range Organization: None |
|  | Gesture | TBD | Type: A array of finger joint position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hand_tracking Organization: OpenXR |
|  | Body action | TBD | Type: bvh format. Frequency: at least 1kHz Organization: BioVision company |
|  | Facial expression | OpenXR | Type: An array of key point position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XrSystemFacialTrackingPropertiesHTC Organization: None |
|  | Sensor information | OpenXR | Type: a new interaction profile path For example, OpenXR EXT format offers the possibility to developers to integrate and benefit from new controllers and sensor subsystems, (e.g., https://registry.khronos.org/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hp_mixed_reality_controller, etc.). Organization: None |

Table 2 illustrates the characteristics of each AR/MR metadata category.

Table 2 AR/MR Metadata category characteristics

|  | Device capability | Media description | Interaction |
| --- | --- | --- | --- |
| metadata | camera information, projection information | Scene description, spatial description, 3D visual mode | FOV, AR anchor, user pose, viewport, gesture, body action, facial expression |
| Data size | Small | Large (>10MB) | Small |
| Frequency | Beginning of the session | Low (event based) | High (>1KHz) |
| Real-time | Sparsely timed (event based)/Non timed | Sparsely timed (event based)/Non timed | Timed |
| Latency |  |  | 50~1000ms |
| Reliability | reliable | reliable | reliable/unreliable |
| Synchronization | N | Y | Y |

# Real-time Metadata transport

The device capability category metadata is usually static and available in advance. The metadata may be exchanged using SDP [2] framework for immersive real-time communication or IMS-based AR conversation stream respectively.

The media description category metadata may be transported over RTP [3] given its size and characteristics. The RTP payload type may indicate the specific media type such as scene description, spatial description, or 3D visual model. The assignment of an RTP payload type may be performed in a dynamic way and indicated in SDP media description lines. The RTP payload format may be refer to the corresponding IETF RFCs.

The real-time interaction metadata may be transported over RTP [3] or WebRTC data channel [4], the candidate solutions are to be explored in 5G_RTP and IRTCW.

# Proposal

It is requested to include clause 2 and 3 of this document in the MeCAR permanent document and take them into account in the discussion of MeCAR, iRTCW and 5G_RTP related topics.

# References

- 3GPP TSG SA WG4 S4-221150, “MeCAR Permanent Document v3.0”, August 2022

- IETF RFC4566, “SDP: Session Description Protocol”, July 2008

- IETF RFC3550, "RTP: A Transport Protocol for Real-Time Applications", July 2003

- IETF RFC8831, "WebRTC Data Channels", January 2021