Agenda item: 	10.7

Source: 	InterDigital Communications

Title: 	[GA4RTAR] Edge-enabled 5G RTC Architecture for split rendering and spatial computing

Document for:	Discussion and Agreement

# Introduction

This contribution addresses one of the objectives of the GA4RTAR work item, namely:

“Specify support for AR relevant functionalities such as split-rendering or spatial computing on top of a 5G System based on this architecture”.

In this contribution, we discuss a 5G RTC architecture extension to support edge-based processing for AR functionalities such as split rendering and spatial computing in the GA4RTAR technical specification.

# 5G RTC Architecture extension

  - Introduction

This section defines an architecture that enables a 5G RTC Application Provider to provision resources in the Edge Data Network (EDN) for an application through the M1 interface.

Media processing in the edge may be achieved in one of two different ways at the application layer:

1.	Client-driven management. 5G RTC Applications that are aware of the edge processing can directly request an edge resource and discover the Edge Application Server (EAS) that is best suited to serve the application.

2.	Application Provider-driven management. The 5G RTC AF automatically allocates edge resources for new streaming sessions on behalf of the application using information in the 5G RTC provisioning session.

An Edge-enabled 5G RTC Client leverages the Edge Computing capabilities as defined in TS 23.558.

  - Extended 5G RTC architecture for Edge Computing

The 5G RTC architecture can be extended to add support for media processing in the edge. The extended architecture is an integration of the 5G RTC architecture defined in TS 26.506 with the architecture for enabling Edge Applications defined in TS 23.558. The extended architecture is as shown in Figure 1.

Figure 1. Edge-enabled 5G RTC architecture

    - Edge Application Server (EAS)

EAS is the application server resident in the EDN, performing edge-based processing for AR functionalities such as split rendering and spatial computing. The Application Client (AC) connects to the EAS in order to avail the services of the application with the benefits of Edge Computing.

It is possible that the server functions of an application are available only as an EAS.

However, it is also possible that certain server functions are available both at the edge and in the cloud as an EAS and an Application Server resident in the cloud.

The EAS can use the 3GPP Core Network capabilities in the following ways, all of which are optional to support:

a)	invoking 3GPP Core Network capabilities via the edge enabler layer through the Edge Enabler Server (EES)

b)	invoking 3GPP Core Network function (e.g., PCF) APIs directly, if it is an entity trusted by the 3GPP Core Network; and

c)	invoking the 3GPP Core Network capabilities through the capability exposure functions, i.e., SCEF/NEF/SCEF+NEF.

The functions of Edge enabler Client (EEC), Edge Enabler Server (EES), Edge Configuration Server (ECS) are as defined in TS 23.558.

    - Edge Interfaces

Based on the extended architecture, the following interfaces are defined for performing edge-based processing for AR functionalities such as split rendering and spatial computing:

1.	A 5G RTC AF that is edge-enabled shall support EES functionality including:

-	EDGE-1 API for supporting registration and provisioning of EEC functions, and discovery by them of EAS instances.

-	EDGE-3 API towards the EAS function of 5G RTC AS instances.

-	EDGE-6 API for registering with an ECS function.

-	EDGE-9 API for media session relocation.

2.	A 5G RTC AS that is edge-enabled shall support EAS functionality including the EDGE-3 API for registration with the EES.

3.	A Media Session Handler that is edge-enabled should support EEC functionality including:

-	Invoking the EES function using the EDGE-1 API.

-	Invoking the ECS function using the EDGE-4 API.

-	EDGE-5 API exposed to the Application Client.

4.	A WebRTC Application that is edge-enabled shall support Application Client functionality and should invoke the ECS function using the EDGE-5 API.

# Proposal

We propose to agree the updated 5G RTC architecture and interfaces discussed in clause 2 of this document to support edge-based processing for AR functionalities such as split rendering and spatial computing and document them in the GA4RTAR TS.

# References

- 3GPP TS 23.558: "Architecture for enabling Edge Applications".