Source:	Samsung Electronics Co., Ltd.

Title:	Registration of SDP attributes for ITT4RT

Document for:	Discussion and Agreement

Agenda Item:	10.10

- Introduction

This contribution addresses some issues on SDP attributes for ITT4RT in TS 26.114 and propose to work on the correction and the registration of SDP attributes for ITT4RT.

- A list of SDP attributes for ITT4RT

The table in below shows a list of SDP attributes for ITT4RT with their <attribute-value> syntaxes and related clauses. It was found that some of parameters do not have complete ABNF syntaxes and are not shown in their <attribute-value>.

| <attribute-name> | <attribute-value> | Related clauses |
| --- | --- | --- |
| imageseq | PT [SP item_count] | 6.2.11 |
| 3gpp_360video | <payload type> [SP FOV] [SP FOV_CENTER] [SP "Stereo"] [VDP] (VDP = "VDP" [SP SLVL] [SP Projection] [SP PPM] SP viewport_ctrl SP viewport_size) | Overall syntax in Y.6.2.1 FOV and FOV_CENTER in Y.6.2.3 SLVL in Y.6.2.8 Projection in Y.6.2.2 PPM in Y.6.2.4 viewport_ctrl in Y.6.2.5 viewport_size in Y.6.2.7 viewportfb_tirgger in Y.6.2.9 (not shown in the overall syntax) |
| itt4rt_group | rest-group *[" /" rest-group] | Y.6.2.6 |
| 3gpp_overlay | overlay_id SP type SP (sphere_relative_overlay_config/ viewport_relative_overlay_config) [ SP overlay_info] | Overall syntax in Y.6.4.3.1 overlay_id, type and free_overlay (not shown in the overall syntax) in Y.6.4.3.1 sphere_relative_overlay_config in Y.6.4.3.2 viewport_relative_overlay_config in Y.6.4.3.3 overlay_info in Y.6.4.3.4 overlay_overlap_flag in Y.6.4.3.5 (not shown in the overall syntax) |
| 3gpp_360video_replacement | [sphere_relative_overlay_config] | Y.6.4.4 |
| 3gpp_fisheye | [SP fisheye] SP fisheye-img SP maxpack | Y.6.5.2 |
| 3gpp-camera-calibration | PT 1*WSP attr-list | Y.6.6 |
| stitch_group | <mid1> SP <mid2> SP <mid3> … | Y.6.6 |
| no_other_overlays | N/A | Y.6.8.2 |

- Requirements for IANA registration

According to RFC 8866 [1], new attribute registration are accepted according to the "Specification Required" policy of RFC 8126 [2], provided that the specification includes the following information:

- Contact name

- Contact email address

- Attribute name

- Attribute syntax (an ABNF definition of the attribute value <attribute-value> syntax)

- Attribute semantics

- Attribution value (the name of an ABNF syntax rule defining the syntax of the value)

- Usage level (one or more of the following: session, media, source, dcsa and dcsa(subprotocol))

- Charset dependent

- Purpose

- O/A procedures

- Mux Category

- Reference

Based on the above requirements, we may need more work on a complete ABNF syntax, Usage level and Mux Category for each attribute to be registered.

- Proposal

We propose to work on the registration of SDP attributes for ITT4RT.

- References

[1]	IETF RFC 8866 (2022): "SDP: Session Description Protocol", A. Begen, P. Kyzivat, C. Perkins and M. Handley.

[3]	IETF RFC 8126 (2017): "Guidelines for Writing an IANA Consideration Section in RFCs", M. Cotton, B. Leiba and T. Narten