Source:	Dolby Laboratories, Inc.

Title:	Proposal for IVAS Performance Requirements for Scene-based Audio

Document for:	Discussion and Agreement

Agenda Item:	7.3

At SA4-e (AH) Audio SWG post 120-e telco on Oct. 7th, the source provided a proposal for IVAS performance requirements for scene-based audio (Tdoc S4aA220010). The contribution got some support and received a couple of constructive comments. The present contribution tries to address these comments in an updated proposal.

- Discussion of comments received

Comment:

- Not all required bit rates covered, consider planar Ambisonics

Discussion:

The draft design constraints require support of the following bit rates: 13.2, 16.4, 24.4, 32, 48, 64, 80, 96, 128, 160, 192, 256, 384, 512 kb/s. The source believes that the performance requirements can easily be extended to 384 kb/s. For the case of 512 kbps, it is suggested to define as requirement to be not worse than EVS multi-mono where each EVS instance is operated at 128 kbps. A better than requirement at this high bit rate does not seem feasible. Extending the requirements to 16.4 or 13.2 kb/s based on 4xEVS multi-mono can only rely on EVS@7.2 kbps. But it is indeed possible to define planar FOA requirements with 3xEVS multi-mono.

Comment:

- The EVS multi-mono reference with EVS@7.2 and EVS@5.9 kbps offers only WB audio bandwidth while the IVAS codec would offer SWB or FB audio.

Discussion:

The source believes that maximum audio bandwidth is generally a codec design choice that guarantees best possible quality at a given bit rate. Lower audio bandwidth of the EVS multi-mono reference does not invalidate the comparison. If it improves the overall quality, even an IVAS candidate could operate at reduced audio bandwidth. It is thus believed that the requirements at very low bit rates implying mixed bandwidth comparisons are acceptable. However, using an EVS multi-mono reference based on EVS@5.9 kbps may be problematic as it is a variable-rate coding mode. It is therefore suggested to add a note that the low-rate requirements involving that reference should not be tested during selection testing.

Comment:

- There may be a focus on SWB and/or FB only

Discussion:

The source agrees that relevant for the quality assessment of IVAS scene-based audio operation is SWB or FB rather than WB. For WB, it can be assumed that the quality level will essentially follow the trends observed for SWB or FB. Thus, no extra requirements need to be defined for WB. Accordingly, the part of the requirement table containing WB requirements can be removed.
Regarding the question to have quality requirements for SWB or FB scene-based audio, the source believes that this choice should be use case dependent. If the main use case is voice communication that may generally be tested with P.800 tests, SWB operation is likely more relevant. If the main use case is, e.g., streaming of high-quality general audio, which may be tested with experienced listener tests like Mushra, FB operation is likely more relevant. It is thus proposed to keep the suggested requirements with (SWB/FB) but to add a note that the SWB requirement will be tested in case of predominant voice content and that the FB requirement will be tested in case of predominant general audio content.

Comment:

- Ambisonics order

Discussion:

The suggested requirements apply to Ambisonics orders 1-3 of the scene-based audio input. However, it is believed that the question with which Ambisonics order to test the requirement should be use case dependent. If the main use case is voice communication that may generally be tested with P.800 tests, FOA operation is likely more relevant. If the main use case is, e.g., streaming of high-quality general audio, which may be tested with experienced listener tests like Mushra, HOA3 operation is likely more relevant. It is thus proposed to add a note that the requirement will be tested with FOA content in case of predominant voice content and that the requirement will be tested with HOA3 content in case of predominant general audio content. It is further believed that tests with FOA and HOA3 content will adequately characterize the performance of the IVAS candidate against the requirements. Testing of the corresponding HOA2 requirements will thus not be necessary during selection.

- Suggested further updates

Compared to Tdoc S4aA220010, the following further changes are suggested.

- Limitation of bit rates for which a requirement with DTX on is defined should be set to 80 kbit/s for which mandatory DTX operation is available for the multi-mono EVS reference (4xEVS@24.4). A rational for that is that DTX is more relevant for use cases with high-efficiency voice communication and low to medium bit rates.

- It appears likely that voice communication use cases will predominantly rely on headphone rendering rather than rendering over a room loudspeaker system. If the main use case is e.g. streaming of high-quality general audio, room loudspeaker rendering may also be relevant besides headphone rendering. It is further believed that tests with room loudspeaker rendering are more adequate for experienced listeners rather than naïve listeners. It is thus suggested to modify the corresponding editor’s note into a formal note expressing that.

- Due to the high number of requirements and limits of the test budget, it may not be possible to test all requirements during IVAS codec selection. It is suggested to add a corresponding note.

- Implementation of the suggested performance requirement

# Below is the suggested update of the relevant part of Pdoc IVAS-3 (v.0.2.0, S4-221103):

## 6.2 Performance requirements for operation with Scene-Based Audio Content

6.2.1	Scene-base audio (first-order and higher-order Ambisonics)

High-level definition of scene-based audio requirements

The general requirement is that IVAS operated at rate X shall either

- be better than the EVS multi-mono system encoding the FOA component channels (while truncating the higher-order channels), where each EVS instance is operated at the closest bit rate to X/4.

- or be no worse than the EVS multi-mono system encoding the FOA component channels (while truncating the higher-order channels), where each EVS instance is operated at the next higher available EVS bit rate than the EVS bitrate closest to X/4.

Detailed scene-based audio requirements

The following table illustrates corresponding detailed performance requirements for scene-based audio content (***:

| Category | BW(*** | Bitrate (kbit/s) | FER | DTX(* | Requirements(**,(*** |
| --- | --- | --- | --- | --- | --- |
|  | SWB FB | 13.2(**** | All | On/Off | NWT EVS @ 3x 5.9 kbps (WB) (planar FOA) |
|  |  | 16.4(**** |  |  | NWT EVS @ 4x 5.9 kbps (WB) OR BT EVS @ 3x 7.2 kbps (WB) (planar FOA) |
|  |  | 24.4(**** |  |  | NWT EVS @ 4x 7.2 kbps (WB) OR BT EVS @ 4x 5.9 kbps (WB) |
|  |  | 32 |  |  | NWT EVS @ 4x 9.6 kbps (SWB) OR BT EVS @ 4x 8 kbps (WB) |
|  |  | 48 |  |  | NWT EVS @ 4x 16.4 kbps (SWB/FB) OR BT EVS @ 4x 13.2 kbps (SWB) |
|  |  | 64 |  |  | NWT EVS @ 4x 24.4 kbps (SWB/FB) OR BT EVS @ 4x 16.4 kbps (SWB/FB) |
|  |  | 80 |  |  | NWT EVS @ 4x 24.4 kbps (SWB/FB) OR BT EVS @ 4x 16.4 kbps (SWB/FB) |
|  |  | 96 |  | Off | NWT EVS @ 4x 32 kbps (SWB/FB) OR BT EVS @ 4x 24.4 kbps (SWB/FB) |
|  |  | 128 |  |  | NWT EVS @ 4x 48 kbps (SWB/FB) OR BT EVS @ 4x 32 kbps (SWB/FB) |
|  |  | 160 |  |  | NWT EVS @ 4x 48 kbps (SWB/FB) OR BT EVS @ 4x 32 kbps (SWB/FB) |
|  |  | 192 |  |  | NWT EVS @ 4x 64 kbps (SWB/FB) OR BT EVS @ 4x 48 kbps (SWB/FB) |
|  |  | 256 |  |  | NWT EVS @ 4x 96 kbps (SWB/FB) OR BT EVS @ 4x 64 kbps (SWB/FB) |
|  |  | 384 |  |  | NWT EVS @ 4x 128 kbps (SWB/FB) OR BT EVS @ 4x 96 kbps (SWB/FB) |
|  |  | 512 |  |  | NWT EVS @ 4x 128 kbps (SWB/FB) |

(* DTX will be tested for rates up to 80 kbit/s where mandatory DTX operation is available for the multi-mono EVS reference.

(** The multi-mono EVS reference shall be produced by individual EVS coding of the 4 FOA B-format component signals in ACN/SN3D format (while truncating the higher-order component signals).

(*** The SWB requirement will be tested with predominant voice content in naïve listener tests. The FB requirement will be tested with general audio content in experienced listener tests.

(**** These low-rate conditions will not be tested during selection.

Further notes:

The FOA requirement will be tested with predominant voice content in naïve listener tests. The HOA3 requirement will be tested with general audio content in experienced listener tests. The HOA2 requirement will not be tested during selection.

It may not be possible to test all requirements during IVAS selection. The choice of requirements to be tested is made in the IVAS selection test plan IVAS-8a.

The requirements will be tested with binaural rendering over headphones using a suitable reference renderer that will be specified in the selection processing plan IVAS-7a. The requirements may also be tested with rendering over a room loudspeaker system in experienced listener tests with general audio content.