Source:	Dolby Laboratories Inc.

Title:	On send side audio performance assessment for Immersive Audio Systems – review of TS 26.260

Document for:	Discussion

Agenda Item:	7.6

# Introduction

This contribution (partly a resubmission of [5]) reviews TS 26.260 test methods available for send side audio performance testing for Ambisonics audio, highlights potential ambiguities and discusses possible variations.

# Review of Objective Test Methodologies for Assessment of Immersive Audio Systems in the Sending Direction of TS 26.260 [1]

## Diffuse-field send frequency response test using periphonic array (26.260 clause 4.1.1)

This method measures the send frequency response G(f) of the omnidirectional sound field component using a periphonic loudspeaker array. G(f) is measured by evaluating the captured spectrum of a DUT compared to the sound pressure spectrum as captured by a reference diffuse field/random incidence microphone. The test stimulus is decorrelated pink noise simultaneously played over all speakers of the periphonic array.

TS 26.260 Figure 1

The specification is unclear on the following aspects:

- The specification seems to suggest that a measurement is done for the omnidirectional Ambisonics component W but possibly also for signal components after B-format to ESD conversion. If this is the case, it remains unclear how these signal components can be obtained for the reference system with diffuse field/random incidence microphone.
In case the test environment is calibrated and frequency-response equalized at the geometric center of the free-field volume, the test using the reference diffuse field/random incidence microphone could be avoided. Or is the intention to correct for a non-perfect equalization?

- It is also unclear why the yellow marked definitions below are different. It should be the same point?

, the estimated sound pressure magnitude spectrum obtained from a diffuse-field scene-based audio capture and reference synthesis at the geometric center of a free-field volume; and

, the sound pressure magnitude spectrum obtained from a diffuse-field microphone recording the same diffuse field at the origin of a spherical coordinate system.

- The term “Sound field synthesis” might need further clarification; A reader might interpret Figure 1 (reproduced below) as if a physical sound field should be created (green box on the left) from the signal that has already been captured by the UE. If this is the case, it would mean that the sound field captured by the DUT is used for sound field re-synthesis over the speakers of the periphonic array. In that case, it is unclear how the estimated frequency response G(f) characterizes the immersive audio capture capabilities of a DUT. A hypothetical DUT with a perfectly equalized single-channel capture (fed only to the W component, other components nulled) would be indistinguishable from a DUT that additionally correctly captures directional Ambisonics components. An additional concern might be that it is unpractical to physically regenerate the soundfield captured by the UE, and therefore be done only if necessary. What is the necessity or the advantage?

It may also be assumed that the intention is to describe a calculated synthesis. The later text concurs with this, e.g. just analyzing the W component of the B-format signal. In addition, it may be intended to analyze the directional Ambisonics components compared to the reference spectrum.

Summarizing; A clarification on the meaning with “sound field synthesis” in this context would be welcome. (During the discussion at the last Audio SWG telco, it was clarified that the intention is indeed that the sound captured by the UE should be reproduced over loudspeakers in the lab. Nevertheless, further discussions on the rationale and clarifications in the specification would be beneficial.)

- Spherical harmonics are mentioned at the end of 4.1.1.2, without giving a reference.

## Diffuse-field send frequency response test using loudspeaker array and turn table (26.260 clause 4.1.1.4)

It is not clear from 26.260 if also this method measures the send frequency response G(f) of the omnidirectional sound field component. It is here assumed that this is an alternative method with a loudspeaker array and turn table. Like in the first method G(f) is measured by evaluating the captured sound pressure spectrum of a DUT compared to a reference diffuse field/random incidence microphone. The main difference is that the method carries out repeated measurements where in each run a test stimulus is played only from a single speaker at a time. In this method an exponential sweep sine signal is used as test signal rather than pink noise. The procedure further builds upon impulse response measurements from which magnitude spectra of the sound pressure (for DuT and reference microphone) are derived, from which then the send frequency response is calculated.

The specification is unclear on the following aspects:

- Why using a sweep sine test signal rather than pink noise or speech and why taking the detour through impulse response calculation?

- As there are measurements for each of the 2N+12 loudspeaker positions, one G(f) measurement is obtained for each of them. It is not clear how or if a direction independent diffuse-field send frequency response is obtained. If the idea is that the measurements from all directions are averaged, then it is not expressed how exactly this should be done.

- It is not clear if the measurements are done only for the omnidirectional Ambisonics component W or if other components e.g. after B-format to ESD conversion should be considered.

- The connection to Figure 1 is unclear especially on the aspect of the sound field (re-)synthesis.

## Directional response measurement for scene-based audio (26.260 clause 4.1.2)

This method measures the impulse response (or transfer function) of a test device given a pre-defined direction of the captured signal. The method assumes that the test stimulus is played from a loudspeaker at position (θi, ϕi), i=1…L.

It is specified that an exponential sweep sine test signal is used, which may suggest that the Farina method (or similar) should be used [2]. The specification is generic regarding the Ambisonics signal to which it may be applied and makes no particular suggestions. It is unclear in what regards the method is different than the method in 2.2 (Diffuse-field send frequency response test using loudspeaker array and turn table) for each individual loudspeaker position.

# Conclusion and next steps

There is a need for clear test methods for the characterization of UE Ambisonics capture and for specifying requirements, for some key parameters.

Some questions have been raised regarding TS 26.260; on how to interpret the text and what remains to be further clarified, developed and added. Ideas for how to progress have been sketched out.

It is at this point suggested to

- Discuss until a clear understanding of 26.260 within the SA4 Audio SWG is obtained

- Document such understanding and draft a CR that removes the ambiguities

# References

[1]	3GPP TS 26.260: Objective test methodologies for the evaluation of immersive audio systems

[2]	A. Farina: Simultaneous Measurement of Impulse Response and Distortion with a Swept-Sine Technique, Audio Engineering Society Convention Paper 5093, (2000 February)

[3]	S4-220729: On ATIAS acoustic performance testing for FOA audio, Dolby Laboratories, Inc.

[4]	S4-191167: Description of the IVAS MASA C Reference Software, Nokia Corporation

[5]	S4aA220018: On send side audio performance assessment for Immersive Audio Systems, Dolby Laboratories Inc.