Agenda item: 	10.6

Source: 	Qualcomm Inc.

Title: 	IMS4AR Call Flow

Document for	Discussion and Agreement

# Introduction

In this contribution, we provide and discuss a call flow for the establishment and operation of IMS-based AR telephony applications based on the architecture considerations documented in TR23.700-87[1].

# IMS-Based AR Call

The following call flow diagram describes the establishment and operation of an IMS-based AR call.

The AR call follows the following steps:

- The calling UE, in this case UE1, sends a SIP invite to the P-CSCF to initiate the call

- The P-CSCF identifies this as an AR call and forward the invite request to the AR IMS AS

- The AR IMS AS sends a request to the DCSF to request the data channel resources for the AR call data channel application.

- The DCSF selects a DCMF and requests the allocation of data channel resources and the execution of the AR call data channel application

- The DCMF confirms the successful allocation of resources and the DCSF forwards the confirmation to the AR IMS AS

- The AR IMS AS then forwards the invite to UE2 via the P/S/I-CSCF

- UE2 accepts the invitation and informs UE1

- Both UE1 and UE2 establish a data channel connection to the DCMF and retrieve the scene description entry point for the AR call

- UE1 and UE2 send pose and user action information to the DCMF

- The DCMF updates the scene description for the AR call based on the latest pose information

- The DCMF sends scene description updates to both endpoints

- UE1 and UE2 may send their media streams to an ARMF for processing

- The ARMF will process and share the resulting media with the other endpoint

- Other media streams of the session are exchanged end to end

NOTE: split rendering for UE1 and/or UE2 needs to adhere to the procedures and formats that are defined by the SR_MSE work item. The ARMF may be the entity that performs the split rendering for the UE.

# Proposal

We propose to agree the call flow and use it as baseline for further work on IMS-based AR calls.

# References

[1]		3GPP TR23.700-87: “Study on system architecture enhancement for next generation real time communication; Phase 2"