3GPP TSG SA WG4#121	S4-221330

Toulouse, 14th – 18th November 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.119-PD | CR | pseudo | rev | - | Current version: | 3.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [MeCAR] Minimum Device Capability Considerations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | MeCAR |  |  |  |  |  | Date: |  |  | 08/11/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 5.7	Minimum Media Capabilities

A MeCAR device can query the capabilities of the XR runtime, the scene manager and presentation engine for audio and visual media, and the MAF.

A MeCAR device at the minimum shall support:

- A set of XR Runtime functionalities

  - allowing to establish a XR session

  - enabling 6DoF and AR video rendering based on swapchain APIs (as defined for example in OpenXR) with pose correction

  - enabling rendering of audio signals with pose correction

  - providing a 6DoF pose of the user to the application

- A subset of 2D video codecs to support display characteristics as defined in 5.4.1.2.1

- A subset of audio codecs (to be defined)

- Encapsulation and delivery based on RTP

- A media playback entry point (e.g. a scene description)

- A security framework to SRTP based delivery

A MeCAR device may in addition support:

- Additional functionalities of XR Runtime or other APIs beyond the minimum set supported

- 3D rendering capabilities that, for example, allow to render point clouds, meshes, depth, etc present in the scenes

- Additional audio and video encoding and decoding capabilities

- Decoding of streaming formats encapsulated in CMAF

- Capabilities to be exchanged with the network in order to support cloud/edge rendering