Source:	NTT

Title:	Modification of requirements for stereo coding in IVAS-3

Document for:	Discussion and Agreement

Agenda Item:	7.5

Introduction

Source, as a mobile network operator, thinks IVAS stereo coding at a lower bit rate is attractive and essential for teleconferences. Stable communications over mobile networks still need low-rate Codec for prioritized connection services such as VoLTE and VoNR. The timely establishment of low-rate codecs for teleconferences has vital market needs.

In addition, generating interoperable EVS stream functionality is also essential for IVAS for the smooth deployment for network services.

Observation

At the final stage of completion of the requirement document, IVAS-3, source would like to propose modifying the document in line with the following observations.

1. The following performance requirements for low-middle rate stereo coding may be too high and may cause failures of timely establishment.

"NWT the system with more than 150% of total bit rates by EVS" or

"BT the system with larger total bit rates by EVS."

2. The conditions for Mono compatible requirements at 13.2 kbps are empty.

3. The processing procedure of Mono compatible requirements may be ambiguous.

Conclusion

The requirements of qualities for stereo coding at 24.4 and 32.2 kbps should be slightly relaxed to avoid any risks of delaying the establishment of IVAS.

Processing procedures and conditions of Mono compatible requirements should be added as follows, although this requirement may not be checked at the selection test phase.

SA4-221525: IVAS Permanent Document IVAS-3: Performance Requirement

Table 1: Stereo requirements.

| BW | Bitrate (kbit/s) | FER | DTX* | Stereo Requirements | Mono compatible requirements |
| --- | --- | --- | --- | --- | --- |
| WB | 13.2 | All | On/Off | NWT EVS @ 2x 8 kbps OR BT EVS-WB @ 2x 7.2 kbps | NWT stereo downmix of EVS @ 2x 7.2 kbps |
|  | 16.4 |  |  | NWT EVS @ 2x 9.6 kbps OR BT EVS @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  | 24.4 |  |  | NWT EVS @ 2x 13.2 kbps OR BT EVS @ 2x 9.6 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  | 32 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
| SWB FB | 13.2 | All | On/Off | NWT EVS-WB @ 2x 8 kbps OR BT EVS-WB @ 2x 7.2 kbps | NWT stereo downmix of EVS @ 2x 7.2 kbps |
|  | 16.4 |  |  | NWT EVS @ 2x 9.6 kbps OR BT EVS-WB @ 2x 8 kbps | NWT stereo downmix of EVS @ 2x 8 kbps |
|  | 24.4 |  |  | NWT EVS @ 2x 13.2 kbps OR BT EVS @ 2x 9.6 kbps | NWT stereo downmix of EVS @ 2x 13.2 kbps |
|  | 32 |  |  | NWT EVS @ 2x 16.4 kbps OR BT EVS @ 2x 13.2 kbps | NWT stereo downmix of EVS @ 2x 16.4 kbps |
|  | 48 |  |  | NWT EVS @ 2x 32 kbps OR BT EVS @ 2x 24.4 kbps | NWT stereo downmix of EVS @ 2x 24.4 kbps |
|  | 64 |  |  | NWT EVS @ 2x 48 kbps OR BT EVS @ 2x 32 kbps | NWT stereo downmix of EVS @ 2x 32 kbps |
|  | 96 |  |  | NWT EVS @ 2x 64 kbps OR BT EVS @ 2x 48 kbps | NWT stereo downmix of EVS @ 2x 48 kbps |
|  | 128 |  |  | NWT EVS @ 2x 96 kbps OR BT EVS @ 2x 64 kbps | NWT stereo downmix of EVS @ 2x 64 kbps |

- To check of Mono compatible requirements, two signals should be compared;

  - CuT: Decoded mono signal from EVS compatible stream generated by stereo downmix for EVS with stereo input.

  - Reference: Downmixed signal of decoded signals from two EVS encoded streams for each channel.

References

[1] SA4-221525: IVAS Permanent Document IVAS-3: Performance Requirement