3GPP TSG-SA4 Meeting #122	S4-230200

# Athens, Greece, 20th Feb 2023 - 24th Feb 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0055 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE data processing for AF-based Network Assistance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony Europe B.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Ph2 |  |  |  |  |  | Date: |  |  | 14-02-2023 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The data collection and reporting feature for AF-based Network Assistance lacks the processing of useful information for service providers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds aggregation functions for AF-based Network Assistance invocations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The data collection and reporting feature for AF-based Network Assistance continues to lack the processing of useful information for service providers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The proposed change is admittedly not explicitly called for in the 5GMS_Ph2 WID, although the change clearly supports improved live service provision. If explicit justification for the change is needed then a revision to the 5GMS_Ph2 WID should be considered, i.e. to add an objective based on the recommendations on network event usage stated in clause 6.8 of TR 26.804. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev - : S4-230200 for SA4 #122. |  |  |  |  |  |  |  |  |

### 4.7.3	UE data processing for 5GMS

#### 4.7.3.1	UE data processing procedures for downlink media streaming

The following restriction dimensions and aggregation functions defined in clause 4.5.2 of TS 26.531 [22] may be provisioned in a Data Access Profile as part of a 5GMSd Provisioning Session and shall, as a consequence, be applied to reported UE data prior to exposing it to event consumers.

Table 4.7.3.1-1: Valid processing of downlink media streaming UE data by the Data Collection AF

|  | Restriction dimension |  |  | Aggregation function |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time | User | Location | None | Count | Mean | Maximum | Minimum | Sum |
| QoE metrics for downlink media streaming | Yes | Yes | Yes | Yes | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) | Yes (NOTE 1) |
| Consumption of downlink media streaming | Yes | Yes | Yes | Yes | Yes (NOTE 2) | No | No | No | No |
| Downlink dynamic policy invocations | Yes | Yes | Yes | Yes | Yes (NOTE 3) | No | No | No | No |
| AF-based downlink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 3) | Yes (NOTE 4) | Yes (NOTE 4) | Yes (NOTE 4) | No |
| Downlink media streaming access activity | Yes | Yes | Yes | Yes | Yes (NOTE 2) | No | No | No | No |
| NOTE 1: Aggregation functions applied individually to all exposed metrics within the scope of the applicable restriction dimension(s). NOTE 2: Number of downlink media streaming sessions within the scope of the applicable restriction dimension(s). NOTE 3: Number of invocations within the scope of the applicable restriction dimension(s). NOTE 4: Aggregation functions applied to bit rate recommendations and throughput estimations within the scope of the applicable restriction dimension(s). |  |  |  |  |  |  |  |  |  |

#### 4.7.3.2	UE data processing procedures for uplink media streaming

The following restriction dimensions and aggregation functions defined in clause 4.5.2 of TS 26.531 [22] may be provisioned in a Data Access Profile as part of a 5GMSu Provisioning Session and shall, as a consequence, be applied to reported UE data prior to exposing it to event consumers.

Table 4.7.3.2-1: Valid processing of uplink media streaming UE data by the Data Collection AF

|  | Restriction dimension |  |  | Aggregation function |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Time | User | Location | None | Count | Mean | Maximum | Minimum | Sum |
| AF-based uplink Network Assistance invocations | Yes | Yes | Yes | Yes | Yes (NOTE 1) | Yes (NOTE 2) | Yes (NOTE 2) | Yes (NOTE 2) | No |
| NOTE 1: Number of invocations within the scope of the applicable restriction dimension(s). NOTE 2: Aggregation functions applied to bit rate recommendations and throughput estimations within the scope of the applicable restriction dimension(s). |  |  |  |  |  |  |  |  |  |